SERVNET Commands and GTESLIP.NET References

SLIPNET Command Reference

1.  Startup GTESLIP.NET

When a MODEM user types FTP, MODEM.EXE shells out to call SERVNET.EXE.

This program uses a file called \GTESLIP.NET similar to NET/NOS's use

of AUTOEXEC.NET.

GTESLIP.NET must be in the root directory of BBS drive.

It is read and executed as though its contents were typed on the

console  as   commands.  This is needed for setting the local IP

address and host name, initializing the IP parameters, and  starting

the  FTP server.  Only FTP service are available, TELNET and SMTP are

not supported.

2.  Other files needed are \HOSTS.NET and \FTPUSERS.

These can be same files (shared) as those you use for NET.EXE.

3. This section describes each of the commands recognized while in  command

mode or read in from GTESLIP.NET.

3.1. #

Commands starting with the hash mark (#) are ignored. This   is   mainly

useful for comments in the \GTESLIP.NET file.

3.2. hostname [<name>]

Displays or sets the local host's name (an ASCII string such  as  "ka9q-

pc",  NOT an  IP address).  Currently this is used only in the greeting

messages from the SMTP (mail) and FTP (file transfer) servers.

3.3.  ip

3.3.1. ip address [<hostid>]

Displays or sets the local IP address.

3.3.2. ip ttl [<val>]

Displays or sets the default time-to-live value placed  in  each  outgo-

ing   IP  datagram.  This limits the number of switch hops the datagram

will be allowed to take. The idea is  to  bound  the  lifetime  of   the

packet   should  it  become caught  in a routing loop, so make the value

somewhat larger than the diameter of the network.

3.4.  start ftp

Starts  the  FTP  Internet  server, allowing  remote   connection

requests.

3.5.  tcp

3.5.1. tcp irtt [<val>]

Display or set the intial round trip time estimate, in  seconds,  to  be

used  for new  TCP  connections until they can measure and adapt to the

actual value.  The default is 5 seconds.  Increasing this when operating

over  slow channels  will avoid the flurry of retransmissions that would

otherwise occur as the smoothed estimate settles  down  at  the  correct

value.  Note  that  this command  should  be given  before  servers  are

started in order for it to have effect on incoming connections.

3.5.2. tcp mss [<size>]

Display or set the TCP Maximum Segment Size in bytes that will  be  sent

on   all outgoing TCP  connect  request (SYN segments).  This tells the

remote end the size of the largest segment (packet) it may send.  Chang-

ing   MSS  affects   only  future connections; existing connections are

unaffected.

3.5.3. tcp window [<val>]

Displays or sets the default receive window size in bytes to  be  used

by   TCP  when  creating new connections. Existing connections are unaf-

fected.

4.  Changes from Phil Karns NET 890421.1 code.

As you have probably noticed, many normal IP commands are missing.  This

is because of the "slip-only" configuration, those commands are no longer

applicable (example: route is no longer needed since EVERYTHING is routed

out the slip port).

4.1   Commands no longer applicable.

arp

attach (still there, but syntax and meaning has changed)

ax25

escape (not need for this IBM-PC only version)

log  (all logging is sent to the screen)

mode

mulport

param

route

trace (removed to reduce code size and not really needed in slip)

udp

4.2   New commands.

4.2.1	   baud <speed>

The baud command is used to change the commport speed.  It is available

only from the command mode.  Remember after changing the baud to go into

modem mode and issue the "AT" command to the modem.  Most modems use this

command to figure out the new commport baud.

4.2.2    attach port <comm> <mtu>

Configure and attach the commport interface to the system.

<comm> is the number of the commport,  ei COM1:  would be 1.

<mtu> is the Maximum  Transmission  Unit  size,  in  bytes.    Datagrams

larger  than   this   limit   will  be  fragmented  at the IP layer into

smaller pieces.

Examples:

# Attach the commport normally known as "com1" (the first

# controller)

# Outgoing packets larger than 1024 bytes are fragmented.

attach port 1 1024

NOTE: Only ONE comm port may be attached and it must be the same one

used for MODEM.EXE (should have been obvious, right?)