8051-Based Stepper Motor Driver

(0) Introduction

This software is part of a larger real-life test system that was designed

to test gear ratios on registers.  The stepper motor was used to precisely

control the total amount of rotation of the gear train in order to get precise

measurements on this ratio.

The system was configured as follows:

* 1 microprocessor-based server, connected to a keypad and display

* 4 microprocessor-based client units, each driving a stepper

motor in open loop configuration.

* One RS-485 serial link between the units.

The server unit was set up to be as non-linguistic as possible (using numbers

and pictures), because (1) people can see faster and more reliably than they

can read, and (2) the unit was to be used abroad.

(1) Multi-tasking Control

Each unit operates in real-time.  That means correctness requires not only

that correct outputs be produced for given input, but that it also be

produced AT THE CORRECT TIME.  This implies the need for process synchronization

and points to the need for an underlying multi-tasking architecture.

The kernel library routines provided with the 8051 package were custom fitted

to this application (KC.lib).  This kernel differs from the kernel used in the

other demo software in that it operates at 3 distinct priority levels, instead

of the usual two: (1) from within a high-priority interrupt, (2) from within a

low-priority interrupt, and (3) outside all interrupts.  As in other

applications, since this is I/O-bound software, there will not be any foreground

processing, so that the last level is effectively disabled.  All processing

occurs inside of interrupts.

(2) Hardware Configuration

A driver unit is connected as follows:

(1) A stepper motor unit via 4 I/O lines.  These lines are for:

P1.0 ... PULSE OUTPUT to control the motor.

P1.1 ... DIRECTIONAL SETTING to control which way the motor rotates

P1.2 ... ENCODE PULSE SIZE to control how many encoder pulse outputs

make up one rotation of the motor (200 or 400).

P1.3 ... ALL WINDINGS OFF to set or reset the motor so that it can be

run.

(2) Two LED units to display the status of the most recent test.

P1.4 ... Red LED

P1.5 ... Green LED.

(3) A placement unit for the stepper motor, with inputs for:

INT1 ... MICROSWITCH to sense the placement of a register unit.

Activates the motor unit on a falling edge.

INT0 ... DIAL SENSOR to sense the position of the register needle.

Produces a falling edge for a sensor interrupt.

(3) Speed Control and Synchronization

(A) Overview

The stepper motor is controlled by sending out pulses of around 20 to 50

microseconds for each step.  Depending on the setting of the motor, either

200 or 400 steps will generate one full rotation of the motor.  The speed of

the motor is therefore inversely proportional to the delay between consecutive

pulses, and the length of time spent at a given speed is equal to that delay

multiplied by the number of pulses sent out.  In this setup, 200 pulses per

rotation is always selected.

The easiest way to accomplish acceleration or deceleration is to use a

look-up table for converting desired speeds into desired pulse delays, and

to "accelerate" the motor by driving it incrementally at higher and higher

speeds.  I have set up such a look-up table for this application that lists

delays for multiples of 25 RPM, which comes out to 250/3 pulses per second at

200 encoder pulses per rotation, with the list ranging from 25 RPM to 2000 RPM.

The natural time unit in this application is 3/250 second (the inverse of

25 RPM).  To drive the motor at N multiples of 25 RPM for T time units will

therefore require that N x T pulses be sent out: N pulses per time unit.

The machine's instruction cycle rate is 921600 Hz, and this is also the rate

of the internal clock used to measure pulse delays.  So the delay for a 25 RPM

pulse would be 921600 * 3/250 = 11059.2 cycles.  Similarily, the delay for N

multiples of 25 RPM will be 11059.2/N.  These are the values stored in the

table.

So accelerating the motor simply comes down to driving it at 25 RPM for a

certain number of time units, at 50 RPM for the same duration, at 75 RPM for the

same duration, and so on up to the desired speed.  A similar method

accomplishes deceleration.  The number of time units spent at each speed is an

empirical constant that is chosen as small as possible.  However, if it is

chosen to be too small then the corresponding acceleration or deceleration will

become so fast that the stepper motor will consistently lose coupling.  So that

determines the lower limit of the choice made.  In general, different constants

are chosen for acceleration and for deceleration.

Driving the stepper motor past 1000 RPM will require time-critical software

so precise that the delays described above would have to be carried out by the

microprocessor down to the last clock cycle.  That means microsecond precision.

The reason for this can be illustrated by the following observation.  At

1750 RPM (70 multiples of 25 RPM), the delay between pulses will only be

11059.2/70 clock cycles, or about 158 cycles.  The speeds corresponding to

157, 158, and 159 cycles are;

11059.2/157, 11059.2/158, and 11059.2/159

multiples of 25 RPM, which comes out to about:

1761 RPM, 1750 RPM, and 1739 RPM

respectively.

An algorithm that consistently misses its delay by random errors of merely 1

clock tick will cause a random plus or minus 11 RPM variation in the motor when

it's running at around 1750 RPM.  That will lead to vibrations that will cause

the motor to lose coupling and break to a sudden stop!

The timing is achieved simply by using a continuously running clock that

automatically reloads itself (TIMER 2 of the 8052).  Then one can keep the

timer going without having to worry about calculating any latencies.

(B) Example

This example will illustrate the process that is gone through to bring

a stepper motor up to 1750 RPM, run it there for a while, and then step it

down.  In this application, motors are consistently set for counter-clockwise

motion with 200 pulses per rotation.

First SetDriver is called.  This will delay a second to wait for the windings

to finish setting.  The value 1750/25 is placed into the accumulator, A, and

Accelerate is called.  When complete, the motor will be running at 1750 RPM.

Then PulseTrain is called.  This routine runs the motor at its current

speed indefinitely.  It only stops when either of the external flags, Change,

or Aborting, is set by another process.  The second flag is used to enable the

operator to abort a test.  The first is used to allow the test to abort

normally.  We will assume that Change was set here.

After PulseTrain is called, the motor is decelerated to a stop.  This is

done by placing the value 0 in the accumulator, A, and then calling Decelerate.

It is possible in this application to bring the motor to an immediate stop, if

desired, by just turning off the pulse timer after PulseTrain is finished.

Afterwards, ResetDriver is called to turn off the windings.

(4) Testing Algorithms, Operator Interface

(A) Overview

There are two types of processes that the driver unit will support: (1)

Test mode, and (2) Jog mode.  The first uses the sensor input to mark off

one full rotation of the register undergoing measurement, the second mode

allows the operator to set the position of the dial on a register.

The control loop is called StepperLoop.  When a test starts, it will either

enter jog mode or test mode depending on the variable, Testing, which must be

set by another process before starting.  The WaitForActive loop is set up to

guard against spurious INT1 interrupts.

(B) Test Mode

A driver unit will receive a command to enable test mode and a specification

of the register unit type undergoing test.  Both the specification and mode

will remain between tests until the unit receives new speficiations.

After test mode is enabled, then from then on each time the operator places

a register in the Client(s) selected a test will automatically start.  The

motor on the Client(s) will accelerate to full test speed (1750 RPM).  When

the dial passes the dial sensor the first time output pulses will be counted.

When the dial reaches the sensor the second time, the counting will be stopped

and the motor will be immediately stopped.

Thus, the dial sensor will serve both the purpose of marking off one full

rotation of the register and of setting up the register's dial.

When a test is thus completed, the driver unit will perform mathematical

calculations to determine the outcome of the test and then light the green LED

if the test passes, or else the red LED.  Accuracy control limits, for this

application, were set to 1/256 (0.4%).  On request from the master unit, the

driver unit will transfer more precise information on the actual percent error.

Communication with the driver unit may take place during the test.  This

makes it possible for the master unit to periodically poll the driver for its

current status and to determine when the driver is finished with a test.  The

driver unit may also receive an abort command from the master over the

communications line, telling it to stop the test, or telling it to change

speed or stop during jog mode.

(C) Jog Mode

When the driver receives the jog command, it will be set in the jog mode

for all the following tests until it next receives a different specification.

Each time a register is placed, the driver unit will automatically start the

jog process.

The motor will accelerate up to a relatively fast speed.  Each time the

driver receives the change-speed command from the master, it will switch its

speed between fast and slow (1750 RPM and 500 RPM).  When it receives the

abort command, it will stop the motor.

(5) Communications and Commands

(A) Overview.

A RS-485 multidrop set-up is used to allow a master unit to communicate with

the driver, and possible several drivers in parallel.  Communications are run

with 9 bits, no parity, 1 data bit and at 9600 baud.

At the driver unit, it is assumed that T0 will be driving the transmit

buffer.  When set, T0 enables transmissions over the RS-485, and when clear

transmissions are disabled.

In general, at no time can more than one unit on a RS-485 line have its

transmission enabled.

(B) Addresses and Address Matching.

The 9th bit can be used by a master unit to distinguish between an address

and data.  If the unit had one master and 4 drivers, then each driver can be

given a different address, particularily, f7, fb, fd, and fe (hexadecimal).

In this setup there can also be one broadcast address (ff).

When these addresses are represented in binary, the look like this:

First Driver  f7: 1 1 1 1 0 1 1 1

Second Driver fb: 1 1 1 1 1 0 1 1

Third Driver  fd: 1 1 1 1 1 1 0 1

Fourth Driver fe: 1 1 1 1 1 1 1 0

Broadcast     ff: 1 1 1 1 1 1 1 1

A driver will respond to any addresses that "matches" its own address. A match

occurs with any address is sent out with 1's occurring in AT LEAST the positions

that the driver's address has 1's in.

As an example that has nothing to do with the above, if the address to be

matched was 33 in hexadecimal (00110011 binary), then the following 16

addresses would match it:

0 0 1 1 0 0 1 1   0 0 1 1 0 1 1 1   0 0 1 1 1 0 1 1   0 0 1 1 1 1 1 1

0 1 1 1 0 0 1 1   0 1 1 1 0 1 1 1   0 1 1 1 1 0 1 1   0 1 1 1 1 1 1 1

1 0 1 1 0 0 1 1   1 0 1 1 0 1 1 1   1 0 1 1 1 0 1 1   1 0 1 1 1 1 1 1

1 1 1 1 0 0 1 1   1 1 1 1 0 1 1 1   1 1 1 1 1 0 1 1   1 1 1 1 1 1 1 1

which have hexadecimal values: 33, 37, 3b, 3f, 73, 77, 7b, 7f, b3, b7, bb, bf,

f3, f7, fb, and ff.  In the above cases, the only thing that matches address

f7, fb, fd, or fe is the address itself and ff.  This justifies calling ff the

broadcast address.

When the 9th bit is set to 0, then it is assumed that data is being sent.

Only the units that have just been addressed will accept the data, the others

will simply ignore it.

A master does not need an address, so each driver will only send data with

the 9th bit set to 0.  No driver ever sends an address.  The software for these

communication routines is contained in RX9.lib.

(C) Polling

A master unit can periodically poll each driver unit to determine when each

has just completed a test.  At that time, it will request more information on

the test results from the driver.

(D) Command summary

These are the possible commands that can be sent to the driver.  The

commands TEST, JOG, CHANGE, and ABORT can be sent with a broadcast address.

The others, STATUS, and DATA can only be send to individual addresses.

The format of a command must be:

Address (9th bit set to 1),

Command (9th bit set to 0)

as described above.  The values of the commands are respectively:

0 ... TEST

1 ... JOG

2 ... CHANGE

3 ... STATUS

4 ... ABORT

5 ... DATA

The following is a summary of the commands:

STATUS: To poll the driver's status:

The driver will respond with two bytes:

Speed ... the current speed of the motor in 25 RPM units,

represented iu Binary-Coded Decimal format.

Status ... the byte containing the status flags:

Bit 0 = 1 if the motor is currently active, 0 else.

Bit 1 = 1 if the Client is in testing mode, 0 for jogging.

Bits 2 to 7 are all 0.

The values of Status are thus:

0 = Motor off.  Waiting to begin jog.  This is the initial state.

1 = Motor running. Currently in jog mode.

2 = Motor off.  Waiting to begin a test.

3 = Motor running. Currently in a test.

The Speed is represented as a Binary-Coded Decimal taking on values 00

for (0 RPM), 01 (25 RPM), 02 (50 RPM), up to 70 (1750 RPM).

DATA: To query the driver's measurement of register accuracy.

The master unit should sent this command immediately after it detects

a transition out of state 3.  The driver will respond by sending the

following bytes:

Sensors ... the number of sensors recorded (0, 1, or 2).

Results ... the byte containing the results of the test:

Bit 0 = 1 if the test passed, 0 if it failed.

Bit 1 = 1 for a pulse match or over-count,

0 for a pulse-undercount.

Bits 2 to 7 are all 0.

Percentage ... a absolute value of the percent error.

The value of Results are thus:

0 = Test failed.  Pulse under-count.

1 = Test passed.  Measured pulse count < Calculated pulse count.

2 = Test failed.  Pulse over-count.

3 = Test passed.  Measured pulse count >= Calculated pulse count.

A test is considered to have passed if it falls within the accuracy

control limit (currently set at 0.4%).

The last two items are only valid if the sensor count is two.  During a

test, the Results and Percentage variables are set to 0.  Percentage is

represented as a Binary-Coded-Decimal, taking on values 00h (0.0%),

01h (0.1%), all the way up to 99h (9.9%).  Any percent error exceeding

9.9% is truncated to 9.9%.

TEST: To enable the test mode for the driver.

Data will be sent to indicate the theoretical number of pulses the motor

requires to rotate the register dial one full rotation.  The driver will

use it to measure the register's accuracy against.  It will switch on

Bit 1 obtained from the STATUS command.

The pulse number is sent to the driver as a 3 byte integer with the lowest

order byte first.

JOG: To enable the jog mode for the driver.

The driver will switch off Bit 1, as read in the STATUS command.

CHANGE: To change the current jog speed.

The driver responds by switching between fast jog and slow jog.  This

only has effect in state 1 (jog mode active).

ABORT: To abort a test, or to stop the jog mode. This only takes effect when

Bit 0 of the STATUS command is on (motor active).

DATA is always responded to, but is only meaningful after a test has completed.

The values returned reflect the status of the last test run, or are meaningless

if no previous test has been run.  For obvious reasons, STATUS is also always

responded to.