Here is a summary of the IXO protocol.  All typos and

misinterpretations are mine.  This is based on reading the Glenayre

Electronics book.  I have not yet implemented the automatic protocol,

but I have implemented the manual one.

You are "remote entry device" and the system that you dial into is

"paging central".

(1) Remote sends CR at two second intervals until paging central

responds with ID= at correct baud rate or until three transmissions

have been completed.  (This step exists to allow for possible future

baud rate recognition).

Some systems have chosen to send ID= from the central if they do not

receive CR in about two seconds.  This variation appears to be

acceptable as the central continues to respond to CR's with ID='s.

(2) Central sends ID=.  Request for ID returned within one second of

receipt of CR.  Paging central may, at its option, send CR or CR LF

after ID=

Paging central may optionally choose to send ID= if it does not

receive CR after appropriate waiting time.

FOR AUTOMATIC REMOTE ENTRY:

At this point you can start following 2A or 2M depending on which

sub-protocol you want to use.

(2A) Remote sends ESC SST  (typically ESC PG1)

ESC signifies entry device intends to talk in automatic dump mode.

SS is a set of two alphanumeric charactoers signifying a type of

service to be accessed:

For a paging service where:

Field 1 = Pager ID

Field 2 = Message

SS will be sent at "PG".

Where T is a single alphanumeric character relating to the type of

terminal or device attempting to send the message.  T = "1" is a

category of entry devices using the same protocol.  The PETs and IXO

devices are members of this category.  T = "7" or "8" or "9" are

reserved for wild card terminals or devices which may relate to a

specific users' system.

(3A) Remote sends 6-char alphanumeric password then CR.  (Password is

optional and is, in general, reserved for future services.  Password

may be interpreted as either a caller ID or a system entry key.

Length of password, when used, may be different in some systems.)

When an incorrect logon sequence beginning with ESC is received, the

paging central may respond with an ID= if it requires a

retransmission.

(4A) Central responds with a message sequence (lines of text, each one

followed by a CR).  (Typical messages are "Processing - Please Wait",

"4 pages sent" or "Bad Phone Line Call Again".

(5A) Central then sends one of three sequences:

CR ACK CR         This means "logon accepted"

CR NAK CR         This means "requested again"

CR ESC EOT CR     This means "Forced disconnect"

(6A) Central may then send another message sequence (optional).

(7A) Central sends ESC [p CR

Message go ahead is sent when paging central is ready for new

information.  The "p" is lowercase.

(8A) Remote sends STX field1 CR field2 CR field3 CR ... fieldn CR

EEE <CHECKSUM> CR

You can send as many fields as you wish.  PG terminals send the

pager-id in field1 and the message in field2.  (no other fields are

defined).  This entire message can be a max of 250 bytes.  A field may

be any length and where necessary may be continued in following

blocks.  A field always ends with a CR.  The CR field delimiter

suggests CR may not be used within a field.  A block always begins

with a STX and ends with a checksum followed by a CR.  The characters

preceding the checksum depends on what, if anything, is continued

beyond the block boundary.

EEE can be an ETX, ETB, or RS.

The ETX is used as a block termination indicator if a given

transaction (fields 1 through N) ends within the block being

transmitted.

The ETB is used as a block terminator if the transaction is continued

into the next block, but the last field in the current block is

complete.

If the last field within the current block is to be continued in the

next block, no CR is inserted at the end of the first portion of the

field and the US character is used as a block termination character.

The CR terminating the broken field is sent at the end of the field in

whatever block the field actually terminates.

No limit is established within the protocol itself regarding the

number of transactions, the number of fields or the number of blocks

per field; however, a particular user system may have limits on any of

these items.

Some systems may be limited to one block per transaction and one

transaction per telephone connection.

Each checksum is computed by performing the simple arithmetic sum of

7-bit values of all characters preceding it in that block.  (This

means that the STX and ETB/ETX are included in the sum).  The checksum

is then the least significant 12 bits of this resulting sum.

The checksum is transmitted as three printable ASCII characters having

values between HEX 30 and HEX 3F (the characters 0123456789:;<=>?).

The most significant four bits of the sum are encoded as the 4 LSB of

the third chacter.  See example.

A normal paging system will have two fields only:

Field 1 = Pager ID

(normally up to eight digits.  May include function and check digit).

Field 2 = Message

Field 1 or field 2 may be empty.  For example, when a page is tone only,

field 2 will be empty.  Even when empty, a field is followed by a CR.

Some systems will reject transactions which have an empty field 2 for

a display page or transactions which have an empty field 1.  Other

systems are less restrictive.

(9A) The response to each block is one of four:

Message sequence CR ACK CR   = OK, send next block.

Message sequence CR NAK CR   = Checksum or transmission error, send last

block again.

Message sequence CR RS CR    = Abandon current transaction and go to next.

RS may occur when the checksum is OK,

but the current transaction violates a

system rule.  At the option of the

system, it may occure in other cases.

Message sequence CR ESC EOT CR   = Begin disconnect.

Any of the responses may have an optional message sequence before

them, although the system designer should understand the consequences

to the user with all planned entry devices.

It is expected that many systems will save their message sequence

responses until immediately before disconnect.  For some entry

devices, it may also be desirable that message describing non-checksum

error assocaiated with a particular transaction in a PG service will

begin with the letter ID followed by the contents of field 1 for that

transaction.

(10A) Remote sends EOT CR

After reception of an ACK or RS for the last transaction in a given

service, the entry device sends EOT CR meaning there are no more

transactions remaining in this service.

(11A) Optional: Central sends a message sequence CR.

Although optional, it is highly desirable.

(12A) Central sends RS CR

An RS CR should be sent at this point if the paging central finds any

data ACK in previous steps by the system to be unacceptable because of

content (e.g. invalid pager number or a message field inappropriate

for the type of pages, etc.)

(13A) Central sends ESC EOT CR

then hangs up.

(14A) stop.

(2M) Remote sends M CR (capitol M then CR)

Lack of ESC at beginning of response to ID signifies manual operation

where applicable.  From here on it's undefined, but most systems ask

(in English) for the info they need and then return you to step (1).

NOTES:

Bell 103 300bps modem is standard, others may be used.  Protocol is

ASCII with XON, XOFF either direction using 10-bit code (1 start, 7

data, 1 parity, 1 stop) with even parity.

In the case of delays, the paging central shall wait at least four

seconds (eight seconds for (1) (2) (2A)) before disconnecting the

remote entry device.  The remote entry device shall wait at least 10

seconds for a character from the central before hanging up.

Paging central may, at its option, send CR LF in place of CR at the

end of any sequence.

For inital use of protocol, the paging central shall be equipped to

receive full duplex using a Bell 103 compatible modem at 300 baud.

Optionally, certain inputs may be capable of receiving 110 baud Bell

103 full duplex, 300/1200 baud Bell 212 full duplex.  No echo shall be

employed in full duplex mode.  Any attempts at automatic baud rate

determination shall be within the restraints of the specified

protocol.

SAMPLE Checksum Example:

STX   000 0010

1     011 0001

2     011 0010

3     011 0011

CR    000 1101

A     100 0001

B     100 0010

C     100 0011

CR    000 1101

ETX   000 0011

10111 1011

1   7   ;

So, the checksum is "17;"

There is also a really nice flowchart, which I can't reproduce here

without much too much trouble.  ...and you wouldn't have known it

existed if I hadn't told you. :-)