(Q_and_a)

Item: 102 by akcs.getz@hpcvbbs.cv.hp.com

Author: [Robin Getz]

Subj: fix for smith corona ram cards?

Date: Fri Jan 10 1992

Hello Hp.

I was wondering if you could tell me if, or how this could damage my 48.

I realize there are some voltage regulation problems with non-HP ram

cards (low voltage shutdown threshold).  [Note: For details about this

problem, see MEMCARDS.DOC on Goodies Disk #3.  -jkh-]  I think I have

found a way to bypass this problem.

There is a control line going to the I/O chip from the voltage

regulator. This line is Vcc until Vcc is lower than 4.1V (on the card

I ripped apart) then it slams to zero, just as you said it would in

all the other documentation you have previous posted on this subject.

What I have done is disconnect this line from the voltage regulator

and reconnect it to Vcc.  Now the card will not shut off until Vcc

reachs card battery voltage (about 3.0 V).  Is not this what we want?

I have this card in my calculator (rip up my warranty card please) and

it works fine.  I have put nearly dead batteries in my calculator to

simulate low voltage conditions and it works just fine.

Before I run around and start telling everyone which connections to

change I thought I would give you the first kick at the cat (per say).

Robin Getz

Electrical Engineer

University of Saskactchewan

873398@bell.usask.ca

If you would like I can send you the changes I have made in order for you

to test in your labs.

[Note: This interesting posting brought a few responses from others,

and no objections from HP....  -jkh-]