NCSA Telnet for the PC

Version 2.2TN and Version 2.2D

July 1988

(ASCII printable version of this document)

This document describes enhancements added by Clarkson University

to version 2.2 of NCSA telnet. Changes to this document are

denoted by  | in the first column of additional or changed text.

Changes from 2.2C to 2.2D and 2.2TN are denoted by a + in the first

column of additional or changed text.

UD-99

US-14

NCSA Telnet Version 2.2 source code and documentation are in the public domain.

Specifically, we give to the public domain all rights for future licensing of

the source code, all resale rights, and all publishing rights.

We ask, but do not require, that the following message be included in all

derived works:

Portions developed by the National Center for Supercomputing

Applications at the University of Illinois at Urbana-Champaign.

| Portions developed by Clarkson University.

| Those portions are also hereby placed in the public domain by

| Clarkson University, as long as the following statement remains

| in the source.

| "Portions developed by The Educational Resources Center at Clarkson University"

+ TN3270 code developed by Greg Minshall of Berkeley and is copyright (c)

+1988 Regents of California.

PROBLEM REPORTS

Mail all suggestions and manual corrections to:

NCSA Documentation and Publications Department

152 Computing Applications Building

605 E. Springfield Ave.

Champaign, Illinois 61820

+| Version 2.2TN bug reports (Bootp, packet driver, domainslist) may be sent

| to bkc@omnigate.clarkson.edu

| Brad Clements

| Educational Resources Center

| Clarkson University

| Potsdam NY 13676

| (315)268-2292

DISCLAIMER

THE UNIVERSITY OF ILLINOIS GIVES NO WARRANTY, EXPRESS OR IMPLIED, FOR THE

SOFTWARE AND/OR DOCUMENTATION PROVIDED, INCLUDING, WITHOUT LIMITATION, WARRANTY

OF MERCHANTABILITY AND WARRANTY OF FITNESS FOR A PARTICULAR PURPOSE.

TRADEMARK ACKNOWLEDGMENTS

PostScript is a registered trademark of Adobe Systems Inc.

UNIX is a registered trademark of AT&T.

SideKick is a registered trademark of Borland International Inc.

VAX, VMS, and VT102 are registered trademarks of Digital Equipment Corporation.

Hercules is a trademark of Hercules Computer Technology.

Enhanced Graphics Adapter, IBM PC, PC/AT, PC-DOS, Personal System/2, and

Topview are registered trademarks of International Business Machines

Corporation.

Microsoft Windows and MS-DOS are registered trademarks of Microsoft

Corporation.

Revolution 512 is a trademark of Number Nine Computer.

Tektronix 4010 and Tektronix 4014 are trademarks of Tektronix Corporation.

NIC is a trademark of Ungermann-Bass.

EtherCard PLUS is a trademark of Western Digital Corporation.

Ethernet is a trademark of Xerox Corporation.

3COM and Etherlink are trademarks of 3COM Corporation.

INTRODUCTION

PURPOSE OF NCSA TELNET

NCSA Telnet Version 2.2 for the PC provides interactive access

from an IBM PC or compatible to telnet hosts on TCP/IP networks.

NCSA Telnet is an implementation of DARPA standard telnet with

added features that take advantage of the local processing power

of the PC.

SPECIAL FEATURES

Special features of NCSA Telnet for the PC include:

%	VT102 emulation

%	Simultaneous logon to a number of computers

%	Ability to capture text to the PC disk or printer

%	File transfer server (standard FTP)

%	Remote copy server (rcp) for use with UNIX hosts

%	Ability to take full advantage of PC colors

%	Topview/Windows compatible mode

%	Tektronix 4014 emulation

%	Domain name lookup

%	Optional use of RARP for determining PC's IP address

|%	Optional use of BOOTP for determining PC's IP address, netmask, gateway

|%	 	and name servers

%	Scrollback

+|%	43/35 Line mode when used w/ EGA capable video cards and Hi-res monitor

+ TN3270 support which allows a single 3270 type session concurrent

+      with other regular telnet sessions.

+ Now includes a Screen Saver

+ Supports the Terminal Type and NAWS Telnet options

+ Packet driver now supports SLIP

SYSTEM REQUIREMENTS

To use NCSA Telnet you must have the following hardware:

%	IBM PC, PC/XT, PC/AT, IBM PS/2 model 30, or "true" compatible

computer with one of the following Ethernet adapter boards:

3COM 3C501 Etherlink

MICOM NI5210

Ungermann-Bass PC-NIC (same as IBM Baseband Adapter)

Western Digital WD8003E EtherCard PLUS

|	Or any ethernet card with an FTP software compatible packet driver (TSR)

%	or IBM PS/2 models 50,60 or 80, or "true" compatible computer

with one of the following Ethernet adapter boards:

Ungermann-Bass NICps/2

3COM 3C523 Etherlink/MC

%	384K minimum memory

%	Ethernet or Thin Ethernet to connect the PC and other computers

To use NCSA Telnet you must have the following software:

%	PC-DOS or MS-DOS Version 2.0 or later.

%	A standard text editor will be helpful when editing the

configuration file.

ABOUT THIS MANUAL

This section describes the organization of this manual, and the

conventions and nomenclature used in developing it.

Organization of This Manual

This manual is organized into six chapters. Each page of each

chapter is given a unique number that consists of the chapter

number, a period, and the number of that individual page

(beginning with the first page of the chapter). For example, page

2.3 is the third page of the second chapter. Each chapter is

divided into sections, and most sections are divided into

subsections.

Manual Contents

This manual is organized into the following chapters:

Chapter 1, "Starting and Quitting NCSA Telnet," describes how to

start NCSA Telnet and how to open and close a connection between

your PC and one remote host.

Chapter 2, "Introduction to Managing Sessions," introduces NCSA

Telnet's capability for multiple connections.  It also discusses

standard VT102 key emulation, keys used for common EDT operations,

and using a capture file.

Chapter 3, "More about Managing Sessions," discusses multiple

sessions in detail.  The Parameters menu options, DOS shell

feature, and several advanced functions are also described.

Chapter 4, "File Transfer," outlines procedures for transferring

files between a PC and a telnet host.

Chapter 5, "Tektronix 4014 Emulation," discusses NCSA Telnet's

ability to emulate a Tektronix 4014 graphics terminal.

Chapter 6, "Installation and Configuration," contains information

for system administrators (and other experienced users) to use

when installing and customizing a system.

Appendix A, "Error Conditions," describes some of NCSA Telnet's

error messages, as well as their causes and solutions.

Appendix B, "NCSA Telnet Command Reference," lists commands

available from the PC keyboard.

Appendix C, "Configuration File Summary," lists the keywords for

configuration file parameters.

Notational Conventions in This Manual

Figure I.1 shows some typical screen/user interactions using the

conventions employed in this manual. Those conventions, and

others, are explained here.

C:\> telnet machinename [machinename...]

National Center for Supercomputing Applications

NCSA Telnet for the PC version 2.2

(c) Copyright 1987,1988 Board of Trustees of the University of Illinois

ALT-H presents a summary of special keys

4.2 BSD UNIX (newton)

login:

Figure I.1.  Example of Notational Conventions

dothis		Information shown in courier boldface type represents

user entries.

ALT-key		Press and hold the ALT key and then press the key

designated by key. Then release both keys at the same

time.

variable	Do not enter the actual characters shown. Command line

characters shown in lowercase courier bold italic type

represent an entry that may consist of different

characters every time you make the entry. In other

words, it is a variable entry. If it indicates a

machine response, it means the actual wording of the

response will vary, depending on the filename,

machinename, and so on.

...		Do not enter an ellipsis. The ellipsis indicates that

you may enter more material similar to the material

preceding the ellipsis.

[ ]		Do not enter square brackets. Material or actions

presented between square brackets is optional and

should be entered only in certain cases.

INSTALLATION NOTE

This manual assumes that NCSA Telnet has been installed on your

system by a system or network administrator who has assigned an IP

address to your PC. Chapter 6 contains information to be used by

system administrators and other experienced users to install and

customize NCSA Telnet.

CHAPTER 1

STARTING AND QUITTING NCSA TELNET

CHAPTER OVERVIEW

This chapter describes how to start NCSA Telnet and how to open

and close a connection between your PC and one remote host. NCSA

Telnet also features multiple-session capabilities. If you want to

use more than one login session simultaneously, see Chapter 2,

"Introduction to Managing Sessions." Also see Chapter 2 for

information on how NCSA Telnet emulates a VT102 keyboard.

STARTING THE PROGRAM

To run NCSA Telnet from the PC command line, from any

subdirectory, enter:

C:\ > telnet machinename

This initiates a connection to the remote computer with the name

given as the machinename parameter. Normally the host machine

immediately prompts you for a login name and password to begin the

session.

Setting the Terminal Type

NCSA Telnet emulates a VT102 terminal. When you log in to a host,

the host operating system does not know what type of terminal you

are using. Consult the operating system manual for how to set

terminal type and try setting it to VT100 or VT102. For systems

that do not support VT102 (such as many UNIX systems), use VT100,

which is compatible with VT102 emulators. These examples show how

to set the terminal type for two popular operating systems, UNIX

(using the C shell) and VAX/VMS.

newton% set term=vt100;tset		For UNIX hosts.

B$ SET TERM/INQ				For VAX/VMS hosts.

+ Version 2.2D and 2.2TN now support the telnet terminal type option.

+ Most 4.3 BSD unix systems will request NCSA telnet to transmit

+ it's terminal type. NCSA telnet will transmit 'VT102' as its terminal

+ type. If the terminal type is requested a second time, version 2.2TN

+ will transmit 'IBM-3278-2' and prepare to enter 3270 emulation.

+

+ The telnet NAWS (Notify about window size) option is also supported,

+ for those hosts that request NAWS info, ncsa telnet 2.2D and 2.2TN

+ will send the current window size (which may not be 24 x 80 if

+ running on an EGA in 43 or 35 line mode)

THE HELP SCREEN

For a command summary, press ALT-H. A one-screen summary of

command keys appears, as shown in Figure 1.1. The online help

summary serves as your quick reference to the command keys.

Keyboard usage for NCSA telnet:

Alt-A     add a session                    Alt-Y     Interrupt Process

Alt-N     next session                     Alt-O     Abort Output

Alt-M     message screen                   Alt-Q     Are you there?

Alt-E     escape to DOS shell              Alt-U     Erase line

Alt-G     graphics menu                    Alt-K     Erase Kharacter

Alt-C     toggle capture on/off            Alt-X     close connection

Alt-R     reset VT102 screen               HOME      exit graphics mode

Alt-H     this help screen                 Ctrl-HOME clear/enter graphics mode

ScrLock   pause/restart screen (DO NOT use Ctrl-NumLock)

ScrLock   enter/exit scroll-back mode

Alt-T     start file transfer as if typed: ftp [internet address]

Alt-I     send my internet address to host as if typed

Alt-S     skip scrolling, jump ahead

Alt-P     change a parameter, one of:

color, capture file name, backspace, session name, screen mode

Alt-F3    abort program completely.  STRONGLY discouraged

|Alt-W	   send internal FTP password

|ALT-L	  last (previous) session

+ALT-D    Dump screen to the Capture file

Press ESC for information page, space bar to return to session:

+ Versions 2.2D and 2.2TN, if compiled with Turbo C, will also

+ display machine information if the <RETURN> key is pressed

+ at the Help screen. Items such as Stack size and depth of stack

+ used, as well as remaining memory are displayed.

Figure 1.1.  NCSA Telnet Help Screen

QUITTING NCSA TELNET

To exit the program, log out of the host machine using the

appropriate logout procedure for that machine. If you have

concurrent sessions with more than one machine, you must log out

of each machine. Then NCSA Telnet exits and the DOS prompt is

displayed.

If one of the hosts crashes, or a session is otherwise hung up,

press ALT-X. NCSA Telnet will first prompt you for confirmation

and then attempt to close the session while preserving your other

live sessions.

When all else fails, and it appears that all of the connections

are completely jammed, press ALT-F3 to abort the program.  Only

use ALT-F3 as a last resort.

+ If telnet is in 43 or 35 line mode on an EGA display, it will return to

+ 24 line mode on exit.

CTRL-C Will Not Exit

Pressing CTRL-C or CTRL-BREAK will send a CTRL-C to the host.

These commands cannot be used to break out of NCSA Telnet, and

they will not end your session.

CHAPTER 2

INTRODUCTION TO MANAGING SESSIONS

CHAPTER OVERVIEW

This chapter introduces NCSA Telnet's capability for multiple

connections with one or more hosts. The chapter describes standard

VT102 key emulation, keys for some common EDT operations, how to

work with multiple sessions, and operations with the capture file.

KEYBOARD USAGE

When NCSA Telnet is running, the PC appears to the host as a VT102

terminal. For keys that the PC keyboard and the VT102 keyboard

have in common, NCSA Telnet transmits the keystrokes unmodified.

The VT102 keyboard has some keys that the PC keyboard does not

have, and treats other keys differently. Many VT102 keys have

special meanings when they are transferred to the host. To provide

full VT102 functionality, some PC keys are used to emulate the

operation of VT102 keys.

Table 2.1 lists the PC keys to use for VT102 terminal emulation.

Table 2.1.  PC Keys Used for VT102 Terminal Emulation

To Send a VT102 Code			Press PC Key(s)

RETURN					ENTER

DELETE 					BACKSPACE

BACKSPACE 				CTRL-BACKSPACE

LINE FEED				CTRL-ENTER

Keypad ENTER 				F10

(not identical to RETURN)

Keypad . (decimal point)		DEL

Keypad - (hyphen)			F5

Keypad , (comma)			F6

Keypad 0 				INS

Keypad 0 to 9				ALT-0 to ALT-9

-, /, ,, .				-, /, ,, .

PF1 to PF4				F1 to F4

See Chapter 3 and Chapter 6 for information on how to switch the

designations of these keys.

VT102 Functions for the EDT Editor

Table 2.2 lists some VT102 terminal emulation functions that help

users who are working with the EDT editor under VAX/VMS.

WARNING: If you are not using the EDT editor, ignore the use of

these keys. They produce unpredictable results on other systems.

Table 2.2.  VT102 Functions for the EDT Editor

For the EDT Operation			Press PC Key(s)

Save-and-exit				F9

Select					F7

Cut					F8

Paste					F1,then F8

Cursor to beginning of this line	HOME

Cursor to end of this line		END

Scroll backward one screen		PG UP

Scroll forward one screen		PG DN

Return to the previous formfeed		CTRL-PG UP

Go to the next formfeed			CTRL-PG DN

EDT help screen				F2

MULTIPLE SESSIONS

When you first run NCSA Telnet, you can start up multiple sessions

with a single command.

C:\> telnet machinename [machinename ...]

Even if you do not choose multiple sessions when you first run

NCSA Telnet, you can later add sessions, as described in the

following paragraph.

Using ALT-A to Open Another Session

When you have opened a connection to a single host, you can open a

second connection, to that host or a different one. To do this,

press ALT-A (A for add). NCSA Telnet responds by prompting for

the name of a computer to which it should attempt a connection.

Indicating the Host

NCSA Telnet can only communicate with host computers that have IP

addresses. IP addresses can be looked up several different ways.

1.	Any name that is in the configuration file can be used.

(Your system administrator can tell you the names listed

there.)

2.	NCSA Telnet can be configured (by the system administrator)

to use the domain-based nameserver to look up host names. Any

name that can be resolved by the domain nameserver can be used.

An example is sri-nic.arpa.

3.	You can use the full Internet number of the machine, in

decimal, separated by periods, such as 192.17.22.20.

4.	If the machine is on the same Ethernet, you can enter a

pound sign (#), followed by the host number it uses on your

Ethernet. The host number is determined by the class of

addressing and the subnet mask (which your system administrator

can determine for you). For example, if your PC is host

192.17.22.20, you can access host 192.17.22.30 by entering #30

for the host address.

|5.	In all cases, you may follow the host name/ip address with

|an option destination port number. The port number can be specified

|in decimal  (25), octal (031) or hexidecimal (0x19). NCSA telnet will

|attempt to connect to the specified destination port on the destination

|address. If no destination port is specified, telnet uses port 23.

Response Time of the Host

NCSA Telnet attempts to make a login connection with the named

host. Usually the connection is instantaneous, and the host

machine prompts you for a login name immediately. However, the

connection attempt may take several seconds. Even after the telnet

connection has been completed, if a remote host is heavily loaded

it may take additional time (up to two minutes) to prompt you for

a login name.

| If you attempt to connect to a host name which is not in your

| configuration file, and

| if you use the domainslist feature in the configuration file, the

| resolution of the ip address (using the domain name server) may

| take a few seconds longer than if the named host was in your

| configuration file.

When the connection is established, the asterisk (*) in the status

box for the connection will disappear or change to a small box. If

NCSA Telnet makes the connection, but the remote host does not let

you log in, you may want to press ALT-X to end the connection.

Using Scrollback Mode

The SCROLL LOCK key does double duty. It prevents the screen from

scrolling when new text arrives, and also puts you into scrollback

mode. In scrollback mode, the up and down arrow keys and PG UP and

PG DN scroll the screen backwards and forwards. When you press the

SCROLL LOCK key again, it ends scrollback mode and resets the

working area of the screen.

The number of lines that are saved is limited by the amount of

memory in your machine and is set in the configuration file. If

your PC runs out of memory, or the specified number of lines have

already been stored, it will start wrapping the scrollback around.

NOTE:  No other commands work while you are using scrollback

mode.

The Status Line

Only 24 lines are needed for VT102 terminal emulation, so line 25

on your PC screen is used to indicate the connection status of

your terminal sessions. For each host you are connected to, a

session name appears in reverse video. The name in the lower left

corner of the screen is the current session. Beside each name is a

status box that indicates various attributes of a session:

(gray)		Connection is established, waiting

(small box)	Active connection, always in lower left corner

*		Connection pending, trying to connect to host

/ or \		Text has been written to this invisible session

| If you specified a destination port number when you opened the

| session, the port number will be displayed in decimal (even if you

| entered it in Hex) after the session name.

|

| If you are using 43 line mode on an EGA monitor, the status line

| will appear on line 43, and you will actually be able to use

| 42 lines on the remote host.

| The status of only four sessions can appear at one time on the bottom

| of the screen. If you have more than four sessions open, the collective

| status of the non-displayed sessions will be shown at the right hand side

| of the status line just before the clock display.

| If none of the non-displayed sessions have had text written to them,

| then the collective status will be shown as a stipled box. If any

| of the sessions have had text written to them, then the collective

| status will be shown as a / or \

+ For version 2.2TN, if the current session is a 3270 session, fewer

+ hosts will be displayed on the status line. The left most section

+ of the status line is used as the Operator Information Area

+ and displays SYS wait, Time wait, insert, numlock and caps lock

+ information. See the Tn3270 section for more information.

Switching Between Sessions (ALT-N)

The status line at the bottom of the screen indicates the active

telnet sessions. To rotate to the next session, press ALT-N (for

next). This action makes the next session the current one and

moves its name to the lower left corner of the screen. The screen

image for this session is now displayed, and you should be ready

to proceed. Characters typed at the keyboard are always sent to

the current session.

| Switching to the previous session (ALT-L)

|

| You can switch to the last shown (previous) session using the ALT-L

| key.

Displaying Console Messages (ALT-M)

In the process of connecting and communicating with other hosts

via TCP/IP, NCSA Telnet provides you with information on the

console (message) screen. To see this screen, press ALT-M. When

you have viewed the messages, press any key to return to your

session. The information on the console screen can tell you about

errors, warnings, FTP access to your PC, and network conditions

that may be of concern to network administrators.

Exiting the Present Connection (ALT-X)

ALT-X initiates the close procedure for the current session.  To

prevent this from happening accidentally, NCSA Telnet prompts you

to confirm that the connection should be closed.   After

confirmation, the connection shuts down.  The close may take

several seconds to complete.

NOTE:  You can press ALT-X to log out of a host as well as

terminate a given session, but it bypasses normal conventions and

may cause problems for the host. The correct way to close a

connection with a host is to log out of the host using the normal

logout procedure for that host.

+ALT-D

+   Copies the contents of the current screen to the capture file.

+   a Form Feed is also placed after the screen contents in the file.

+   if the capture file is open by another session then ALT-D is ignored.

CAPTURE FILE OPERATIONS

Text that appears on the screen can be captured and sent to a file

or the local printer. When you press ALT-C, capture is turned on.

Pressing ALT-C again turns capture off. Any text that appears on

the screen is captured and appended to the capture file. NCSA

Telnet never erases the capture file, only appends text to it. The

default capture file is named capfile, but you can change that, as

discussed next.

+ Capture file does not function for a TN3270 session. But

+ 7171 Transparent data mode can be captured. See the section

+ on TN3270.

+ You can use ALT-D to dump a tn3270 session screen to the capture

+ file.

Designating a Capture File

Designate a different capture file by using the Parameter menu.

When you press ALT-P the Parameter menu is displayed. (Most of

the Parameter menu choices are discussed in Chapter 3.) Use the

arrow keys to select the Parameter menu entry for the capture file

name.  Press RETURN to clear the current entry and then enter

your new file name. After the name of the capture file has been

changed, all subsequent ALT-C commands use the new capture file.

You can also change the name of the capture file "permanently" in

the configuration file (see Chapter 6).

Using PRN

If you want to send all captured text directly to a local printer,

use the filename prn as the designated capture file name. PC-DOS

reserves this filename for the printer.

A common problem can occur while you are capturing files to the

printer that can cause NCSA Telnet to abort. If the printer is

offline or out of paper when a capture begins, the message:

Error, A(bort) R(etry) or I(gnore)?

appears on the screen. At this point, turn on the printer and then

press R for retry. Do not press A at this point because it will

abort NCSA Telnet and cut off all of your network connections.

One Capture at a Time

You can capture text to a file or the printer in any session, but

not for more than one session at a time. When a capture is active

for a session in the background, you cannot invoke capture on the

current session. If you attempt to do so, you will not get the

confirmation message on line 25 and NCSA Telnet reminds you of the

error. The capture procedure will continue to capture all text for

the background session while you interact with the current

session.

+Extended Print Through Functions to Capture file

+

+Version 2.2D and 2.2TN now support the ANSI print through options

+that allow hosts programs to turn on and off capture file operations

+as well as forcing a screen dump to the capture file.

+

+	<ESC> [ P i

+

+	where P is               Does

+

+	    0		  Screen dump of the current screen to capture file

+	    4,6		  Turns off capture file

+	    5,7		  Turns on capture file

+

+All other codes for P are ignored. Screen dump and capture

+file operations only work if the capture file is not currently

+opened by a different window. A screen dump will work to a

+currently open capture file.

+

+For best results, a two second delay should be used before transmitting

+the capture file open or capture file close commands. (Due

+to the limitations of Telnet, extra information may be

+captured in the first case, and some information may be lost

+in the latter case. In all cases the Open capture file

+escape sequence will be captured in the capture file.)

CHAPTER 3

MORE ABOUT MANAGING SESSIONS

CHAPTER OVERVIEW

This chapter continues the discussion of multiple sessions and

describes more advanced aspects of the NCSA Telnet working

environment. The chapter explains how to reset the VT102 screen,

how to abort NCSA Telnet, the DOS shell feature, and Parameters

menu options.

KEYBOARD COMMANDS

Skip Command (ALT-S)

The skip command causes the screen to "skip ahead" over scrolling

text. The text is placed into the scrollback region, but the

screen update advances to the end of the local network buffer

instead of printing every line on the screen. The purpose of this

feature is to help you when you type a command which produces so

many lines of output that you don't want to wait for it to scroll

by. Press ALT-S and the screen will pause, then redraw at the end

of the local buffer. This won't solve all of the network buffering

problems for interactive use, but it should help. Capture to disk

and scrollback are not affected by skip.

Resetting the VT102 Screen (ALT-R)

ALT-R clears and resets all modes associated with the current

virtual VT102 screen. Some host programs can accidentally set

graphics mode on or fail to leave graphics mode. You press ALT-R

to override all VT102 mode settings. These include turning off

wrap mode, resetting graphics mode, setting the keypad mode back

to the default, and resetting tabs to every eight spaces. After

using ALT-R, you may wish to set your terminal type again to allow

the host to reset whatever VT102 modes it needs.

Aborting the Program (ALT-F3)

If for some reason none of the current connections respond, SCROLL

LOCK is not on, and ALT-R and ALT-X do not appear to have any

effect, ALT-F3 is a general abort command that exits NCSA Telnet

and returns you to DOS. If NCSA Telnet is aborted with ALT-F3, it

cannot notify the host(s) that your session(s) are terminated.

Your sessions are left in an undetermined state. This can cause a

variety of problems for the host, because it cannot immediately

determine that the session has ended. For example, you may have

programs which continue to run on the host, even though you exited

telnet with ALT-F3.

+ ALT-F3 does not abort Telnet if pressed in a TN3270 session.

+ This allows the key to be used as a 3270 AID key if required.

Escape to DOS Shell (ALT-E)

Pressing ALT-E to escape to DOS preserves your connections while

you use the DOS command shell. You may use most DOS commands and

programs, including those which look at directories, edit source

files, or even compile programs. To return to NCSA Telnet, you

must enter the command exit at the DOS prompt.

During the escape to DOS, NCSA Telnet is monitoring the Ethernet

for incoming packets every one-half second. If you run other

programs which affect the Ethernet or the timer which NCSA Telnet

uses, your connections may be lost or worse, your computer may

crash. The following programs are examples of those which

terminate the network handling of NCSA Telnet, causing connections

to be lost or DOS to crash:

%	SideKick (it turns off timer processes, so it can cause lost

connections)

%	All network programs (they reset the Ethernet board)

%	NCSA Telnet (it is a network program)

%	User FTP (it, too, is a network program)

%	format, the DOS format utility for floppy disks

%	FileCommand II

NOTE: Watch memory usage to prevent crashing. Also, remember to

exit when you have finished your DOS activities.

+ You can exit to DOS from the Server Mode Screen.

+ You can see how much memory is available by activating the

+ help screen, and then pressing the <RETURN> key.

PARAMETER MENU

Press ALT-P to display the Parameter menu options, which appear

on the screen as shown in Figure 3.1. These options control the

settings for text color, echo mode, backspace key, session name,

terminal type, capture file name, screen mode and file transfer

mode. You can change the fields that are shown in bold to

different values.

ALT-P                         Parameter menu

<      Select parameters, F1 to accept, F10 to leave unchanged      >

--------------- Color setup and session parameters -----------------

Text:         normal    reverse   underline

Normal Foreground (nfcolor)		-	green

Normal Background (nbcolor)		-	black

Reverse Foreground (rfcolor)		-	black

Reverse Background (rbcolor)		-	white

Underline Foreground (ufcolor)		-	blue

Underline Background (ubcolor)		-	black

Use remote echo or local echo		-	Remote echo

Backspace key sends			-	Delete

Session name				*>	ahostname

Terminal type				-	VT102 and Tek4014

-------------- Parameters which apply to all sessions --------------

Capture file name			*>	capfile

Screen mode (for BIOS compatibility)	-	Direct to screen

File transfer is			-	Enabled

+       TN3270 Transparent data goes to         -       TekScreen

Use arrow keys to select, Enter clears changeable field (*>)

Figure 3.1.  Parameter Menu

The up arrow, down arrow, HOME, and END keys allow you to move the

cursor from option to option. The left and right arrow keys rotate

through the allowed settings for each option. For each field that

you wish to change, move the cursor to that field and press the

left or right arrow until the desired value appears. After

changing all of the fields that you wish to change, press the F1

key to return to your session with the changes in effect. If you

make a mistake, or decide that no changes are necessary, press

F10 to return to your session without any changes taking place.

+ Or press the <ESC> key to return to your session without any changes

+ taking place.

There are two special fields, marked with the symbol *>, to

indicate that the present value may be changed by typing another.

To change one of these, place the cursor on top of the changeable

field and then press RETURN. The program blanks the field and

allows you to enter a new value. Press RETURN when you are

finished entering the new value. After you enter a new value, but

before you press F1, the old value still remains in memory and

you may use the left or right arrow keys to regain the old value.

Following are descriptions of the parameters you can set.

Screen Text Colors

Host programs expect to be able to control the attributes of

characters on the VT102 screen. Command codes are sent to your PC

to display characters in normal, reverse, underlined, blinking,

and bold attributes, or combinations of these. When normal,

reverse, and underline text are required, you have control over

which colors are used. When blink or bold attributes are required,

NCSA Telnet adds them to your selected colors.

For normal, reverse, and underlined characters, NCSA Telnet gives

you your choice of foreground and background colors, a total of

six settings. The color choices for a PC color video display are

black, blue, green, cyan, red, magenta, yellow, and white. For

+ Capitalized colors, BLACK, BLUE, GREEN etc are also available

+ and, if used in the foreground color, set the bold bit, if used

+ in the background color, set the blink bit.

each of the text modes, normal, reverse, and underlined, use the

Parameter menu to select the foreground and background colors. As

you change the selections, the sample text on the fifth line of

the screen provides an example of how the text will appear in the

VT102 emulation.

On monochrome screens, when you display a character in the color

blue, the PC produces a visible underline on the screen instead of

blue coloring. Color screens do not have the visible underline

capability and use a visible blue color instead of underlining.

Bold and blinking attributes from the VT102 screen appear as bold

and blinking characters on the PC screen.

+Extended Ansi Colors

+

+Version 2.2D and 2.2TN now support the ansi color definitions as defined

+in the MSDOS/PCDOS user's manual under ANSI.SYS. You can set the

+actual color of the foreground and background pallette, select bold,

+blink and invisible(foreground set to background). Reverse and underline

+still use the colors that you have defined in your config.tel file.

+Italics uses bolded underline.

+

+

Remote Echo and Local Echo

When the response time of the network is long (such as with

satellite transmission), you may want to buffer your keyboard

input locally, only sending characters to the host when you press

RETURN. This is often referred to as a line mode or local echo

mode. The alternative to line mode is called character mode or

remote echo mode, where the local program immediately sends, and

remote host echoes, every character over the network.

In local echo mode, most characters typed at the keyboard are

buffered by NCSA Telnet until you press RETURN, which sends the

characters to the host all in one packet. There are some

exceptions to this rule.

%	CTRL-U erases the local buffer.

%	Backspace (CTRL-H) erases the most recent character added to

the local buffer.

%	Tab (CTRL-I) forces NCSA Telnet to send the local buffer (with

the tab).

%	All other control characters echo with a caret (e.g. ^A) and

force NCSA Telnet to send the local buffer with the control

character included.

%	Arrow keys and all other unprintable ASCII characters, the

escape character for example, force NCSA Telnet to send the

local buffer, with the character included.

%	You cannot send a CTRL-U or Backspace to the host in line mode.

%	ALT keys and other local command keys are not affected by local

echo mode.

NOTE: For full screen editing, character mode (remote echo) is

necessary, so most hosts use this mode. When full screen editing

is not required, line mode may be more efficient.

+ Line Mode may no longer function in version 2.2D or 2.2TN

Function of the Backspace Key

NCSA Telnet automatically translates BACKSPACE keypresses into

DELETE codes, for compatibility with systems that prefer the use

of DELETE to BACKSPACE. If you find that your backspaces are not

being accepted, the host you are connected to may prefer the

reverse setting. To test this possibility, use the Parameter menu

to change the default translation so the BACKSPACE key functions

as backspace. If the result is that your backspaces are accepted,

then the host does prefer the BACKSPACE to DELETE. If you require

the reverse setting frequently, you or your system administrator

may want to permanently reset the backspace function using the

configuration file, as described in Chapter 6.

Changing the Session Name

The status line shows a 14-character session name for each active

session. For each session, the machine name that you typed to

initiate the session displays as the default. The Parameter menu

allows you to change the status line name for the current session.

Press RETURN to clear the current entry and type in a new name.

Only the first 14 characters are used.

Changing the Terminal Type

Generally, the default terminal type of VT102 with Tektronix 4014

is acceptable. There may be specific cases where you need to force

some restrictions on the way the terminal acts, so you may want to

limit the amount of emulation that NCSA Telnet does. The three

choices of terminal types are:

%	VT102 terminal with Tektronix 4014 graphics

%	VT102 only, graphics commands ignored, but VT102 commands

accepted

%	Dumb terminal, no VT100 codes or Tektronix codes interpreted

+ You can not change the terminal type to 3270. The remote host

+ must negotiate the request for 3270 emulation.

Changing the Capture Filename

The name of the current capture file can be changed on the

Parameter menu. Press RETURN to clear the current entry and then

enter your new filename. After the name of the capture file has

been changed, all subsequent ALT-C commands use the new capture

file. You can also change the name of the capture file

"permanently" in the configuration file (see Chapter 6). See

Chapter 2 for a description of capture file operations.

File Transfer Mode

The Parameter menu allows you to disable or enable file transfers.

When you select Disabled, neither FTP nor rcp is allowed. When you

select Enabled, the FTP and rcp servers are restarted. Chapter 4

contains more information on the file transfer methods for NCSA

Telnet. The default file transfer modes for FTP and rcp can be set

in the configuration file (see Chapter 6).

Screen Mode (BIOS Compatibility for Windowing

Packages)

In the normal, fast mode, writes are made directly to the screen

for increased speed, but this mode is incompatible with windowing

packages like Topview or Microsoft Windows and may cause snow on

some screens. NCSA Telnet has a compatibility mode where IBM's

BIOS calls are used for all access to the screen.

Usually, the windowing compatibility mode is set up in the

configuration file with the bios option. However, if a change is

needed once you are in the program, the Parameter menu option is

available.

+ The TN3270 transparent data mode only appears if the current session

+ is in 3270 emulation mode. It allows the user to direct a

+ 7171 transparent data stream to either the TEK screen (default),

+ the VT102 Screen or to the capture file. (You must set the name of

+ the capture file in the capture file name section.)

+ There is no way to set this in the config.tel file.

CHAPTER 4

FILE TRANSFER

CHAPTER OVERVIEW

This chapter describes procedures for transferring files between a

PC and a network host, using NCSA Telnet's two built-in file

transfer servers: FTP and rcp. Guidelines and information are

presented for file name conventions, path specification, and

background file transfer.

TERMINOLOGY

The following terms are used in this chapter.

ASCII file, text file

An ASCII or text file can be read by humans, and can be used with standard

editors on the PC or host. When text files are transferred, the end-of-line

markers are changed.

Binary, graphics, or IMAGE file

A binary, graphics, or IMAGE file cannot be read by humans. When

transferred, binary files are not changed in any way (unlike text files).

Client/Server

The client is the system that requests services and the server is the system

that provides them. The client is not always your PC, despite appearances.

When you use NCSA Telnet to connect to a host, your PC is the telnet client.

When you request a file transfer from your PC, the transfer is actually

initiated on the host, making the host the FTP client and your PC the FTP

server. So the PC is a telnet client and an FTP server at the same time.

File transfer

In a file transfer, the contents of a file are copied, and the copy is

placed on another computer's disk drive.

FILE TRANSFER GUIDELINES

Full Path Specification, Conversion of Forward Slashes

With either FTP or rcp, if you do not specify a full pathname,

files are transferred to the default directory. If you need to

specify a different directory, type the full pathname as

documented in the DOS manual. The full pathname may include the

disk drive name, with a colon, for example:

D:\myfiles\graphics\image.dat

or

D:/myfiles/graphics/image.dat

You may use forward slashes (/) rather than backslashes (\),

because NCSA Telnet automatically converts any forward slashes in

UNIX pathnames to backslashes for compatibility with PC-DOS.

Because backslashes are special characters in the UNIX shell and a

special format is required to enter them, forward slashes are more

convenient to use.

Some Simultaneous Activity Permitted

With both FTP and rcp, file transfers are processed at the same

time as other telnet sessions. Therefore, while a file transfer is

in progress you can perform other NCSA Telnet activities such as

interacting with a telnet session, switching sessions, adding new

sessions, or changing parameters.

However, do not initiate another file transfer while one is

already in progress. While an FTP request is being processed, a

second FTP request will be ignored. However, if you try to start

an rcp transfer while another transfer is already underway, both

transfers may fail.

NOTE: Do not quit NCSA Telnet while a file transfer is in

progress, or the file transfer may fail.

Rules for Filenames Sent to the PC

Whether you are using FTP or rcp to transfer files to or from a

PC, check these rules for PC filenames:

%	A legal PC filename is eight characters or less, followed by an

optional period and an up to three-character extension.

%	Upper- and lowercase are considered identical for filenames on

PC disks.

%	Names of files which are transferred from the PC are given in

all lower case.

%	A filename longer than eight characters is truncated.

%	If a period occurs in a filename, the next three characters are

read as the extension, and any others are truncated.

%	A filename should not contain "special characters" or spaces.

If it does, the file may be lost or unreadable. Examples of

special characters are the asterisk (*), the dollar sign ($),

the pound sign (#), the hyphen (-), and control characters.

File Transfer Reports

The FTP server produces several kinds of information for the local

user to help keep tabs on FTP access to the local hard disk.  This

information can be viewed with the ALT-M command (show console

messages), as documented in Chapter 2.  The following list

describes the information which FTP provides:

%	Reports when a command connection is initiated, with the IP

address or host name of the remote machine which initiated the

connection

% 	Reports the name of the user who logs in, if the USER

command is used

% 	Reports the file names which were transferred to or from

local disk

% 	Reports when the command connection ends

The rcp server also produces reports for the console screen.

%	Reports when an rcp transfer begins

%	Reports when the rcp transfer ends

FILE TRANSFER USING FTP

The FTP server in NCSA Telnet is the minimum standard FTP server,

similar to that in 4.2 BSD UNIX. NCSA's FTP supports the following

features:

%	Stream transfer in text format (ASCII) or binary format (IMAGE)

%	Change directory

%	Print current directory

%	List files in current directory (with wildcard specifications)

%	Send and receive multiple files with one command, using

wildcards

Prerequisites

%	The host machine must support FTP file transfer. If you do not

know whether it does, see your system administrator.

%	You must not have disabled the file transfer capability of NCSA

Telnet. That is done with the Parameter menu (Chapter 3), or in

the configuration file (Chapter 6). If the file transfer

capability has been switched off, when you attempt to start up

FTP, you will get a message from the host to the effect that

the computer is not responding. If you get such a message,

check the file transfer mode on the Parameter menu to make sure

that it is enabled.

When to Use FTP

Use FTP rather than rcp in either of the following situations.

1.	You are not thoroughly familiar with the host operating

system. FTP is easier to use than rcp, and requires less

familiarity with the host commands and directory structure.

2.	You are transferring PC text (ASCII) files to a host. FTP is

a better choice than rcp, because FTP automatically converts

the end-of-line characters in text files from CRLF used by PC-

DOS, to LF used by UNIX machines.

Invoking FTP on the Host Computer

FTP is initiated by the host, so the FTP commands vary, depending

on the host system. For full documentation of FTP and commands

within FTP, you must refer to the manuals for the host computer.

With UNIX systems, you can see online documentation by using the

man ftp command.

THE FTP COMMAND

On most systems, the FTP command is entered at the prompt, with

the name or IP address of the target machine. For example, if your

PC is named "mymachine" and your IP address is 192.17.20.22, then

you might enter:

% ftp mymachine

or

% ftp 192.17.20.22

which generates a response like this:

Connected to 192.17.20.22.

220 PC Resident FTP server, ready

Name (192.17.20.22:timk):

Most FTP clients will now prompt you for your username and

password.  If NCSA Telnet is configured for passwords (see Chapter

6), then these are required.  Otherwise, just press RETURN to

bypass the prompts.  If you are  not prompted for username and

password, then assume that you are logged in and continue with

your FTP commands.

|If your version of Telbin has FTPPW enabled (see your administrator)

|You will be able to use the ALT-W key to transmit an internal

|random password that will work for ANY user name entered.

|See chapter 6 for more information on using ALT-W.

ALTERNATE METHODS OF INVOKING FTP (ALT-T AND ALT-I)

The most common procedure for starting FTP uses the shortcut

keystroke, ALT-T. When you press ALT-T, NCSA Telnet types the

ftp command described above, automatically including your PC's IP

address and the RETURN to initiate the command.  For convenience,

you will probably prefer ALT-T to the other ways of entering the

command. You may want to think of this as a keyboard macro that is

pre-set with the FTP command and your IP address.

Another available macro is ALT-I which types your IP address for

you.  When entering any networking command, FTP for example, you

can instantly produce your own network IP address with ALT-I.

The following sequence is equivalent to pressing ALT-T.

ftp

(SPACEBAR)

(ALT-I)

(RETURN)

Use whichever method of invoking FTP you feel comfortable with.

Your host computer may or may not accept FTP commands as described

here, so you may have to try some variations to find the easiest

method for your site.  Your system administrator may be able to

help.

+ The ALT-W, ALT-T, ALT-I keys do function as expected under 3270

+ emulation mode if you have version 2.2TN

FTP Commands

For most FTPs, after FTP has been invoked and passwords have been

checked, you are prompted for individual FTP commands.  These

commands are also documented in the manuals for the host computer.

Most of the FTP implementations have similar commands because they

are modeled after the Berkeley UNIX version of FTP. Table 4.1

lists FTP commands that are common to most implementations.

Table 4.1.  Common FTP Commands

Command		Action

ascii		Sets mode to ASCII transfer mode (default)

binary		Sets mode to binary (image) transfer mode

cd path		Sets a new default directory on PC

dir		Shows filenames in PC's default directory

get filename	Gets a file from PC, sends to host

help		Shows online list of FTP commands

put filename	Sends a file from host to PC

pwd		Shows current PC directory name

quit		Exits FTP

ASCII, BINARY

The default mode for FTP transfers is ASCII format. If you are

transferring graphics or binary data files, change to binary mode

before you use the put or get commands. Do this by entering the

command binary. To reset the ASCII format after sending a binary

file, enter the command ascii. Figure 4.1 shows an FTP

transaction with an ASCII file. Figure 4.2 shows an FTP

transaction with a binary file.

DEFAULT DIRECTORY

Unless you include a full pathname with the command, NCSA Telnet

transfers the file to the default directory. As Table 4.1

indicates, you can use FTP commands to identify the current

directory (pwd), or change the current directory (cd).

The change directory command, which the user types as cd, allows

you to change the default disk drive in addition to the default

directory.  Just use PC standard disk notation, with forward

slashes instead of backslashes, for example:

ftp> cd D:/files

The pwd command also returns disk information.  After the previous

example cd command, a pwd command returns:

D:\FILES

TRANSFER TO THE PC

Even though you seem to be initiating the transfer from the PC,

the transaction operates from the host's side. The practical

effect of this could make the commands seem

intuitively "backwards." To transfer a file from the host to your

PC, you use a put command. The form this command takes is:

put filename.ext

Figure 4.1 shows an example of a put command used with an actual

file, named temp2. The boldface type represents user entries.

newton_45% ftp -n 192.17.20.124

Connected to 192.17.20.124.

220 PC Resident FTP server, ready

ftp> put temp2

200 This space intentionally left blank <  >

150 Opening connection

226 Transfer complete

262145 bytes sent in 32.61 seconds (7.8 Kbytes/s)

ftp> quit

221 Goodbye

newton_46%

Figure 4.1.  Put to the PC, ASCII File

TRANSFER TO THE HOST

A request to send a file from the PC to the host is called a get.

get filename.ext

Figure 4.2 shows a get operation, using a binary file named

bridge.pic. Note that the file was in the directory named ibmg, so

the cd command was used. Again, the boldface type represents user

entries. If a text file were to be sent after this, the ASCII mode

would have to be reset, using the FTP ascii command.

newton_41% ftp -n 192.17.20.124

Connected to 192.17.20.124.

220 PC Resident FTP server, ready

ftp> bin

200 Type set to I, binary transfer mode

ftp> cd /ibmg

250 Chdir okay

ftp> get bridge.pic

200 This space intentionally left blank <  >

150 Opening connection

226 Transfer complete

262144 bytes received in 9.22 seconds (28 Kbytes/s)

ftp>

ftp> quit

221 Goodbye

newton_42%

Figure 4.2.  Get from the PC, Binary File

FTP STATUS INDICATOR

After you have entered a put or get command, in the lower right

hand corner of your screen you will see the filename and an

indication of the number of bytes transferred (in the case of a

put) or left to be transferred (in the case of a get). These

numbers are updated every couple of seconds to help you keep track

of the progress of the file transfer.  After the command has

completed, the filename goes away and the host usually prints a

confirmation message, as seen in Figures 4.1 and 4.2.

MPUT, MGET

On many telnet hosts, you can transfer multiple files sequentially

with one command, either mput or mget, used with wildcard

characters. (PC wildcard characters are ? for a single character

and * for multiple characters.)

BUG WITH MGET

If you transfer multiple binary files using a UNIX host, note that

there is a bug in mget as implemented on some systems (especially

4.2 BSD UNIX). When used in binary mode, mget adds a carriage

return to the filenames as they are transferred. The files

themselves are not affected. Use a UNIX utility to remove the

carriage return from the filename. In ASCII mode, there is no

problem.

FILE TRANSFER USING RCP

rcp is a Berkeley UNIX feature found in Sun UNIX and other 4.2 BSD

systems. The rcp program cannot do the end of line translation

required for text files, so it is typically used for binary files

only. UNIX rcp is designed for files copied UNIX-to-UNIX, but NCSA

Telnet's rcp server supports transfer between a UNIX system and a

PC.

NOTE: NCSA Telnet's rcp server does not support the recursive

option (-r).

Prerequisites

To use rcp efficiently, you should be familiar with the UNIX host,

including:

%	rcp command and options

%	directory structures used with rcp

%	the relationship between the UNIX shell and rcp wildcards

Details on these concepts are in the manuals for UNIX.

The internet (IP) number and machine name for each PC must be in

the UNIX /etc/hosts file for every machine that is transferring

files to and from your PC.  If rcp does not recognize your PC's

name, have your system administrator check for that name in the

UNIX machine's hosts file.

When to Use Rcp; Advantages of Rcp

Use rcp for binary transfers to or from a UNIX system if you are

familiar with the UNIX hosts, as described earlier.

There are two advantages of using rcp rather than FTP, on binary

file transfers to or from a UNIX machine.

%	rcp handles wildcards more efficiently than FTP does.

%	rcp handles directories more easily than FTP does.

When Not to Use NCSA Telnet's Rcp Server

Do not use the rcp server to transfer text files. Text files on

the PC contain extra RETURN characters that NCSA Telnet's rcp

server does not remove when transferring to UNIX.

NOTE: Do not initiate an rcp while a file transfer is already in

process, or both transfers may fail.

Rcp Between the PC and UNIX

The rcp program is fully documented in the manuals for UNIX hosts

that support rcp transfers.  Refer to those manuals to learn about

rcp.  As a reminder, the general syntax of the rcp command is

given here.  The target filename can be a directory name and

wildcards are allowed, as noted in the next section.

% rcp mypc:filename unixfile		From PC to UNIX.

% rcp unixfile mypc:filename		From UNIX to PC.

EXAMPLE: PC TO UNIX

Following is a sample command to transfer a file named image from

the PC named mypc to the current directory on the UNIX host

(indicated by a dot or period). Note that the only blank spaces

are after the command rcp, and between the filename and the target

directory name (in this case, the dot for the current directory).

% rcp mypc:image .

EXAMPLE: UNIX TO PC

Following is a sample command to transfer a file named mygraf in

the directory graphs from the UNIX host and put it in the

directory grafs on the PC.

% rcp /graphs/mygraf mypc:/grafs

Using Wildcards

Wildcard characters can be used with the rcp command to transfer

multiple files between a PC client and a UNIX host. Wildcards that

refer to UNIX files should be in the UNIX regular expression

format. Wildcards that refer to PC files must be set apart using

backslashes or quotes. See the following examples.

EXAMPLE: BACKSLASHES USED WITH WILDCARDS

One way of using PC wildcards with the rcp server is to use the

backslash character (\) to keep the UNIX shell from interpreting

the special characters. Suppose you want to transfer all files

that have image at the beginning of the filename. Instead of the

usual PC format, image*.*, you must precede each asterisk with a

backslash. The full rcp command would look like this:

% rcp mypc:image\*.\* .

Without the backslashes, the UNIX shell would try to translate the

asterisk on the host rather than on the PC.

EXAMPLE: QUOTES USED WITH WILDCARDS

The second way of using PC wildcards with the rcp server is to

insert quotes around the entire argument. The command to transfer

the files beginning with image would look like this:

% rcp "mypc:image*.*" .

CHAPTER 5

TEKTRONIX 4014 GRAPHICS

CHAPTER OVERVIEW

NCSA Telnet can emulate the Tektronix 4014 graphics terminal. This

emulation includes text modes, Tek 4014 text sizing, zoom, pan,

and multiple output devices. It supports CGA, EGA, Hercules, and

the Number Nine Revolution 512x8 video systems, with the

capability to write out images in PostScript, HPGL, or Tek drawing

commands. The use of Tektronix graphics with NCSA Telnet depends

upon host programs that can produce graphic images. When these

programs run and produce Tektronix 4014 graphics commands, NCSA

Telnet automatically switches into graphics mode and does the

drawing.

This chapter describes steps in preparing to emulate the Tektronix

4014 and how to use the Graphics menu.

BEFORE YOU BEGIN TEKTRONIX EMULATION

Initializing a Plotter

If you will be plotting directly to a plotter, the communications

port for the plotter must be initialized before you run NCSA

Telnet. This is done using the DOS mode command. See the DOS

manual and the plotter manual for examples.

Configuration File Settings

The configuration file is used to install the hardware and

software configuration for Tektronix graphics. There are several

settings you should insert in the configuration file, detailed in

Chapter 6, before you use Tektronix emulation.

1.	You should include tek=yes in the configuration file to

enable graphics operation.

2.	The video type is specified with the video parameter in the

configuration file. This is the only step required for

selecting the video display because all of the screen drivers

are included in NCSA Telnet.

3.	You should install specific filenames to be used when

writing each of the three different kinds of drawing commands.

The hpfile parameter selects the file to receive HPGL commands,

the psfile option selects the file to receive PostScript

drawing commands, and the tekfile option selects the file to

receive Tektronix drawing commands. If you do not make any

settings here, the default files are hp.out, ps.out, and

tek.out.

4.	If you want HPGL output to go directly to a plotter that is

connected to COM1 rather than to a disk file, add hpfile=com1

to your configuration file. Note that COM1 is used without a

colon.

SWITCHING TO AND FROM GRAPHICS MODE

Graphics Mode

When host programs produce Tektronix 4014 graphics commands,  NCSA

Telnet automatically switches into graphics mode and does the

drawing.  The exact command sequence which causes the switch to

graphics mode is the Tektronix clear screen command, ESC-FF, which

is sometimes written as ESC-CTRL-L. A copy of your text screen is

saved while you are in graphics mode.

The second way to enter graphics mode is to press CTRL-HOME.

This key switches from text mode to graphics mode and clears the

graphics screen. Host programs that produce Tektronix 4014

graphics commands cause the screen to automatically go to graphics

mode, so you will not usually need to use CTRL-HOME to switch to

graphics manually.

The third method of starting graphics mode is to re-display the

last graphics image. The re-display option is from the Graphics

menu, described later in this chapter. At the completion of the

re-display command, the screen remains in graphics mode.

Text Mode

To return to text mode, press HOME. When you are in graphics

mode, HOME will set your display back to text mode and redraw the

text screen which was visible before the graphics mode began.

There is no host command which automatically returns to text mode.

When you are in text mode, HOME has a different function. See

Chapter 2 for details.

| If you are using 43 or 35 line mode on an EGA or VGA display

| telnet will correctly switch back to text mode. However,

| you can instruct telnet to use 51 line mode under MS-WINDOWS

| or any number of lines w/ any video card. In these cases

| telnet does not know how to return to the proper text mode.

| This may cause you to lose 51 lines on the screen depending

| on your configuration. Telnet only fully understands EGA/VGA

| and 43 or 35 lines, and will switch between text and graphics mode

| correctly for that configuration. Other combinations of

| video cards and lines (other than the default 25 lines) are

| not supported and may produce unpredictable results.

GRAPHICS MENU (ALT-G)

The Graphics menu is started with ALT-G. Figure 5.1 shows the

Graphics menu, as it appears with the default file names. The

fields in bold can be changed by the user.

ALT-G                           Graphics menu

<            Press the appropriate function key or ESC to resume        >

F1 - Write postscript to a file called: ps.out

F2 - Change postscript output file name

F3 - Write HPGL code to a file called: hp.out

F4 - Change HPGL output file name

F5 - Write Tektronix 4014 codes to a file called: tek.out

F6 - Change Tektronix output file name

View region is currently: 0,0,4095,3119

F7 - Set a new view region (Zoom, Pan)

RETURN - draw picture on screen in current zoom factor

Enter choice:

Figure 5.1. Graphics Menu

From this menu, you press the appropriate function key, or RETURN

to redraw the most recently displayed graphics picture.

Writing Graphics Files to Disk

To write out graphics images use the Graphics menu. First, the

image should be displayed on the screen using host software that

generates Tektronix images. NCSA Telnet always keeps the last

graphic image in memory. Select one of the options which writes

this image to disk. Images are written using the current view

region.

If you select Tektronix 4014 format, NCSA Telnet writes the image

directly to disk and then returns you to your current session.

After selecting the HPGL or the PostScript option from the

Graphics menu, you are returned to your session while the file is

written to disk in the background. After this image has been

completely copied from memory to disk, you are notified on the

console screen that the process has completed.

Renaming Graphics Output Files

If you did not set up alternate files to use in the configuration

file and you do not want to use the default filenames hp.out,

ps.out, or tek.out, you can change the names before or after

writing the file. Use the Graphics menu options to change one of

the current file names before writing the image to disk. If you

want a name change after writing out the file, use ALT-E to escape

to DOS, then use the DOS rename command. Use exit to leave DOS and

return to NCSA Telnet.

A special feature allows you to append the current image to an

existing disk file instead of overwriting.  Enter any of the three

file names as you would normally, adding a plus (+) to the

beginning of the name.  NCSA Telnet takes the plus (+) to indicate

that you want to open that file for "append", adding another image

to the file.  A filename which does not begin with a plus (+),

causes NCSA Telnet to overwrite the file when saving each image.

Set the Viewing Region (Zoom Factor)

Another Graphics menu choice lets you set the viewing region, or

zoom factor. Any portion of the image can be enlarged and made to

fill the screen. Tektronix drawing commands are resolved to a 4096

by 3120 pixel resolution. They start with 0,0 in the lower left,

and go to 4095, 3119 in the upper right.

The default view region for NCSA Telnet is (0, 0, 4095, 3119),

which is the full image. Figure 5.1 has examples of possible

viewing regions that you can use.

Figure 5.2. Sample Viewing Regions

Redraw Last Image

The Graphics menu option, Redraw Last Image, is used when you want

to review the image that is currently in memory. This option

always zooms and enlarges the image according to the view region

that is visible. The selected view region always fills the screen

so you can see more detail.

This command leaves you in graphics mode, so press HOME if you

want to return to text mode.

PLOTTING A STORED FILE

If you have used the Graphics menu to store a file of HPGL drawing

commands, the DOS copy command can send those commands to an

attached plotter (which is connected to COM1 in this example).

C:> copy filename.out com1

+ Graphics under 3270 emulation.

+ Tek4010 commands do function under 3270 emulation. Your host software

+ must open a 7171 transparent data stream to your terminal, AND you must

+ have transparent data directed to the TekScreen (default). SAS Graph

+ supports 7171 datastreams by including the following statements

+ in the SAS command file.

+

+ GOPTIONS NOTEXT82 GPROTOCOL=GSAS7171 GEPILOG='18'X GPROLOG='1B00'

+

+ Your local SAS options may vary.

+

+ After displaying a graph, the user must press ALT-R to reset

+ the screen before entering any 3270 AID keys. This is because there

+ is no way for telnet to know exactly when the graph is done.

CHAPTER 6

INSTALLATION AND CONFIGURATION

CHAPTER OVERVIEW

This chapter contains information for system administrators (and

other experienced users) to use in installing and customizing a

system. The chapter describes how to install NCSA Telnet, change

the configuration file, the domain name lookup, how to protect FTP

transfers with passwords, and compatibility issues.

INSTALLING THE DISTRIBUTION FILES

There are three files required for the standard configuration of

NCSA Telnet. The combination of these with the use of the DOS path

command allows you to call up NCSA Telnet from any subdirectory on

your PC.

TELNET.BAT

This batch file begins the execution of NCSA Telnet. It must be in some

directory that is in your DOS path. It contains the locations of the

TELBIN.EXE and CONFIG.TEL files. You must update this batch file (with any

standard editor) when you change the location of TELBIN.EXE or CONFIG.TEL.

| Optionally, you can set an environment variable (CONFIGTEL) to point

| to the location of your config.tel file. You can then rename

| telbin.exe to telnet.exe and not use a batch file at all.

TELBIN.EXE

This is the program itself. It can be in any directory. The TELNET.BAT file

calls up this program when it runs.

CONFIG.TEL

This is the configuration file that contains your PC's IP address, the kind

of Ethernet hardware that you are using, and other information about your

network. Gateways, nameservers, and setup information are stored here. The

TELNET.BAT file tells TELBIN.EXE where it can find this file on your disk.

Sample Configuration

Suppose you want to install NCSA Telnet in a directory called

\ncsa on your hard disk drive, drive C. Copy the TELBIN.EXE and

CONFIG.TEL files to C:\ncsa. Suppose also that you have a

particular directory C:\bat, which you use to store commonly used

batch files. Your DOS path (see DOS manual) contains an entry for

C:\bat already. Copy the TELNET.BAT file into C:\bat. You must now

edit the TELNET.BAT file to update the locations of the TELBIN.EXE

and CONFIG.TEL files. When finished, it should look something like

this:

if .%1 == . goto defh

c:\ncsa\telbin -h c:\ncsa\config.tel %1 %2 %3 %4 %5

goto exith

:defh

c:\ncsa\telbin -h c:\ncsa\config.tel ncsab

:exith

This batch file calls up TELBIN.EXE at a known location and passes

it the -h parameter, indicating the location of the CONFIG.TEL

file. In addition to the file locations, the fifth line indicates

which machine NCSA Telnet should connect to by default.

| Second example:

| Suppose you placed your config.tel file in  C:\NCSA\CONFIG.TEL

| You can add the following line to your autoexec.bat file

| 	set CONFIGTEL=C:\NCSA\CONFIG.TEL

| And rename TELBIN.EXE to TELNET.EXE

| From then on (after rebooting to set the environment) telbin.exe

| will use C:\NCSA\CONFIG.TEL unless you also specify the -h

| option on the command line.

| In this example you will NOT need TELNET.BAT, you can use TELBIN.EXE

| directly or rename TELBIN.EXE to TELNET.EXE

Required Configuration Information

To set up a new configuration file, you will need a standard text

editor. The template provided with NCSA Telnet is set up to

reflect commonly used default values. It may be worth the time to

go through the list of configuration options in this chapter and

fine-tune your configuration to match your local computing

situation more closely. In the meantime, the following fields in

the configuration file must be installed for NCSA Telnet to

operate.

myip=192.17.20.10

You must provide a unique IP address for

each machine on your network. This must be

installed in the configuration file of

each computer.

netmask=255.255.255.0

Subnet mask, in decimal. This field is

required if you are using subnets on your

network.

hardware=3C501

NCSA Telnet can support several different

brands of Ethernet adapters. Each brand

has its own unique identifier which must

be set for the hardware parameter. See the

hardware section in this chapter for a

complete list of settings. Most brands

also require address and ioaddr to be set.

video=ega

If you wish to use the Tektronix graphics

options, you must provide the correct

screen type. Incorrectly installed, this

can crash NCSA Telnet. The alternative is

to install the tek=no option. Legal types

are cga, ega, hercules, no9.

| Or video=auto. In which case telnet will attempt to automatically

| deduce your video card type.

gateway=1

If you are going to be using gateways to

reach other networks, at least one machine

must be designated as a gateway, using

this gateway option after the name field.

The entry for each gateway must include a

hostip field, according to the

configuration file directions in this

chapter.

The TELNET.BAT File

The actual .exe file that the PC executes is not named telnet.exe.

A batch file called TELNET.BAT loads the software in several steps

that are normally transparent to you. Here is a typical entry in

TELNET.BAT:

C:>\apps\telbin -h c:\apps\config.tel %1 %2 %3 %4

The TELNET.BAT file, which must be in the DOS path, specifies

completely where the program is, and where the configuration file

is to be found. There may also be a default machine name that the

system connects to when you do not explicitly specify one.

Command Line Options

Telnet goes through two stages as you load it. The TELNET.BAT file

must be in the DOS path so it can be found first. In the batch

file, the location of the TELBIN.EXE file and any command line

defaults are used to call up the program in the next stage. Many

command line arguments are included for you in the batch file,

TELNET.BAT; you add others when you run the telnet batch file.

This is a full description of those options. The syntax for

loading NCSA Telnet with options is:

C:\ > telnet [options] [machine1 machine2 ...]

The options must precede the list of machine names on the command

line. Options can be in any order; parameters on options are

required. There must be one space between the options and between

each option and the parameter for that option. The options are:

?

Shows command line options. This causes NCSA

Telnet to display a summary version of the

command line options instead of initiating the

program.

-c colorcode

Sets the default color for screens. The

colorcode string is six digits long and follows

the coding scheme documented later in this

chapter.

-h filename

Specifies where to find the configuration file.

Normally, this option is included in TELNET.BAT,

and you never have to use it explicitly. If the

configuration file is not specified there or on

the command line, there must be a file named

CONFIG.TEL in the default directory.

| Or you must have an environment variable CONFIGTEL set to point to the

| location of the config.tel file.

If a

suitable configuration file cannot be found

according to these rules, NCSA Telnet

terminates. NOTE: filename should include a

pathname. For example,

C:\bat\config.tel

-s

Enters server mode. NCSA Telnet waits for

external FTP and rcp requests. It allows you to

leave your PC and access files there from a

remote machine.

-t

Disables direct writes to the screen. Without

the -t option, NCSA Telnet defaults to writing

directly to screen memory. This greatly enhances

the screen speed, but makes NCSA Telnet

incompatible with Topview, Microsoft Windows,

and other windowing systems. To make NCSA Telnet

completely compatible with BIOS-level programs,

use the -t option. This option can be

permanently installed in the TELNET.BAT file or

the CONFIG.TEL file (bios option).

| -l nn

|

| Specify the number of lines on the screen.

| You can use any number from 25 to 70. If you have an EGA/VGA card

| and you specify  -l 43 or -l 35, telnet will automatically switch the video

| adapter into 43 or 35 line mode (in which case you will be able to use

| 42 lines in your host session.

| If you do not have an EGA/VGA or you specify something other than 43 or 35

| lines, telnet will not do anything with the video adapter, but will

| assume that you really have nn lines on the screen. (Such as

| -l 51 under MS-windows on VGA)  In this case you may have difficulty

| returning to Text mode after displaying graphics.

+

+ -n  causes telnet to not switch video modes if run with an EGA display

+     and -l 43 or -l 35 has been specified.

+

+ -d nn    Set the screen saver timeout to nn minutes.

+          The default is five minutes. The maximum is one hour.

+          Setting the timeout to zero disables the screen saver.

THE CONFIGURATION FILE

Overview of the Configuration File

The configuration file replaces the hosts file of NCSA Telnet

Version 1.2. This file contains information on local operating

parameters (such as which type of graphics screen your PC has, and

your IP number), plus a list of commonly accessed hosts and

optional network tuning parameters for each of those hosts.

The configuration file is accessed once when the program is

initiated and is not used again. All of the machine names are read

into memory, so it may save memory to limit the number of

computers you specify in the file.

The configuration file is typically named CONFIG.TEL. If this file

is in the default directory, it will be found automatically, but

the use of the -h flag in the TELNET.BAT file is recommended.

Syntax for Entries in the Configuration File

The configuration file is a list of keywords and values for those

keywords. The one overall requirement for the entries in the file

is that they alternate, keyword then value, keyword then value,

and so on. There are many different formats possible using any of

the allowable delimiters. The delimiters are the colon (:),

semicolon (;), equal sign (=), and any of the whitespace

characters. To include delimiters in a value field, enclose the

field in double quotes. Quotes cannot be a part of the actual data

field. Wherever a pound sign (#) is found, everything from there

to the end of line is a comment. Though multiple formats can be

used in the same file, you will probably want to find and keep a

consistent format. For example, each of the entries in Figure 6.1

specify the same information.

Note that example #3 is not a recommended format, but is included

to show how strange an entry may look.

name=nic      # comment field to end of line ->

host=sri-nic.arpa

hostip=10.0.0.51

scrollback=300

contime=60

- - - - - -Example #1- - - - - - - - -

name=nic; host=sri-nic.arpa; hostip="10.0.0.51"; scrollback=300; contime=60

- - - - - -Example #2- - - - - - - - -

name

nic

host sri-nic.arpa : hostip=10.0.0.51; scrollback=300; contime:60

- - - - - - Example #3- - - - - - - - -

Figure 6.1.  Displaying the Same Information in Different Entry Formats

PC Information Entries in the Configuration File

The first entries in the configuration file are the PC environment

entries. They specify what types of hardware are to be used, names

of graphics output files, and network configuration information.

Here is the list:

myip=10.0.0.51

This is the IP address to use for the PC.

This number must contain all four parts of

the IP address. This field is required.

myip=RARP

This is a special form of the myip entry.

Setting myip to RARP will cause NCSA

Telnet to query a RARP server to get its

IP address. See the section on RARP in

this chapter.

| myip=bootp

|

| This is also a special form of the myip entry.

| This will cause NCSA Telnet to query a BOOTP server on your local

| network to determine your ip address, netmask, nameserver and gateways.

| If you use this option, you will not need a netmask, nameserver or

| gateway statement, though you may enter them if you wish.

| The netmask will be over-ridden by BOOTP, but nameserver and gateways

| will be cumulatively added.

netmask=255.255.255.0

This is the subnet mask for your local

network. It is optional for networks

without subnets.

arptime=3

Specify how long to try to reach a host on

the local wire, in seconds. A value of 2

works fine for the network at NCSA, but

larger values may be needed for hosts that

are slow to respond. Smaller values are

more convenient to use.

|domain="ncsa.uiuc.edu"  	*** Super-seeded by domainslist, see below

|This entry in the configuration file

|affects hostname lookups which are sent to

|the domain nameserver. All names which do

|not contain a period have the default

|domain appended before the nameserver

|request goes out. If the hostname you type

|contains a period, then the name is

|unmodified before NCSA Telnet sends it to

|the nameserver. There is no "ring-down" or

|multiple lookup feature to try more than

|one form of the name.

|domainslist="soe.clarkson.edu,clarkson.edu,umd.edu"

|

| This new feature allows multiple domain name searching when doing a

| host name lookup.  Each subdomain must be seperated by commas with

| no extra spaces.

| In the above example, an entered hostname of  sun  will be matched

| against   sun.soe.clarkson.edu, sun.clarkson.edu, sun.umd.edu and sun

| An entered name of pebbles.erc will be matched against

| pebbles.erc.soe.clarkson.edu, pebbles.erc.clarkson.edu  ... and so on.

| Any entered name with a trailing dot (.) will not be matched using the list

| but rather will be searched for directly. IE: an entered name of

| sri-nic.arpa.  will only look for sri-nic.arpa.   If the trailing

| dot (.) is left off by accident, then all three sub-domains will be searched

| as described above.

|

| The more entries you place in the search list, the longer it will

| take to resolve if you mis-type an entry. The best choice is usually

| your local sub-net.domain  and your local domain. IE:

| If you usually connect to hosts within the soe sub-domain of clarkson.edu,

| you should probably use  domainslist="soe.clarkson.edu,clarkson.edu"

|

| It is not a good idea to place top-level domains in your search list.

| IE: do not do this  domainslist=".com,.edu,.arpa"

domaintime=2

Time in seconds to wait between the first

domain lookup and the second. If you only

have one nameserver, then this is the same

as a simple timeout. If you want to rotate

nameservers quickly because the first one

may be down, set this to a smaller number.

To force a larger number of retries, set

this to a smaller number. To wait longer

for a domain response, increase this

value.

domainretry=4

Number of times to query domain

nameserver(s). Each time a retry is sent,

the timeout value (above) is doubled. Each

time a retry occurs, NCSA Telnet tries the

next nameserver, wrapping around to the

first nameserver when there are no more.

tek=yes

There is no overhead to unused Tektronix

graphics emulation, but you may wish to

disable graphics support. tek=no will

disable Tektronix graphics emulation.

video=ega

Tektronix 4014 graphics emulation is

included in NCSA Telnet for a variety of

video devices. Supported are: hercules

(Hercules monochrome graphics), cga (IBM

and compatibles original Color Graphics

Adapter), ega (IBM Enhanced Graphics

Adapter and compatibles), and no9 (Number

Nine Computer's Revolution 512 x 8 board).

| You may also set video=auto, in which case NCSA Telnet attempts to

| automatically determine your video card type.

|

bios=no

NCSA Telnet has two screen-writing modes.

One is direct-to-screen (bios=no) which is

considerably faster on most machines. The

other uses BIOS (bios=yes) and follows the

IBM BIOS conventions at all times. BIOS

mode makes NCSA Telnet compatible with

popular windowing systems and can reduce

the amount of flicker and snow on the

screens of IBM CGA-equipped systems.

hardware=3C501

NCSA Telnet can support several different

brands of Ethernet adapters. Each brand

has its own unique identifier which must

be set for the hardware parameter. See the

hardware section in this chapter for a

complete list of settings.

| You can also use  hardware=packet   for a packet driver TSR.

interrupt=3

There are several choices for interrupt

vector on the 3COM 3C501 Etherlink board.

NCSA Telnet defaults to IRQ3, but can be

set to use different interrupts by

indicating the interrupt number here.

Match this number with the interrupt

setting from your Etherlink board.

address=d000

This is the segment address of the

Ethernet board's shared memory, entered

as four hex digits. This only applies to

boards that have jumpers or registers

which can be used to set the shared

memory address. After setting the board's

jumpers to a particular value, use the

address parameter to configure NCSA

Telnet with the same setting. d000 (hex)

is a common address to use and it rarely

conflicts with other boards.

| If you set hardware=packet, you must set address=0, or set it to the

| packet driver class which you want to access. Normally it should be

| set to 0, or left out of the config.tel file altogether.

ioaddr=360

This is the I/O base address of the

Ethernet board in hex. Some Ethernet

boards have selectable I/O addresses.

These generally range from 200 to 400

(hex) and must be entered as two, three,

or four digits.

| If you set hardware=packet, set ioaddr to the hex interupt value

| where your packet driver is located. If you set it to 0 or any value

| not between 60H and 7FH then ncsa telnet will search for the first packet

| driver found between 60H and 7FH.

ftp=yes

FTP serving is enabled by default. Access

to your PC can be controlled by the FTP

password file. To disable FTP serving

completely, change this line to ftp=no.

rcp=yes

rcp serving is enabled by default. To

disable rcp serving, change this line to

rcp=no.

capfile=

Specify a name for the capture file. For

example,

capfile ="c:\temp\myfile"

When you capture to disk, all data is

appended to this file. If no name is

included in the configuration file,

capfile is used by default.

capfile=prn

Specifies that the printer device (PRN)

should be used for the capture file.

hpfile=hp.out

Tektronix graphics can be dumped to a file

in several formats from the Graphics menu.

When you write out HPGL (HP plotter)

codes, this file name is used. For

example,

hpfile="c:\hpgrafs\hp.out"

The default is hp.out in the current

directory.

hpfile=COM1

With an HP plotter hooked up to your PC,

HPGL plotter codes can be sent directly to

the plotter. Use the mode command to

preconfigure the serial port first.

psfile=ps.out

PostScript commands can be printed by

several types of available laser printers.

NCSA Telnet's PostScript output has been

tested on the Apple (Sun) LaserWriter,

which is connected to Sun workstations.

The default filename is ps.out in the

current directory.

tekfile=tek.out

Tektronix 4014 graphics commands are a

very compact way to store a Tektronix

graphics image. This option specifies the

filename to use when you write Tektronix

codes to disk. The default name is

tek.out.

passfile=

Specify the file in which FTP usernames

and passwords can be found. For example,

passfile="c:\bat\ftppass"

There is no default name for this file. If

the file is specified, then FTP will

require a username and password for all

attempted FTP connections. If the file is

not specified, then there is no password

checking for FTP. Use the program

TELPASS.EXE to encrypt new passwords. See

the FTP Password Protection section later

in this chapter.

| You can also set passfile="nul" in this case, you will only be

| able to FTP into your machine if you use the internal FTP password

| which is transmitted by pressing the ALT-W key.

| This is a relatively secure way of supporting FTP for your machine

| without allowing anyone else but yourself to access your files.

| The internal password is regenerated EVERY time the ALT-W key is

| pressed. When you are prompted for a user name, enter any value,

| then when prompted for a password press ALT-W

| if passfile= some file which does not exist, there will be no

| way to ftp into your machine, even if you use ALT-W.

+tnmode=

+   this sets the mode in which TN3270 mode is implemented

+   in version 2.2TN (ignored for 2.2D)

+   choices are

+

+	auto    -   automatically switches into 3270 mode if:

+		    a. the host requested the terminal type  twice

+		    b. the host requested EOR and Binary handling.

+		    (this mode is the default if tnmode is not specified)

+

+	flagged -   will only switch into 3270 mode for those host

+		    entries in the config.tel file for which the

+		    variable  CRMAP=FLAGGED

+

+	notflagged - opposite of above.

+

+	off     -   will never switch into 3270 mode and will always

+		    claim to be a VT102.

+

+map3270=

+	specify the name of the 3270 keymap file if you do not

+	want to use the default keymap. You can generate your own

+	keymap file using the makemap.bat file. See the tn3270 section.

+

Host-Specific Parameters of the Configuration File

Following the PC configuration options, you may specify zero or

more hosts, and host-specific information for each host. By making

the host entries in the configuration file, you can specify color,

backspace, scrollback, connection timeout, and several tuning

parameters for each host. You may want to have more than one

session name associated with a single computer, such as purple

cray and red cray with different screen colors to fit your mood of

the day.

Typically, the first host listed will be name=default, which

stores the default values for the other hosts. Any keyword listed

under later hosts will override the default setting for that host

session.

NOTE: The keyword name is special because it separates entries.

The parameters following name up to the next keyword name are all

associated with the session name. The parameters are installed

whenever a connection is opened with that session name.

name=nic

The name that you want to appear on the

bottom line of the screen when the

connection opens. It is the primary name

associated with a list of parameters. It

is common to have more than one session

name for a host, each with different

parameters, perhaps with different colors

or different amounts of scrollback. This

parameter is required because it separates

entries.

host=sri-nic.arpa

This is the hostname or alternate name. If

you want to associate both a session name

and a hostname with a particular set of

parameters, you may include both. Note

that the name parameter is required, while

the host parameter is optional. The rule

of thumb is: When you have only a

hostname, insert it as name=hostname. If

you have both a session name and a

hostname, enter both name=sessionname  and

host=hostname. When you want to open a new

connection, either hostname or sessionname

works.

hostip=10.0.0.51

This is the IP address of the host. If

this is not present, the domain nameserver

must be queried to get the IP number of

the host. For efficiency, include the IP

addresses of all commonly accessed hosts.

IP addresses of gateways and nameservers

must be in the configuration file.

copyfrom=nic

The copyfrom parameter is probably the

most important one. It causes all

unspecified parameters to be copied from a

previous session name. Note that the

session name which you want to reference

with copyfrom must appear before the

session name that uses a copyfrom

directive. Parameters that are specified

for a session name along with a copyfrom

directive override that copyfrom

directive.

gateway=1

This specifies the gateway precedence for

this host. To reach hosts not connected to

your local network, you must have at least

one gateway entry. The hostip keyword must

be present for this host. Gateway numbers

must start at 1 and increase by ones.

Gateway 1 has the highest precedence, but

the first gateway to respond to an ARP

will be used. ICMP redirects can affect

how gateways are used, but not

permanently.

| If you use myip=bootp, you do not need a gateway, but you can enter one

| if you wish.

nameserver=1

This specifies the nameserver precedence

for this host. NCSA Telnet uses UDP to

query domain nameservers for machinenames

that are not in the configuration file.

Each machine that is to be used as a

nameserver must have this keyword listed.

The hostip keyword must be present for

this host. Nameserver 1 has the highest

precedence. Nameserver numbers must start

at 1 and increase by ones.

| Again, for myip=bootp, you will normally not need to include this,

| you can if you want.

nfcolor=white		normal, foreground

nbcolor=black		normal, background

rfcolor=black		reverse, foreground

rbcolor=white		reverse, background

ufcolor=blue		underline, foreground

ubcolor=black		underline, background

These new color options can be used

instead of the more complicated color code

from previous versions. For each value,

specify a string from the following list

of colors available on IBM-PC class

machines with color displays:

black

blue (underline on mono screen)

green

cyan

red

magenta

yellow

white

The foreground and background colors are

combined to create the text appearance on

the screen. You may wish to experiment

with the Parameter menu (Chapter 3), which

interactively allows you to select a color

scheme. The previous version's color=

entries are still supported.

+ Note that this option is now case sensitive.

+ lower case entries perform the same as older versions of telnet.

+ Upper case entries now select the BOLD bit if used in a foreground

+ color, or set the BLINK bit if used in a background color.

+ The entire color must be either upper case or lower case.

scrollback=100

This specifies the number of lines of

scrollback for this session. Be aware that

scrollback occupies at least 86 bytes per

line saved. There can be a different

number of lines of scrollback for each

session. Plan your use of scrollback

wisely unless you have memory to spare.

+ If you will be using 3270 mode, set the scrollback to a small number

+ for those VM hosts you will be connecting to. The Scroll back buffer

+ does not work as expected in a 3270 session. Lines are saved in the

+ scrollback buffer only if the local clear screen AID key is pressed

+ (usually ^Z). So, there is no benefit wasting memory with a large

+ scrollback buffer in a 3270 session.

clearsave=yes

Scrollback is updated when the screen is

cleared. When clearing the screen, all of

the visible lines are saved into the

scrollback region. If you prefer not to

have the text saved when the screen

clears, set clearsave=no. In the case of

host programs which clear the screen one

line at a time, the lines are never saved

into the scrollback region.

erase=delete

This sets the backspace translation for

this host. Some hosts prefer the backspace

key to be DELETE and some prefer the

backspace key to be BACKSPACE. Set this

value erase=delete or erase=backspace.

crmap=4.3BSDCRNUL

This is a special compatibility option for

4.3 BSD UNIX. There is now a UNIX bug fix

to take care of the problem, but some

hosts may still want crnul to be used for

end-of-line. The default is crmap=crlf,

which sends CRLF when you press RETURN. In

line mode, CRLF is always used.

+ This variable is also used with the tnmode option. See tnmode=listed

+ above.

duplex=half

This parameter only applies to hosts that

negotiate non-echoing mode but do not

expect local line editing. All character

keys are sent and echoed to the screen

immediately. This parameter has no effect

in echo mode.

contime=10

This is the connection timeout in seconds.

When you are making a connection attempt,

after this amount of time has elapsed NCSA

Telnet gives up on opening the connection

and deletes the window. For congested or

slow networks, this value should be made

larger.

retrans=7

This is the initial retransmit timeout in

18ths of a second. Increasing the value of

this parameter may help in reducing the

initial burst of retries that is typical

of connections with high round-trip times.

mtu=512

This is a byte count (1-1024) of the

| this has been increased to 1500 in version 2.2C

+ But there is a bug in the NI5210 driver which causes packets larger

+ than 1100 bytes to trash the receive buffer pool, so this has been

+ set back to 1024 in version 2.2D and version 2.2TN

largest amount of data to put in the

packets that are sent. If you are sending

to the ARPANET you should use mtu=512. If

you are sending to local hosts, you should

use mtu=1024.

maxseg=512

This is a byte count (1-1024) of the

| increased to 1500

largest TCP segment that can be received.

This value can control the size of packets

that are sent over the connection.

Reducing this value can eliminate IP

fragmentation that we cannot reassemble.

maxseg=512 should force the sending host

to never fragment.

+ A smaller mseg is important if you are using a SLIP packet driver.

+ to avoid a large number of transmissions due to line errors and to avoid

+ fragmentation.

rwin=512

This is a byte count (1-4096) of the

largest size of the TCP window to

advertise to other hosts. Unfortunately,

some hardware and networks, the 3COM 3C501

Etherlink for example, cannot handle

receiving back-to-back packets. This

requires us to limit the TCP receive

window that we advertise to other hosts.

For communicating to slower hosts, or when

using the other brands of Ethernet boards,

a larger window (4096) may work better.

Converting UNIX /etc/hosts Files

Included with the distribution is an awk script called newh. Used

with the following command under 4.X BSD UNIX, it will convert the

/etc/hosts file format into a format compatible with NCSA Telnet's

configuration file. Note that domain name lookup should make this

operation obsolete, or apply to only a small subset of your

/etc/hosts file.

% awk  -f newh /etc/hosts >config.temp

After creating this new file, prepend the PC-specific information

and download the file to the PC.

HARDWARE OPTIONS

Combined Ethernet Drivers

There is only one distribution program for NCSA Telnet Version

2.2. All of the Ethernet and video device drivers are combined

into one executable program. You must use the hardware entry in

the configuration file to inform NCSA Telnet which Ethernet board

you have installed. Choose from the list of supported Ethernet

boards in Table 6.1; the value to use is case insensitive. The

requirements for interrupt, address, and ioaddr entries in the

configuration file are listed.

Table 6.1.  Ethernet Boards Supported by NCSA Telnet

hardware=

Value		Ethernet Board			Requires

For PC bus -

3C501		3COM 3C501 Etherlink		interrupt,ioaddr

PCNIC		Ungermann-Bass PC-NIC 		address,ioaddr

or IBM Baseband adapter

NI5210		MICOM NI5210 			address,ioaddr

WD8003		Western Digital WD8003E 	address,ioaddr

For PS/2 MCA bus -

NICps2		Ungermann-Bass NICps/2 		address

3C523		3COM 3C523 Etherlink/MC 	address,ioaddr

|packet		FTP software's Packet Driver spec	address,ioaddr

|		Currently only class 1(ethernet) supported.

+		Class 6 (SLIP) is now supported also.

|		address=class, or 0, ioaddr=int value or 0 for search

Performance Tuning

The values of maxseg, mtu and rwin in the configuration file must

be set correctly to get maximum data transfer throughput between

machines. There are some rules of thumb to use when setting these

values.

%	The maximum reasonable values for these parameters are:

rwin=4096

|	mtu=1024

|	maxseg=1024

+|	mtu=1024  	NEW

|	maxseg=1500	NEW  (should be mtu-ip/tcp header)

%	The best setting for local network use, with a high performance

Ethernet board is:

rwin=4096

mtu=1024

maxseg=1024

%	The best setting for ARPANET use, or any situation with a lot

of unknown gateways, but with a high performance Ethernet board is:

rwin=4096

mtu=512

maxseg=512

%	The setting required for the 3COM 3C501, and any other

troublesome network situation, also the most conservative

setting, is:

rwin=512

mtu=512

maxseg=512

rwin specifies what the other computer is allowed to send you at

any one time, so it depends mostly upon your local Ethernet board.

If the board can handle it, always set rwin=4096. maxseg is used

to avoid fragmentation. If you get fragmented packets, lower the

value of maxseg for that host until fragmentation stops occurring.

RARP for Dynamic IP Address Assignment

myip=RARP

When this special form of the myip entry is used, NCSA Telnet

looks up your local machine's IP address with a broadcast request

to the network. The "Reverse Address Resolution Protocol" is used

to attempt to translate the local machine's assigned 48-bit

Ethernet address to a corresponding IP address. From our tests

with the RARP server from Sun Microsystems' SunOS 3.X, the RARP

lookup takes about 3 seconds. You must install the 48-bit Ethernet

address from each PC in the RARP hosts' tables before RARP will

work. The value of arptime, in seconds, multiplied by three, is

used for a timeout value.

NOTE: Sun's RARP lookup will not work if you have a double-zero

in your Ethernet address. Other leading zeros can be dropped as

well. Use a single zero, i.e. 8:0:20:1:5a:90, instead of

08:00:20:01:5a:90.

| BOOTP for Dynamic IP Address Assignment

|

| myip=bootp

|

| First, you should consult RFC1048 to get the full scoop on this.

| Second, your site will require a BOOTP server for each subnet on

| which you wish to support BOOTP.

| This version  of BOOTP client (with some help from Stanford U)

| gets the following from the server (and your server should try and

| send the following)

|  Your IP address, netmask, nameservers, gateway and client host name.

|  Any nameservers returned by the BOOTP server (domain servers, not

|  IEN-116 servers) are added to any that where listed in the config file.

|  Same for gateways.

|  The client host name, if not truncated, is used to fake up a domainslist

|  unless there is already one specified.

|  The netmask value over-rides any entered in the config file.

Interrupts

When you configure the 3COM hardware, an important option is the

choice of interrupt request number (level). 3COM boards arrive set

for IRQ3, which corresponds to the PC-DOS definition of the COM2

communications port. If you have a port configured as COM2, you

cannot set your Ethernet board to IRQ3. Another common value to

use is IRQ5, which is the same interrupt that DOS uses for LPT2.

Make sure that your PC hardware configuration does not conflict in

any way.

WARNING

The most common hardware problem is interrupt conflicts.

PCs have a limited number of interrupts, and you cannot have two

devices which expect to use the same interrupt. PC Ethernet boards

almost always consume an interrupt, even though we do not use

interrupts for all Ethernet boards. Interrupts are also used by

COM ports and LPT ports in your machine. You must check for

conflicts.

Even though it works, it may stop working. Interrupt

conflicts don't always show up right away. They are software

controlled. If the interrupt on your COM2 port is not activated,

it may not conflict with a 3C501 board on IRQ3 until next week

when you try to use a modem terminal emulator and then switch to

NCSA Telnet. NCSA Telnet might work fine until you try to FTP to

your hard disk; then disaster strikes.

Hard disks use interrupts too. We have traced two serious

hard disk problems to interrupt conflicts. IRQ2 is the default

setting for some Ethernet boards when they come out of the box. On

a PC-AT, this interrupt is used for the second interrupt

controller, and a conflict can shut off the whole second

controller which includes the hard disk. On the PC/XT and PC

compatible computers, IRQ5 is sometimes assigned to the hard disk

rather than LPT2. Strange hard disk problems like disk write

errors may be caused by the Ethernet conflicting with the hard

disk.

Moral:  Set your Ethernet board to an unused interrupt.

DOMAIN NAME LOOKUP

When NCSA Telnet cannot find a name in the configuration file, it

can still find the IP number if you are running a domain

nameserver. To use the domain system, at least one host must be

running the nameserver and the configuration file has a nameserver

entry for that host. There may be more than one. If one nameserver

fails to respond, the one with the next higher precedence is

queried. As soon as a response is received, NCSA Telnet will add

its name to the status line and attempt to open a telnet

connection.

Domain Search Order

When you enter a name to open a connection, there is a specific

search order:

1.	The name is looked up as a session name from the

configuration file.

2.	The name is looked up as a hostname from the configuration

file.

3.	The name is sent as a domain query to the first nameserver.

| If you have a domainslist, a HOST NOT FOUND error from the

| name server causes the next subnet suffix to be added to the

| name and a retry is initiated.

4.	If the domain request times out, the query is repeated, but

to another nameserver. This is repeated until the maximum

number of retries is reached or a response is received.

With the domain nameserver, the number of hosts in the

configuration file can be kept to a minimum. Each host in the

configuration file will be a commonly used computer that may have

special screen color or scrollback requirements. The IP addresses

for rarely used hosts will be accessible if the domain name

retrieval system can resolve those hosts.

FTP PASSWORD PROTECTION

The presence of the passfile keyword in the configuration file

enables FTP password protection. If you have a password file, FTP

will not allow any FTP connections to open without a correct

username and password. You can have several usernames and

individual passwords for each user. The password file can be

easily read by typing out the file. The passwords are encrypted,

but not with a secure encryption system. Only trusted users should

be allowed access to the password file.

To create or change passwords for users, run a separate program

called Telpass. This program reads in the password file so you can

add and change users and passwords. There are no special

installation instructions for Telpass as it is menu-driven. Start

up Telpass from DOS with the name of the password file that you

wish to edit.

C:\ telpass pwfile

Telpass automatically saves the list of names and passwords when

you exit. Just add a passfile option to your configuration file to

enable FTP passwords.

| For extra security, you can set passfile="nul".

| This way, only you can ftp to your machine, and you can only do

| it by using the ALT-W key to send the internal random FTP password.

| (The password changes each time it is sent).

| Remember, though, if passfile= some file which does not exist,

| you will not be able to FTP at all, even using ALT-W

COMPATIBILITY NOTES

Ping

NCSA Telnet responds to ping (ICMP echo) requests. This is used by

other hosts to determine whether your PC is online.

VT102

The VT102 emulator is nearly complete. VT102 features not emulated

are double width and double height characters, VT52 mode, origin

mode for cursor positioning, and 132 column mode. Included in the

emulator are variable spaced tabs, graphics characters, keypad

modes, and character attributes.

+Extended Ansi Commands

+

+The ansi commands for setting colors and screen attributes are

+now supported. See the section on Setting Screen Colors.

+

+The ansi print through commands are now supported, see the section

+on Capture File Operations.

+

FTP

The FTP server in NCSA Telnet is close to the DARPA specification

of the minimum implementation. Exceptions are:

%	The command connection does not perform telnet negotiation.

%	Block mode of FTP is not supported.

%	Some error conditions may display as command not understood

instead of returning more appropriate messages.

FTP has not been thoroughly tested on low-bandwidth or low-

reliability networks. It should be most successful on local

networks, with unknown throughput for long-haul or satellite

networks.

Terminate and Stay Resident (TSR) Programs

Programs such as SideKick from Borland International Inc. take

complete control of the PC when they are initiated with key

sequences. The use of these programs does not crash NCSA Telnet,

but it suspends the network communications capability while the

TSR program is active. When network communications are suspended,

whether from the ALT-E DOS escape feature or from using TSR

programs, it takes the host approximately one minute to decide

that your PC has crashed, and then it cuts you off. As long as

these programs are used for short (less than 30 seconds)

intervals, you should have no problem.

Telnet

The standard telnet protocol has several potential options that

can be invoked if both parties of the telnet connection agree.

NCSA Telnet refuses most of these options, but accepts the

following: echo, option 1; and suppress go ahead, option 3. There

are some obscure features of telnet that are not implemented in

this version: out-of-band interrupts are not available, go ahead

signals do nothing, and telnet acknowledge signals are not

acknowledged. If there are any problems with the limitations of

NCSA Telnet, please submit a bug report.

+ Other options supported now include

+ EOR, BINARY, TERMINAL TYPE, NAWS. Binary mode is refused

+ unless EOR processing has already been negotiated. This is to

+ avoid VMS systems using CMU-TCP from requesting binary mode and

+ then messing up CR/LF key mapping.

Limitations

There is a 20-session limit for NCSA Telnet. This is somewhat

arbitrary and may be altered if there appears to be a need for it.

NCSA Telnet can run out of memory, often before the 20-session

limit. You are notified on the screen if this happens.

+TN3270 Information -- For version 2.2TN only.

+

+The tn3270 support is based on Greg Minshall's tn3270 version 4.1.1 for unix

+systems. The two most important things to understand about how

+to use tn3270 mode is: 1) how NCSA telnet decides to start tn3270

+mode, 2) how to change your personal keymappings. Item one

+is most important to your system manager.

+

+

+Establishing a TN3270 connection

+

+When connecting to a VM host, ncsa telnet expects the following

+to occur to establish a connection:

+	1. Host requests Terminal Type

+	2. Telnet sends  VT102.

+	3. VM host asks for Terminal Type again.

+	4. Telnet sends   IBM-3278-2 (or -3 or -5 for 35 and 50 line mode)

+	5. Host requests EOR processing.

+	6. Telnet Agrees

+	7. Host requests Binary processing.

+	8. Telnet agrees if the last terminal type sent was IBM-3278

+

+If your VM host does not ask for the terminal type a second time,

+and then closes the connection, you can use the tnmode=flagged variable

+and flag the individual hosts in your config.tel file (using crmap=flagged)

+This will cause telnet to always send IBM-3278 for those flagged hosts.

+

+Important: You can not start a connection in line mode, and then

+switch to full screen 3270 mode.

+

+When a connection switches into 3270 mode (only one is allowed at a time)

+telnet examines the map3270 variable to determine which key map file to

+use. If the map3270 variable is not set, telnet looks for /etc/map3270

+on the current drive (a throwback to unix), if /etc/map3270 is

+not found then the internal key mappings are used. (See the keymap

+shown below).

+

+

+Creating your own 3270 key maps.

+

+You can create your own 3270 keymap file using the makemap.bat

+file. This process is not for the faint of heart. You need the

+following files:

+	tncpp.exe

+	makemap.bat

+	3270keys.h

+	gmap3270     (for regular keyboards)

+	gmap3270.ext (for 101-key keyboards)

+

+  1. Obtain a hard-copy printout of the 3270keys.h file. Keep for reference.

+  2. Edit either gmap3270 or gmap3270.ext depending on whether or not

+     you have a 101-key keyboard.

+

+  3. The 3270 function is listed on the left, and the keys bound to

+     the function are listed on the right.

+

+     example:

+	pf1= F1 | AF1;

+

+     This example sets pf1 to be F1 or Alt-F1

+

+     You must use the tokens defined in the 3270keys.h file.

+     (FUNC1 is not defined in 3270keys.h, and would not work)

+

+     You can not use keys that are already defined for other functions.

+     (such as ALT-X which will close the current connection)

+

+  4. After editing the file, run the makemap.bat file as follows:

+	makemap  gmap3270  map3270

+

+	or makemap gmap3270.ext map3270

+

+     the second filename is the output file name (don't use one that

+     already exists as it will get erased.)

+

+  5. Set the map3270= variable to point to your map3270 file.

+

+When you next start telnet and enter a 3270 connection, press ALT-M

+to see if any error messages were printed on the console. Any

+error messages indicate that a mistake was made while editing the

+gmap3270 file (or gmap3270.ext) and the mistakes must be corrected

+there. Then makemap must be run again.

+

+

+Misc Notes about 3270 mode.

+

+Scroll back does not work in 3270 mode. To conserve memory

+you should use a small scrollback value in your config.tel file

+for VM hosts.

+

+If you have an EGA display, you can use the -l 35 command line

+option to get a 32 line 3270 display. To get a 43 line 3270

+display you need 44 or more lines on the screen. The next highest

+value would be a 50 line ega display (hard on the eyes). The

+-l 43 command line option still only gives a 32 line 3270 display,

+but regular telnet connections can use 42 lines.

+

+Transparent data mode (7171 datastream) works well, you can

+direct 7171 datastreams to the tek window, vt screen or capture

+file. This option is setable using the parameter window.

+(it is not setable in your config.tel file, sorry, and defaults

+to Tek window).

+

+

+Default Keymappings.

+

+If you have a 101-key keyboard, and use the gmap3270.ext file

+to create a map3270 file. You can use the extended keys listed

+in the middle column, otherwise, the default keymap assumes that you

+have a regular keyboard and uses those keymappings listed in

+the right-hand column.

+

+                               Clarkson University

+                            Educational Resource Center

+

+                          NCSA Telnet TN3270 Key Mapping

+

+                                 March 13, 1989

+

+     3270   Function         Enh keyboard               Std keyboard

+                          (requires MAP3270)       (default)

+

+    Enter               Return                   Return

+    New Line (NL)       Ctrl-Enter               Ctrl-Enter

+    Tab ->              Tab                      Tab

+    Tab <-              Shift-Tab                Shift-Tab

+    ->                  ->                       ->

+    <-                  <-                       <-

+    Up arrow            Up arrow                 Up arrow

+    Down arrow          Down arrow               Down arrow

+    Delete              Delete                   Delete

+    E EOF               End                      End

+    E INP               Ctrl-End                 Ctrl-End

+    PF1                 F1                       F1

+    PF2                 F2                       F2

+    PF3                 F3                       F3

+    PF4                 F4                       F4

+    PF5                 F5                       F5

+    PF6                 F6                       F6

+    PF7                 F7                       F7

+    PF8                 F8                       F8

+    PF9                 F9                       F9

+    PF10                F10                      F10

+    PF11                F11                      Shift-F1

+    PF12                F12                      Shift-F2

+    PF13                Shift-F1                 Shift-F3

+    PF14                Shift-F2                 Shift-F4

+    PF15                Shift-F3                 Shift-F5

+    PF16                Shift-F4                 Shift-F6

+    PF17                Shift-F5                 Shift-F7

+    PF18                Shift-F6                 Shift-F8

+    PF19                Shift-F7                 Shift-F9

+    PF20                Shift-F8                 Shift-F10

+    PF21                Shift-F9                 Ctrl-F1

+    PF22                Shift-F10                Ctrl-F2

+    PF23                Shift-F11                Ctrl-F3

+    PF24                Shift-F12                Ctrl-F4

+    PA1                 Page Up                  Page UP

+    PA2                 Page Down                Page Down

+    PA3                 Ctrl-Page Down           Ctrl-Page Down

+    -> fast             Ctrl ->                  Ctrl ->

+    <- fast             Ctrl <-                  Ctrl <-

+    Test req            Ctrl-t                   Ctrl-t

+    Reset               Ctrl-r                   Ctrl-r

+    Clear               Ctrl-Home                Ctrl-Home

+    Dup                 Ctrl-u                   Ctrl-u

+    Field Mark          Ctrl-y                   Ctrl-y

APPENDIX A

ERROR CONDITIONS

Most of the error conditions in NCSA Telnet are nonfatal. The most

important and common error messages are listed here with a short

summary of the symptoms and causes.

The following messages may appear on your screen during the

operation of NCSA Telnet. Any other messages that appear are

protocol-specific messages that require additional diagnosis from

the system administrator. If a message that is not documented here

occurs repeatedly, please contact your system administrator first,

and if no solution is found, submit a bug report as included in

this manual.

Cannot find or open hosts file

Cause:

NCSA Telnet will not operate without a configuration file that

contains the PC's IP address. This file could not be found.

Solution:

The configuration file can be either in the current directory, or

it can be specified with the -h parameter on the command line.

This is normally customized in TELNET.BAT. Your system

administrator should check your installation and make sure the

configuration file is there.

Host machine not in hosts file

Cause:

You have entered a machine name that cannot be found in the

configuration file. This usually is caused by a misspelling.

Solution:

Check that the name you entered was spelled correctly. If you want

this computer to be accessed by name, add its name and IP address

to your configuration file.

ICMP:	Destination unreachable

Cause:

Another machinePPprobably the gatewayPPhas determined that your

message cannot reach its destination from your system.

Solution:

Check the IP address in your configuration file. Notify your

system administrator that the gateway cannot connect you to the

destination you want to reach. There may be a problem with the

gateway.

ICMP:	Echo requested (ping requested)

Cause:

Someone has "pinged" your machine.

Solution:

No action is required; you are just being notified that a program

on another computer has tried to determine if your machine is

alive and connected to the network. Your machine answered "yes".

ICMP:	Redirect, another gateway is more efficient

Cause:

NCSA Telnet is sending packets to a gateway that is reforwarding

them onto your local network. You will get one of these messages

for each packet until NCSA Telnet adjusts the local tables and

automatically switches to the correct gateway.

Solution:

NCSA Telnet responds to these messages and automatically switches

to the correct gateway, so you don't have to take any action.  You

may want to change the gateway assignments in your configuration

file to use a more appropriate gateway.  Some situations will

always produce redirects.

Local HOST or gateway not responding

Cause:

Possible reasons this error occurs are: network problem, a

configuration file problem, the computer you want to connect to is

down, or the gateway that you need is down.

Solution:

If the computer is on your local network, check to see that the

network is up and running. If the computer is not on your local

network, check to see if the gateway is up and running. Ask the

system administrator to check the specification of the gateway

(gateway=) in your configuration file. Check the IP number of the

computer that you are trying to connect to. Check to make sure

that your computer is attached to the network. Check the integrity

of the network cable.

Memory allocation error, cannot open port

Cause:

Your system ran out of memory. This is the most common barrier to

opening more sessions.

Solution:

Log off of some of your sessions or provide more memory in which

NCSA Telnet can run. Providing more memory may mean buying more or

rearranging your windowing system to allow more memory. NCSA

Telnet Version 2.2 requires about 250K of memory plus 15K per

session. In addition, scrollback requires 86 bytes per line.

Network jammed, probable break in wire

Cause:

The PC's Ethernet board could not transmit a packet due to a low-

level hardware error.

Solution:

The system administrator should ensure that the Ethernet T

connector or transceiver is correctly connected to the back of

your PC, then check the integrity of the Ethernet wire according

to Ethernet specifications. There could be a short circuit in the

wire, but the usual cause is that the wire has been disconnected

somewhere.

No internal TCP ports available

Cause:

You are trying to do too many activities at the same time, or some

combination of your activities has not closed the TCP sessions

correctly. This will happen if you open too many sessions to other

computers.

Solution:

Close some of your existing sessions. If necessary, exit the

program by logging off of all of the other computers, then restart

NCSA Telnet.

Packet received for invalid port -- reset sent

Cause:

Some other computer is sending you packets that NCSA Telnet does

not expect or understand. This often happens when you press ALT-F3

to exit NCSA Telnet and then start it back up again. It can happen

under a variety of network breakdown conditions.

Solution:

Do not press ALT-F3 to exit NCSA Telnet. If you already have, or

some other network breakdown caused the problem, the other

computer will eventually time out and stop sending you the extra

packets. If you continue to receive them, notify your system

administrator.

Reset received: syn sent

Cause:

The machine to which you are attempting to connect is refusing

your telnet connection.

Solution:

Check to see why that machine is not up and running. If it is,

check your configuration file to see if you are using the correct

IP number.

APPENDIX B

NCSA TELNET COMMAND REFERENCE

Key(s)			Action

ALT-A			Adds a session

ALT-N			Next session

ALT-M			Message screen

ALT-E			Escapes to DOS shell

ALT-G			Graphics menu

ALT-C			Capture file toggled on/off

ALT-R			Resets VT102 screen

ALT-H			Help screen

ALT-T			Starts file transfer as if typed: ftp [internet

address]

ALT-I			Sends internet address to host as if typed

ALT-S			Skips scroll, jumps ahead

ALT-P			Changes a parameter; one of: color, capture file

name, backspace, session name, screen mode

ALT-F3			Aborts NCSA Telnet completely. Strongly discouraged

ALT-Y			Interrupts process

ALT-O			Aborts output

ALT-Q			Are you there?

ALT-U			Erases line

ALT-K			Erases character

Key(s)			Action

| ALT-W			Sends the internal FTP password as if typed.

| ALT-L			last (previous) session

+ALT-D			Dump the contents of the current screen to the

capture file.

ALT-X			Closes connection

HOME			Exits graphics mode

CTRL-HOME		Clears/enters graphics mode

SCR LOCK		Pauses/restarts screen (do not use CTRL-NUM LOCK)

SCR LOCK		Enters/exits scrollback mode

APPENDIX C

CONFIGURATION FILE SUMMARY

This appendix contains a quick reference list of the keyword values

which are allowed in a NCSA Telnet configuration file.  There are two

sections, the first list contains the PC configuration entries which

apply to the overall program or to all sessions.  The second section

contains the list of attributes which may be assigned to individual

sessions.  Each keyword has an example value and a one-line summary of

its purpose.  For details on allowed values, see Chapter 6,

Installation and Configuration.  A template CONFIG.TEL file is included

on the distribution disk.

LOCAL CONFIGURATION

myip=10.0.0.51		IP address to use for the PC.

myip=RARP		Query the RARP server for my IP address.

| myip=bootp		Query the BOOTP server for my IP address, etc.

netmask=255.255.255.0	Subnet mask for your local network.

arptime=3		Local network timeout for ARP requests.

|domain="ncsa.uiuc.edu"	Default domain suffix. ** OBSOLETE

|domainslist="soe.clarkson.edu,clarkson.edu"  Default domain search list

domaintime=2		Domain lookup timeout per retry.

domainretry=4		Number of times to query domain nameserver(s).

tek=yes			Enable/Disable Tektronix graphics mode.

video=ega		Video display to use: cga, ega, hercules, no9.

|			or auto to automatically get your video type.

bios=no			Use BIOS or write directly to screen.

hardware=3C501		Type of Ethernet hardware to use:

3C501, PCNIC, NI5210, WD8003, 3C523, NICPS2

interrupt=3		Interrupt number for 3C501 board.

address=d000		Segment address of the Ethernet board's shared

memory in hex.

|			Or packet driver class if hardware=packet

ioaddr=360		I/O base address of the Ethernet board in hex.

|			or packet driver int if hardware=packet

ftp=yes			FTP server enable/disable, default is yes.

rcp=yes			rcp server enable/disable, default is yes.

capfile="\mycapture"	Specify a name for the capture file.

capfile=prn		Capture file is printer device (PRN).

hpfile=hp.out		HPGL codes for graphics go to this filename.

hpfile=COM1		HP plotter attached to COM1.

psfile=ps.out		Filename for PostScript output.

tekfile=tek.out		Filename for Tektronix 4014 codes when output

to disk.

passfile="\mypasswd"	Filename for FTP usernames and passwords.

|			or passfile="nul" for secure FTP operation

+tnmode=auto		or flagged,notflagged,off. sets the 3270 connection

mode

+map3270="map3270"	Filename of the 3270 key map.

HOST-SPECIFIC PARAMETERS

NOTE: The keyword name is special because it separates entries.

name=nic		Session name, separates lists of parameters for

sessions.

host=sri-nic.arpa	Hostname or alternate name. Usually full domain name.

hostip=10.0.0.51	IP address of the host. Required if nameserver not used.

copyfrom=nic		Copy all missing parameters from a previous session.

gateway=1		Gateway precedence, starts at 1, goes up by ones.

nameserver=1		Nameserver precedence, starts at 1, goes up by ones.

nfcolor=white		Normal, foreground color.

nbcolor=black		Normal, background color.

rfcolor=black		Reverse, foreground color.

rbcolor=white		Reverse, background color.

ufcolor=blue		Underline, foreground color.

ubcolor=black		Underline, background color.

Black, blue, green, cyan, red, magenta, yellow, or

white.

+			Upper case colors in foreground set Bold, in background

+			set Blink.

scrollback=100		Number of lines of scrollback for this session.

clearsave=yes		Save cleared screen into scrollback region.

erase=delete		BACKSPACE key is set to "backspace" or "delete".

crmap=4.3BSDCRNUL	Use a different set of codes for carriage return.

duplex=half		Local echo, but sends all characters immediately.

contime=10		Connection timeout in seconds.

retrans=7		Initial retransmit timeout in 18ths of a second.

mtu=512			Limit on outgoing packet size.

maxseg=512		Limit on incoming packet size, given as TCP MSS option.

rwin=512		TCP receive window, can make up for low performance

hardware.

BUGS AND SUGGESTIONS

We would appreciate notification of any bugs you have found and

suggestions that you might have.  You may send an electronic mail

message to (Internet format):

telbug@ncsa.uiuc.edu

or send this report by mail to:

NCSA TELBUG

152 Computing Applications Building

605 E. Springfield Ave.

Champaign, Illinois 61820

+Version 2.2D or 2.2TN bugs should be sent to

+Brad Clements

+Educational Resources Center

+Clarkson University

+Potsdam, NY  13676

+

+bkc@omnigate.clarkson.edu, bkc@clgw.bitnet

+

Name:

Installation:

Address (Electronic):

Address (U.S. Mail):

Telephone:  (        )             -

Version of NCSA Telnet and IBM System software:

Description of Problem/Suggestion: