Addresses:

----------

C900:0A00	Control/status port

C900:0C00-0FFF	Data port (1KB), adress ignored

Command port:

-------------

[7]	SCSI enable		E

[6]	Interrupt enable	IE

[5]	Parity enable		PE

[4]	Start arbitration	ArbR

[3]	ATN

[2]	BSY

[1]	SEL

[0]	RST

Status port:

------------

[7]	Arbitration complete	ArbC

[6]	Parity error		PErr

[5]	SEL

[4]	REQ

[3]	C/D

[2]	I/O

[1]	MSG

[0]	BSY

Operation:

----------

Default Cmd bits: EnableIRQ,EnableParity

RST:

Cmd  = E+IE+PE+RST

.. some time

Cmd  = IE+PE

.. some time

Arbitrate:

Cmd  = IE+PE

Data = HA target ID as bit mask

Cmd  = E+PE+ArbR

.. wait for ArbC (300 msec)

Data |= 1 << LUN

Cmd  = E+PE+IE+SEL+ATN

.. wait for BSY (100 msec)

Cmd  = IE+PE

.. wait (20 msec)

Cmd  = E*IE+PE+ATN

.. wait for (MSG+CD+IO+REQ) == (MSG+CD+0+REQ) (200 msec)

Data = 0xC0+LUN# (message out: identify, enable disconnect)

Cmd  = E+IE+PE

Data phase (x: out=0, in=IO):

while (data available)

.. wait for REQ

if (MSG+CD+IO) != (0+0+x) then break

if slow then

transfer single byte

else

fast block transfer (max 1K)

while (MSG+CD+IO+REQ) == (0+0+x+REQ)

transfer dummy byte

.. wait for REQ or timeout (a few msec)

Execute transfer, controlled by target, until disconnected:

while not long timeout (sec)

if not BSY then return error

if not REQ then continue

case MSG+CD+IO of

0+0+0:		-- data out

Data out phase

0+0+IO: 	-- data in

Data in phase

0+CD+0:		-- command out

Data = next command byte

0+CD+IO:	-- status in

Next status byte = Data

MSG+CD+0:	-- message out

Data = null message (0x08)

Cmd  = E+IE+PE

MSG+CD+IO:	-- message in

message byte = Data

case message byte of

Complete, Disconnect:

Cmd = IE+PE

SavePtr:

save data ptr&len

RestorePtr:

restore data ptr&len

MessageReject:

Nop:

Identify:

ignore

otherwise:

if message bit [7] then

ignore (identify)

else

error (unknown)

Interrupt /reselect):

if not SEL then

error, return

.. wait for IO (some 10 msec)

id = Data

if HA target address bit not set in id then

error, return

lun bit = id and not (HA target address bit)

Cmd = E+IE+PE+BSY	-- acknowledge reselection

.. wait for SEL (some 10 msec)

.. wait for BSY (some 10 msec)

Cmd = E+IE+PE

Execute transfer, ...