From: Roderick Maxwell <major@vicksburg.com>

Date: Sun, 16 Mar 1997 19:39:19 -0800

Subject: Re: ADMIN - overquoting!

Tesla-2 wrote:

>=20

> T>From: Roderick Maxwell <major@vicksburg.com>

> T>Date: Sat, 15 Mar 1997 21:11:57 -0800

> T>Subject: Re: 10KV RMS @ 300MA for less than $100

>=20

> T>Tesla-2 wrote:

> T>>

> T>> From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

> T>> Date: Fri, 14 Mar 1997 20:00:26 -0500

> T>> Subject: Re: 10KV RMS @ 300MA for less than $100

> T>>

> T>> >

> T>> >

> T>> > Could you please tell me how you tested the transformers and exactly

> T>> > how you wired them in series? I have just ordered two of them and I

> T>> > don't want to take a chance on frying them the first time I apply power.

> T>> >

> T>> >

> T>> >

> T>> >

> T>> >                                Frankensteins Helper

> T>> >                                       Max

> T>>

> T>> Max,

> T>> I have been busy trying to gather all the info I can both by experimenting

> T>> with the two units I bought, and by checking with people that I know have

> T>> experienced success with these units.

>

> <SNIP!!>

>

> T>  Thank you for your information I'll TRY not to use them until I put

> T>them under oil, and build a larger capacitor.

>   Hey folks!  PLEASE let's not quote 300+ lines of message for a 4 line

> reply!  Please help unstuff everyones mailboxes by cutting out

> unnecessary parts of the messages we are replying to.  Thanks!!

>

>    Ralph Hartwell

>

>      Listowner, TESLA-2 and NEWMAN-L  Mailing Lists

oops sorry!

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Steve Falco <sfalco@worldnet.att.net>

Date: Sun, 16 Mar 1997 08:13:02 -0500

Subject: Re: 10KV RMS @ 300MA for less than $100

> By the way, Steve, don't throw the ruined unit out. The core is still

> useable, as is the primary. You should be able to use the remnants to build

> something useful, such as a primary choke. By adding a heavy duty shorted

> secondary (just a few turns of really *thick* wire or rod or bar stock),

> you can have a mini equivalent of a shorted arc welder. For use in driving

> a primary circuit.

I tried to disassemble the ruined unit.  It is not easy.  I got the

bolts out, but one snapped because there was so much varnish in the

threads.  My fault, I should have burned the varnish off with a torch

first.  Getting the top of the core off was tough.  The dried varnish

welded it on.  I tried using a block of wood and a hammer to break it

loose.  No good.  I finally wound up using a cold-chisel and after much

hammering it came off.  This did not appear to hurt it - just scuffed it

slightly.

Next, I tried to pop out the primary.  It was glued in by the varnish

too.  By the time I got it out, it was trashed.  I suspect I could have

gotten it out in one piece if I had spent enough time with an X-ACTO

knife, but I really wasn't in the mood.  The primary would be easy

enough to rewind with fresh magnet wire since there are so few turns.

(We are "coilers" after all!)

I tried to remove the shunts.  They are apparently put in with a press,

and are very tight.  So far I have not been able to get them out, but I

did mutilate them a bit.  I'm not sure of the right approach here.  I

have not tried to get the secondary out, because I am sure it would not

come loose any easier than the primary.  I did cut into it to see what

wire gauge was used, however.  I measured it at about 0.014 with the

insulation still on the wire, but with the excess varnish removed.

Allowing a thousandth or two for the insulation, I'd say it is #28 wire,

which is pretty beefy.

I am tempted to rebuild these units completely, just saving the cores.

I would use kraft paper between the turns and dump them in oil.  The

kraft paper should wick the oil in.  Has anyone tried to build an HV

transformer from scratch?  Am I nuts for trying?

By the way, one of the remaining untouched units I have shows a burnt

area in one of the end papers.  I don't know if this coil failed in the

copier, but I'll have to test it before I decide to keep it.  Given that

they are surplus, there is no guarantee that they actually work.  H&R

will replace them, but you get stuck for the shipping.

Steve Falco

sfalco@worldnet.att.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

T>>   Hey folks!  PLEASE let's not quote 300+ lines of message for a 4 line

T>> reply!  Please help unstuff everyones mailboxes by cutting out

T>> unnecessary parts of the messages we are replying to.  Thanks!!

T>  oops sorry!

Hey, it happens to all of us occasionally!  Sometimes I'll hit the

wrong key and not realize that the entire message I was quoting is below

my reply and is not visible on the screen I am typing on.  Needless to

say, that sends the Whole Thing out again as a quote!    We all just

need to be a bit more alert occasionally and check our outbound messages

to make sure they aren't over quoted.

Ralph

---

Error: SALARY.COM -- system interrupted. Get a job.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: "R+S Gordon" <ritchcom@execulink.com>

Date: Sun, 16 Mar 1997 23:07:47 -0500

Subject: tank circuit

I often see Two different tank circuit configurations, one with the cap

across the neon's secondary and the other with the spark gap across the

secondary.  Is one configuration  preferable to the other, and if so, why.

If anyone can enlighten me on this please reply.

Ritch Gordon

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: "R+S Gordon" <ritchcom@execulink.com>

Date: Sun, 16 Mar 1997 23:14:08 -0500

Subject: re: tank circuit

I guess I should read my mail before I post a question.  It seems the same

question was aksed earlier today.

Ritch Gordon

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Bert Hickman <bert.hickman@aquila.com>

Date: Sun, 16 Mar 1997 23:30:06 -0800

Subject: Re: tank circuit

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: "R+S Gordon" <ritchcom@execulink.com>

> Date: Sun, 16 Mar 1997 23:07:47 -0500

> Subject: tank circuit

>

> I often see Two different tank circuit configurations, one with the cap

> across the neon's secondary and the other with the spark gap across the

> secondary.  Is one configuration  preferable to the other, and if so, why.

> If anyone can enlighten me on this please reply.

>

>         Ritch Gordon

>

Ritch,

You'll probably have longer life on your neon with the gap across the

secondary. When the gap fires, high amplitude RF is generated in the

Tesla Coil primary tank circuit. Neon sign transformer secondaries don't

survive long if high voltage RF is impressed across the secondary. Since

the gap, cap, and primary winding are all in series, the coil will

operate in either configuration. However, by putting the gap across the

transformer, you will effectively shunt high power RF away from the

secondary when the heaviest amount is being generated. If you instead

have the cap across the secondary, you will directly impress high

amplitude RF onto the winding - the internal insulation breakdown and

overheating will eventually destroy the transformer.

It turns out that, even with the gap across the transformer, some RF

does make it into the transformer, eventually causing its death. Using

high voltage RF chokes, series resistors, bypass caps, and a safety gap

are all measures that have been taken by members on this list to further

protect the transformer, with varying degrees of success.

Safe coilin' to you!

- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Kevin Nardelle <knardell@mailhost.accesscom.net>

Date: Sun, 16 Mar 1997 23:28:38 -0600

Subject: Re: tank circuit

At 04:07 AM 3/17/97 GMT, you wrote:

>From: "R+S Gordon" <ritchcom@execulink.com>

>Date: Sun, 16 Mar 1997 23:07:47 -0500

>Subject: tank circuit

>

>I often see Two different tank circuit configurations, one with the cap

>across the neon's secondary and the other with the spark gap across the

>secondary.  Is one configuration  preferable to the other, and if so, why.

>If anyone can enlighten me on this please reply.

>

>        Ritch Gordon

>

>

Ritch,

in my case and I HAVE NO IDEA WHY but my coil will only work (the gaps will

only fire) if the caps are across the neon. There is no explanation for this

because I have tried by many resuests to do it the "conventional" way

however no results at the gaps.

I DO NOT have the thing wired wrong because it is not possible to do it

wrong as many times as I did this "cenventional" hookup. My results are

fantastic and I would not have it any other way. I am going to install

overkill protection for my neon because putting the caps shunt across it is

not good for the life of a neon.

To answer your question, I get better than adverage with my configuration

compared to other coils with simmilar specs. my coil secondary is only 3.5"

in dia. and has 851 turns of #22 wire. My primary is 15 turns of 1/4" copper

tube spaced 1/4 inch apart flat spiral with a inside diameter of 6"

I do run a rather large neon 110VAC in and 12KV @120MA out with the best

sparkgap design in town (Thanks to Mike Hammer for his design of these

gaps). My total spark legnth was MAX (I have only seen this a few times) And

I say MAX 4 feet long jump out and wanna grab ya killer sparks. I adverage

around 2.5-3 feet weather permitting. I hope I have answered your question.

I tried anyway.

Look at the BIG picture, N tesla used this configuration himself but the

vast majority of coilers out there today do it with the gaps across the neon.

This has been rather contraversial as to whether this is a good

configuration but whatever floats your boat, Keep in mind I have a nice

supply of backup transformers in case this one dies so I care not about the

life of this neon.

I know a guy Mr. M.H. that is waiting for it to die so he can get his hands

on it.:)))

Hope I have been of some help.

PS this email got me out of the bed, I was just about to fall asleep and I

heard the "Hey this is great man!" sound from good ole Cheech and Chong in

my mail program. I felt I had to answer.

THE BEST OF LUCK TO YOU RITCH!

end

Regards,

Kevin Nardelle

**********************************************

Kevin Nardelle   knardell@accesscom.net

THE TESLA AND HV WEB SITE-VISIT TODAY!

http://www.icorp.net/users/kev/tesla

http://www.accesscom.net/nardell

**********************************************

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Kevin Nardelle <knardell@mailhost.accesscom.net>

Date: Sun, 16 Mar 1997 23:29:41 -0600

Subject: re: tank circuit

At 04:14 AM 3/17/97 GMT, you wrote:

>From: "R+S Gordon" <ritchcom@execulink.com>

>Date: Sun, 16 Mar 1997 23:14:08 -0500

>Subject: re: tank circuit

>

>I guess I should read my mail before I post a question.  It seems the same

>question was aksed earlier today.

>

>        Ritch Gordon

Thats ok, you got more input this way. I guess I should read all messages

before replying though. <G>

end

Regards,

Kevin Nardelle

**********************************************

Kevin Nardelle   knardell@accesscom.net

THE TESLA AND HV WEB SITE-VISIT TODAY!

http://www.icorp.net/users/kev/tesla

http://www.accesscom.net/nardell

**********************************************

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Sun, 16 Mar 1997 23:47:28 -0600

Subject: Re: tank circuit

At 04:07 AM 3/17/97 GMT, you wrote:

>From: "R+S Gordon" <ritchcom@execulink.com>

>Date: Sun, 16 Mar 1997 23:07:47 -0500

>Subject: tank circuit

>

>I often see Two different tank circuit configurations, one with the cap

>across the neon's secondary and the other with the spark gap across the

>secondary.  Is one configuration  preferable to the other, and if so, why.

>If anyone can enlighten me on this please reply.

>

>        Ritch Gordon

>

>

The two circuits operate the same. One will not give any better

results than the other.

However the shunt gap (gap across the transformer) helps

isolate the transformer from the ringing tank circuit

by essentially applying a short circuit across the transformer.

This serves to keep RF from getting into the transformer secondary.

But a shunt gap is not a substitute for a safety gap.

You need both. And RF chokes. Place the chokes in the HV lines between

the safety gap and main gap.

The use of all 3 protective measures (shunt gap, RF chokes and safety gap)

will go a long way in protecting your neons from harm.

If you run neons (and even if you don't) this is the circuit to use.

Your transformer will thank you.

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: death.star@sympatico.ca

Date: Mon, 17 Mar 1997 07:33:48 -0500

Subject: I'm new and I need some help

Hi my name is Andrew Todd and I'm a real beginner at building Telsa coils.

If any of you have any designs that I could look at they would be greatly

appreiciated.  I am especially fuzzy on this safety-gap thing.  I know what

it is for but I don't know what it is.  I hope to be able to contribute to

this serve in the future, but until I get a better grasp on what it is I'm

doing I'll just sit back and see what you have to say.   Any diagrams or

suggestions would be greatly appriciated.

Thanks,  Andrew Todd

Get Down Wit yo Bad Self

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Nathan_Bonsal@intuit.com (Nathan Bonsal)

Date: Mon, 17 Mar 1997 09:07:57 -0800

Subject: Re[2]: 10KV RMS @ 300MA for less than $100

--IMA.Boundary.693516858

Content-Type: text/basic; name="note.txt"

Content-Transfer-Encoding: 7bit

Content-Description: MS-DOS text file

Content-Disposition: inline; filename="note.txt"

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Sat, 15 Mar 1997 15:42:25 -0500

Subject: Re: 10KV RMS @ 300MA for less than $100

Dear Fellow Coiler,

I am sending this compendium of posts to all persons that I am currently

aware of who have purchased the H&R transformers, or who expressed an

interest in buying them. I am also sending this set of posts to the Tesla

list and the Tesla2 list (Tesla2 is a new list dedicated to helping

beginner coilers.) The good news is that these babies are quite useful. The

bad news is that I am still awaiting full details on several aspects of

their successful use. Please read *everything* in these posts. The info is

somewhat scattered throughout the posts.

Bottom line: you *need* a safety gap for each transformer. Failure to do so

will almost surely result in early demise of these excellent transformers.

The ones with quick connect connectors are more prone to failure due to the

position of the inside winding quick connect. Immersion in oil should

eliminate all problems with insulation, but safety gaps are still a must.

Due to the large currents produced you need to use a large (such as .06

mfd) HV capacitor in tank circuit. Otherwise your spark gap will flame. The

spark gap has to be really heavy-duty, as the currents are high.

As I get more information in, I will make sure that it gets posted to the

Tesla list and the Tesla2 list. I am sending the current posting to

individuals as well as to the list since I want to alert users of this

transformer as soon as possible so that no one else unneccesarily damages

one of their transformers.

I know that the urge is to just slap these babies into an existing Tesla

system as-is. That could be disastrous. It would be a real shame if these

transformers got a bad reputation simply because we didn't take the time to

implement them properly.

You will note that my own experiments were run without using the resonant

winding and its associated capacitor. Thus I was only generating 1/2 of the

available voltage. That may explain why my abuse of the transformers did

not result in any damage. Personally, I am waiting until Barry Benson

answers my second set of questions before I attempt to run these suckers in

resonant mode. I plan to oil immerse the secondaries first!

Please share with the rest of us your experiences with these transformers.

I hope to take all the information we gather on these things and create a

fact sheet so that others can benefit from what we learn.

Please be careful! There is a *known* failure mode related to the

secondary. Thus we should all be able to work around this failure mode.

Some may try oil immersion. Others may try moving the secondary connections

after careful disassembly. Some may try additional insulation such as RTV

silicone, etc. Whatever you do, please let me know both about your

successes and any failures. Be as explicit as possible about what you did.

Your information is only useful if it is complete and exact. For example,

if using RTV silicone, let me know what brand, how much you used, and any

nitty gritty details that will help others duplicate your success.

I would appreciate it if all users of this transformer would send me copies

of any e-mail discussing these transformers so that I can gather all the

relevant information into a single document that we can post to the lists

and get into the archives.

Fr. Tom McGahee

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: tom_mcgahee@sigmais.com

> Subject: Re: 10KV RMS @ 300MA for less than $100

> Date: Friday, March 14, 1997 9:42 PM

>

> From: Steve Falco <sfalco@worldnet.att.net>

> Date: Fri, 14 Mar 1997 21:42:12 -0500

> Subject: Re: 10KV RMS @ 300MA for less than $100

>

> > Those transformers may not be suitable as TC power supplies. It

> > appears that they may be ferro resonant transformers and even more

> > fragile than neons.

> > I was tempted to get a couple myself but I'm waiting until

> > someone actually reports results.

> > The verdict is still out on these. Since you already ordered 2 of them

> > report back to us what results you get. (even if bad)

>

> I ordered four of these transformers.  One had quick-disconnect

> terminals and the other three were the "directly connected wire" type.

> I hooked them up as stated on the instruction sheet, with the provided

> capacitor to resonate the iron core.

>

> I wired the secondaries in series and the primaries in parallel to get

> 10kv out with 120v in.  I found that I had to tighten up my spark gaps

> considerably to get the coil to fire  (I had been using a 12kv neon).

> The spark gaps had a much greener look from the vaporizing copper

> because of the increased current drawn.  I suspect the gaps would not

> last very long with 3 KW of power applied!

>

This is an important point. The increased current will indeed chew up spark

gaps more quickly. This should come as no big surprise to any of us. The

tuning of the gaps is obviously also an issue. You can't just throw these

transformers in a system an not expect to have to do some tuning and

upgrading of the spark gap system. The extra current availability of the

transformer requires that the capacitor be bigger, otherwise the spark gap

will flame instead of spark. Again, this is not a defect in the

transformer, but something that must be contended with when increasing the

available current in any Tesla coil system.

> The spark output was down considerably from my 12kv 30ma neon presumably

> because of the lower voltage and because the spark gaps were overloaded

> badly.  A much bigger main capacitor might have helped soak up the extra

> current available.

>

Absolutely correct. Unless a larger capacitor is used, the spark gap will

flame and power will be *lost* instead of used.

> Sadly, the transformers only ran for a few seconds, then I saw smoke.

> Turned out the transformer with the quick-disconnect terminals arced

> over between one of the high voltage terminals and the secondary

> windings.  I'm afraid this transformer is totalled, because the paper

> insulation between the secondary layers charred away.

This is precisely the problem that Barry Benson had with his first attempts

at using these transformers. The ones with the high voltage wires tend to

stand up better. The manufacturer has both quick connect HV terminals on

the same side of the transformer, and fairly close to one another. But the

real problem here is that the wire from the center of the secondary coil

has been brought up to be on top of the secondary so that the quick connect

terminal can be soldered to it.

In normal operation this appears to work OK, but in Tesla service the kick

back voltage can initiate a spark between the inner secondary wire and the

outer portion of the secondary. Once a spark has begun, it ignites with a

vengeance because of the current available.

It seems to me that immersion in oil and removal of all air bubbles after

immersion would totally eliminate this failure mechanism. To facilitate the

removal of air bubbles it might be useful to carefully perforate the

protective layer that currently covers the secondary.

The transformer looks like it would be very easy to dismantle. If this was

done, then careful unwrapping of the outer insulatory material might allow

one to attach High Voltage insulated wire in place of the quick connects.

Barry Benson mentioned that he packed his transformers with RTV silicone

for increased HV safety. I assume that he meant that he did that to the

secondary windings after disassembly.

Note that if you use oil immersion you should be able to get by without

having to worry about re-wiring the quick connects, because the oil will

provide adequate insulation.

Another very important thing that we can do to prevent these transformers

from self-destructing is to use a good protective spark gap *at each

transformer*. Please do not skimp here! Even if oil immersed, this item

seems to me to be very important! Do not try to protect all the

transformers with just one protective spark gap! Use a separate gap for

each transformer. Begin with the gaps set too close, and then widen the

safety gaps until they don't fire anymore. This will go a long way towards

protecting these transformers. Do not omit the safety gaps!!!

>

> I'm not sure if I should send back the two transformers I have not

> touched yet or keep them.  At a few seconds of operation per

> transformer, they are pretty expensive.  If they can be made to hold

> together they would be fantastic...

>

Steve, these transformers require some special care, but I believe that in

the end the extra effort will be worth it. I am still waiting for word from

Barry Benson as to further details concerning what we have to do to get

these transformers working in a safe way. I recommend that all of us that

have these transformers *WAIT* until Barry Benson (and anyone else that has

sucessfully used these transformers) can get back to us with full details.

No sense in any more of us having to learn the hard way what the "gotchas"

are with these babies!

By the way, Steve, don't throw the ruined unit out. The core is still

useable, as is the primary. You should be able to use the remnants to build

something useful, such as a primary choke. By adding a heavy duty shorted

secondary (just a few turns of really *thick* wire or rod or bar stock),

you can have a mini equivalent of a shorted arc welder. for use in driving

a primary circuit.

Below I have appended some additional stuff that I hope is useful to all

who have bought these transformers.

>         Steve Falco

>         sfalco@worldnet.att.net

>

>

----------

> From: Mike Hammer <mhammer@misslink.net>

> To: Thomas McGahee <tom_mcgahee@sigmais.com>

> Subject: H&R transformers

> Date: Friday, March 14, 1997 6:28 PM

>

> Fr. Tom,

>

> I guess by now you know that you have created a minor

> upheavel in the coiling community when you told us about

> the transformers that H&R has.

> I received 5 emails today alone from people asking me

> if these transformers are for real.

> It seems almost too good to be true.

>

> Well some people have cast doubt on the suitability of these

> because of them being ferro resonant devices.

>

> I once used a ferro resonant transformer from a large IBM copier

> that sounds similar to these and it expired rather quickly.

>

> So my question is do you have a pair of these?

> Have you done any tests on them?

> Have you run them on a coil?

> Any info would be helpful.

>

> I would like to get a pair of them myself but

> I firmly believe that if something seems to good to be true

> it usually is.

>

> TTYL!

> Mike Hammer

> mhammer@misslink.net

Mike,

I have been busy trying to gather all the info I can both by experimenting

with the two units I bought, and by checking with people that I know have

experienced success with these units.

Since you expressed a desire for this information I am attaching some of my

experiments, one of my queries, and the response I got so far.. as you will

see I followed it up with a request for more information.

The good news is that these puppies are really quite useful, even though

some people have had some bad experiences with them. Barry's experience

shows both the problem aspect, but even more importantly, the final great

result that can be had with a little patience.

These transformers are a bit out of the ordinary, that I will admit! For me

the bottom line is useful results. And Barry Benson is one user who has

used them with success. You may want to wait until he answers the second

set of questions I sent him. I'll tell you this much: if I had the money I

would order another one or two myself and see how far I can push these

suckers! I would want to oil immerse them first, though, as they are *not*

indestructible as they come out of the box. But a little oil can easily

clear up *that* problem!

Please keep in mind that I am new to these transformers myself. I, too have

a lot of questions. But from what I have already seen of the two units I

bought, I would have to say that they were worth what I paid. Stay tuned!

Hope you find this useful.

From: "tom_mcgahee"@sigmais.com@PMDF@PAXMB1

To: Benson Barry

Subject: H&R Transformers

Date: Tuesday, March 11, 1997 9:09PM

<<File Attachment: 00000000.TXT>>

Barry,

I included information you posted on February 11 to Malcolm Watts in the

following post that I have sent out to the Tesla list. I am sending this

copy direct to you, because I would appreciate it if you would send a post

concerning your use of this type of transformer to the Tesla list under the

subject heading "RE: H&R Transformers"

In particular, if you could share with the rest of us your impression of

this transformer, and any pertinent details about using it that you can

share with us. Like, did you remove the shunts? Do you use the AC capacitor

that it came with? Are you happy with the transformer? I know that you are

probably very busy (as we all are), but this is something that you could

share with us that would be most useful. I will be looking foreward to

hearing from you. I would appreciate it if you would CC me a copy of your

response when you send it to the Tesla list, as this will allow me to read

your response earlier.

Thank you for helping Malcolm, by the way. I hope that I can perhaps get

you to fill in a Project Coiler form some time in the near future. Project

Coiler is my ongoing attempt to gather information on operational coils and

make this information available to all interested persons on the Tesla

list. I have a standard form that helps to organize this data in a way that

will hopefully be useful to many coilers.

Fr. Tom McGahee

Here is Barry Benson's response:

Benson_Barry%PAX5@mr.nawcad.navy.mil

TRANSFORMER, PHOTOCOPIER, H & R

Hi Fr. Tom McGahee, All,

I started using these transformers after seeing

a system built with them in the T.C.B.A. news.  I have been

able to get 56" of spark with 120VAC before tripping the

breaker with a 6" X 27" X #18 gauge magnet wire wound

secondary terminated with a 4 X 20" toroid using a 0.06 uF

capacitor and 14 point X 10" diameter X 1725 RPM driven

rotary wheel using 8-32 stainless steel screws with 4

parallel sets of 1/8" stationary tungsten rods.

When I first bought a pair of these transformers 10 years

ago I cooked both of them.  The high voltage winding lead out

was put too close to the low voltage side of the winding.

When stressed with Tesla coil use without protective gaps

it arced over and made one the transformers useless.  I

rebuilt the other by scraping off the carbonized insulation

and repotting with RTV.  I now run with safety gaps and

a 4" X 3" X 1" cross section cored common mode choke

with 100 turns for each side.  This limits the current and

protects the transformers from RF spikes.  I ordered a

new transformer recently that came with silicon rubber

insulated wires for the high voltage lead.  This is a vast

improvement.

These are serious transformers.  They are like mini

pole pigs.  I suppose that if you put one in oil that it

would be indestructible.  I have been thinking of using

six of these transformers with primaries in series parallel

for running 10 KW off of 220 VAC.

Barry

----------

Barry,

Thank you ever so much for responding to my request, and for sending a copy

to the Tesla list. Forgive me, but I have a few questions to ask. They may

seem trivial to you, since you have been using these type transformers for

a while, but there are many of us who are only beginning to use them.

1) Do you use the resonating winding at all? The capacitor?

2) Did you make *any* modifications at all to the transformer itself, such

as removing the shunts? If so, could you explain what you did and how you

did it? What were the practical results? Any "gotchas"?

3) You mentioned repairing one of the transformers. What is the quality of

their original construction in your opinion? Do you remember approximately

what size wire was used for the secondary? Can they really handle the rated

300 ma continuously in your opinion?

4) Of the failure modes you have experienced, what were they, and how can

they be reduced?

5) Have you added any auxiliary stuff such as external current limiting? If

so, be as explicit as you can as to what you have added, and why.

6) The common mode choke you mentioned: is this homemade, or can we buy one

somewhere? Details?

There are a fair number of Tesla list members (myself included) that have

just acquired a few of these puppies, and many more looking on from the

sidelines to see what becomes of our venture. Your practical experience in

this matter can be of great benefit to all of us. I hope that my request

for more information does not put you out, and on behalf of all of us, I

thank you again for what you have already shared with us!

Fr. Tom McGahee

*****

Fellow Coilers,

Had a few hours yesterday and today to do some experimenting with the two

H&R transformers that I have. At the moment I have only one Tesla Coil set up

in my shop. All the others are in various states of disarray as I am

attempting some other experiments with various component parts of the

systems.

The working Tesla coil was being used with a 7,500 volt @ 60 ma

transformer. Not your standard transformer, but that's par for the course

around here, where most everything is scrounged rather than bought. Anyway,

I disconnected the regular transformer and connected up *two* of the H&R

transformers with their primaries in parallel and their secondaries in

series. I did *not* connect up the resonant capacitor that was supplied.

Spark gap used was a single variable static gap (because I wanted to test

the transformers at the extreme by opening the gap all the way out to where

the spark gap stopped firing. For all these tests I removed all  chokes, as

I wanted to see if the transformer was prone to failure).

The results are: the secondary coil produced a spark that was twice as long

and much hotter than with the original transformer. I attribute this to the extra

current capability of the H&R transformers. Please note that I was *not*

using the transformer in its resonant mode. In its resonant mode it

produces 5KV per transformer, but without the resonant mode it acts as a

2,600 volt 300 ma straight transformer as far as I can tell. That means

that together the two transformers were producing about 5,200 volts at 300 ma.

While I am disappointed that the 5KV rating per transformer could not be

attained in straight transformer mode at 120 AC input, I am still happy

with my purchase. I ran the poor thing for 15 minutes continuously as a

sort of torture test. The transformers were just barely warm at the end of

the test. In fact, they were almost cold.

*** note: I am waiting for input from Barry Benson as to whether or not he

uses the resonant winding or not. Until then I am *not* using the resonant

winding, because this way I am using it as a straight transformer. The fact

that I have not burned up my transformers despite the abuse I have been

giving them may be due to the fact that I am *not* using the resonant

winding. I will wait until I hear from Barry Benson before attempting

*that* part of my experimentation!

***

I am interested in seeing what these transformers will do when driving a

decent multiple static gap. By the way, quenching becomes even more

important as you increase the *current* supplied by the transformer. If the

arc is not blasted with air the spark gap spark can begin to flame. My

single static spark gap looks more like a Jacob's ladder or flamethrower

when I turn off the blower. If the gap is allowed to become too hot, then

it flames and secondary spark activity decreases accordingly. This is not

due to any defect in the transformers being tested... it is just what

happens naturally with higher input currents. Make sure you have a spark

gap setup that can handle the increased currents! Otherwise you will get

disappointing results.

I tried running *one* of these puppies with 220 volts on the

primary in straight transformer mode. The core must have been saturating,

because the quality of the spark at the spark gap changed drastically. The

primary warmed up considerably, so I terminated this part of the experiment

after only about 4 minutes of testing. I definitely do not recommend

running these things too much above the rated primary voltage. Looks like

you can push them to 150 VAC with little problem, however, so you can crank

up your variacs on these! *** oil immerse first, please!***

At 220 VAC there was no permanent damage done to the transformer,

but it did heat up, and the output became very flame-like at the

spark gap.

*** I was foolish to try this, but since it was my post that has persuaded

a number of others to stick their necks out and purchase some of these

transformers for themselves, I feel a certain moral obligation to do all

that I can to make sure that no one else has to unnecessarily destroy one

of these transformers.

***

By the way, I have been informed that H&R is supplying several variations

on this transformer. One of them uses quick connect terminals and is very

clean looking, and the other model has insulated wires for all the inputs

and outputs. This second model is the same internally, but does not look as

"spiffy" as the first model. They work identically and have the same

ratings.

*** It turns out that the one with quick connect terminals is less

desireable, as it has more of a tendency to flash over. Read the reply from

Barry Benson for more details.

***

I am sorry that these transformers are not exactly as great as I at first

thought they were going to be. However, it turns out that they are still

quite useable. My tests indicate that it should be possible to run at least

three and possibly even four of these with the secondaries in series. ***

in oil! ***

Remember that the insulation is designed for 5KV operation per transformer,

and that without the resonating capacitor the voltage out is about half that.

Exercise prudent caution with these transformers. The raw arcs really flame up!

If you are concerned about the level of high voltage when putting these in series,

you can opt to connect the secondary center tap of the two-transformer pair to a

Good RF Ground.

There is no need to remove the resonating winding... just don't connect

anything up to it.

*** I am waiting for more information before I attempt to use the

resonating winding. When it is forthcoming I will communicate it to the

Tesla list.

***

Several people asked about using these transformers for Jacob's ladders.

Two of them in series make a pretty good Jacob's ladder. You might want to

try playing around with using the resonant winding and its associated

capacitor in this application. There are several possible combinations that

can be tried. Be aware that the arc is quite HOT, and it will melt metal

electrodes quite readily if the arc is allowed to stay in one place for

more than a fraction of a second. Mine chewed up a chunk of #12 solid

copper wire and spit out little metal balls. So your metal electrodes need

to be something that can take the heat. I only ran a quick test for Jacob's

ladder operation. Enough to show me that the arc produced is very nice. I

did not run an extended test to see if the transformers got warm or

anything. I'll leave that to someone who has nothing better to do with

their transformers. I'm putting *my* two transformers into permanent

service on my present coil. And I hope to maybe add another transformer in

the near future to raise the voltage level up a bit higher.

Someone asked about putting these units in oil. It looks like they would

operate well in such an environment. If you do this, you might want to

carefully perforate the outer protective covering on the HV winding so that

air bubbles can be more readily removed. I will be running my two

transformers as-is for now, and if I experience any future difficulties I

will report them to the Tesla list.

*** Note: It is extremely important that *each* transformer is equipped

with a safety gap. This one little item alone can save you much grief. I

did not use safety gaps during any of my experiments, but that was very

foolish. It appears that the major failure mode for these transformers is

related to internal flash-over arcing when used for Tesla service. This is

especially the case for the units that come with the quick-connect

terminals. Failure to observe safety precautions mentioned can result in

very quick destruction of the transformer. If at all possible, oil immerse

the transformer. I found a sort of rectangular plastic container at the

local plastics store that is just large enough to hold two of the

transformers. I am goind to bolt the two of them together using short metal

strips so that they will not shift relative to one another. I will use 30KV

HV silicone wire to connect the transformer secondaries to HV terminals

that I will install on the lid. I am going to bring out the HV center tap

as well as the two end HV wires, so that I can attach a separate safety gap

to each secondary. The primary wires will also be brought out to the lid.

Since the oil insulation is really only needed at the secondaries, I plan

on stuffing the container with non-conducting stuff that is heavier than

oil. This should greatly reduce the amount of oil I have to use to bring

the level up so that it covers the secondary with an inch to spare. It will

look awful, but this should save me a couple of bucks.

***

It is evident that the increased current capability over that of a neon is

very desireable. I will continue to look for other high current

transformers that can be used in Tesla coil applications.

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla-2@emachine.com (Tesla-2)

Subject: Re: 10KV RMS @ 300MA for less than $100

Date: Sat, 15 Mar 1997 20:42:25 GMT

Message-Id: <9703151514567751@emachine.com>

Organization: The Energy Machine Information System 504-734-0526

To: nathan_bonsal@intuit.com

THIS LIST IS FOR INFORMATION< NOT ADVERTISEMENT. PLEASE DO NOT

POST ADVERTISEMENTS TO ME. I HAVE ALL THE EQUIPMENT I NEED.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Mon, 17 Mar 1997 10:20:21 -0600

Subject: Re: I'm new and I need some help

At 12:33 PM 3/17/97 GMT, you wrote:

>From: death.star@sympatico.ca

>Date: Mon, 17 Mar 1997 07:33:48 -0500

>Subject: I'm new and I need some help

>

> Hi my name is Andrew Todd and I'm a real beginner at building Telsa coils.

> If any of you have any designs that I could look at they would be greatly

>appreiciated.  I am especially fuzzy on this safety-gap thing.  I know what

>it is for but I don't know what it is.  I hope to be able to contribute to

>this serve in the future, but until I get a better grasp on what it is I'm

>doing I'll just sit back and see what you have to say.   Any diagrams or

>suggestions would be greatly appriciated.

>

>                        Thanks,  Andrew Todd

Andrew,

The safety gap is a single static gap placed directly across the

neon transformer output. It is a protective device. Its sole purpose

is to limit the voltage that can appear on the transformer secondary.

RF voltages can and will back up into your transformer secondary

where it will eat away at the insulation until your transformer fails.

The safety gap will limit this voltage to a safe level by arcing

over once the voltage reaches a certain level. You determine

the voltage it arcs over by varying the spacing of the safety gap.

The best method is to connect the safety gap to the neon transformer

by itself. No other components. The gap directly across the neon.

Start with the gap narrow and open it a bit at a time until it will

just barely not break down. Thats the safest setting.

The physical construction of a safety gap can be most anything.

It doesn't need to be efficient like the main gap does.

I use a pair of 1/4 20 bolts placed head to head. One of them

is adjustable.

A more efficient safety gap can be made by placing a grounded

center post in between the 2 electrodes. This is grounded to your RF ground.

The arc overs then go directly to ground.

I have used both methods and either one will be of great help.

TTYL!

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Ken Smith <ksmith@ihug.co.nz>

Date: Tue, 18 Mar 1997 06:37:33 -1200

Subject: Safeties and RF Chokes

For Mike Hammer :

Mike,

you recently answered a question on safety gaps very nicely (in the

proper sense of the word) and although I have one on my system now I was

unsure about the gapping or how to handle a ground.  So I was a wally not to

ask and someone else did - and now I know etc.

So I do have a simple question.  There is discussion about RF Chokes to

protect the Neon also.  Well now I have coils that are coming upto tune I

have obviously got to protect the neons some more.  I have seen what happens

to the voltages when she goes on song and thinking about the secondaries of

those poor neons makes me shudder.

So please.  Can I (we) have a simple explaination of RF Chokes, how to make

one and where to put it please.

Thanks in advance

Ken

Ken Smith

Auckland

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Mon, 17 Mar 1997 12:01:08 -0600

Subject: Re: tank circuit

At 05:28 AM 3/17/97 GMT, you wrote:

>From: Kevin Nardelle <knardell@mailhost.accesscom.net>

>Date: Sun, 16 Mar 1997 23:28:38 -0600

>Subject: Re: tank circuit

>

>At 04:07 AM 3/17/97 GMT, you wrote:

>>From: "R+S Gordon" <ritchcom@execulink.com>

>>Date: Sun, 16 Mar 1997 23:07:47 -0500

>>Subject: tank circuit

>>

>>I often see Two different tank circuit configurations, one with the cap

>>across the neon's secondary and the other with the spark gap across the

>>secondary.  Is one configuration  preferable to the other, and if so, why.

>>If anyone can enlighten me on this please reply.

>>

>>        Ritch Gordon

>>

>>

>Ritch,

>

>in my case and I HAVE NO IDEA WHY but my coil will only work (the gaps will

>only fire) if the caps are across the neon. There is no explanation for this

>because I have tried by many resuests to do it the "conventional" way

>however no results at the gaps.

>I DO NOT have the thing wired wrong because it is not possible to do it

>wrong as many times as I did this "cenventional" hookup. My results are

>fantastic and I would not have it any other way. I am going to install

>overkill protection for my neon because putting the caps shunt across it is

>not good for the life of a neon.

Well Kevin, as I have told you many times before you must

be wiring something incorrectly because the shunt gap configuration

cannot "not work".

But as long as you have a functional safety gap and decent RF chokes

then the shunt cap arrangement will work. I used this arrangement

for a long time before I even knew of other configurations.

<SNIP>

>

>Look at the BIG picture, N tesla used this configuration himself but the

>vast majority of coilers out there today do it with the gaps across the neon.

>This has been rather contraversial as to weather this is a good

>configuration but whatever floats you boat, Keep in mind I have a nice

>supply of backup transformers in case this one dies so I care not about the

>life of this neon.

For the most part Tesla used the shunt gap arrangement. Nearly all the diagrams

in the Colorado Springs Notes show the shunt gap circuit.

Being the experimenter he was he tried virtually every possible configuration

and ended up with the shunt gap placement as being best.

>I know a guy Mr. M.H. that is waitting for it to die so he can get his hands

>on it.:)))

Its only a matter of time! I can't wait.:)

>                                       Regards,

>                                            Kevin Nardelle

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Mon, 17 Mar 1997 12:59:40 -0500

Subject: Fr. Tom's Posts on H&R

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: tom_mcgahee@sigmais.com

> Subject: Re[2]: 10KV RMS @ 300MA for less than $100

> Date: Monday, March 17, 1997 12:07 PM

>

> From: Nathan_Bonsal@intuit.com (Nathan Bonsal)

> Date: Mon, 17 Mar 1997 09:07:57 -0800

> Subject: Re[2]: 10KV RMS @ 300MA for less than $100

>

*** super-SNIP ***

>

>

> THIS LIST IS FOR INFORMATION< NOT ADVERTISEMENT. PLEASE DO NOT

> POST ADVERTISEMENTS TO ME. I HAVE ALL THE EQUIPMENT I NEED.

Dear Tesla-2 list members,

If it is the consensus of list members that you do not want any more

postings about the H&R Transformers, then I will be glad to stop posting on

this subject. I hope that no one else out there thinks that I am trying to

advertise anything. My intent has been to share information that I

personally thought was of some value to other coilers.

In some of my posts I included rather lengthy sub-posts only because I

thought they were important to the discussion. I have tried to report on

both sides of the issue. Experimentation with these transformers is

currently in progress by myself and several other members, and my intent

was to keep all interested parties informed as to what we have found out so

far. I am sorry if my posts have made anyone upset or angry. That surely

was not their intent.

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Nathan_Bonsal@intuit.com (Nathan Bonsal)

Date: Mon, 17 Mar 1997 10:52:23 -0800

Subject: Neons- question on unpotting them

I got 5 15,000V 30Ma neons from a local neon shop. they are literally

FILLED with tar. If I immerse them in kerosene to remove the tar, does

anyone know whether it will damage the shellac on the windings?

I would like to know this because heating them up in my house would be

impossible not logistically, but politically. (wife)

Anybody ever tried this?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Mon, 17 Mar 1997 15:29:18 -0600

Subject: Re: Fr. Tom's Posts on H&R

At 05:59 PM 3/17/97 GMT, you wrote:

>From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

>Date: Mon, 17 Mar 1997 12:59:40 -0500

>Subject: Fr. Tom's Posts on H&R

>

>

>

>----------

>> From: Tesla-2 <tesla-2@emachine.com>

>> To: tom_mcgahee@sigmais.com

>> Subject: Re[2]: 10KV RMS @ 300MA for less than $100

>> Date: Monday, March 17, 1997 12:07 PM

>>

>> From: Nathan_Bonsal@intuit.com (Nathan Bonsal)

>> Date: Mon, 17 Mar 1997 09:07:57 -0800

>> Subject: Re[2]: 10KV RMS @ 300MA for less than $100

>>

>*** super-SNIP ***

>>

>>

>> THIS LIST IS FOR INFORMATION< NOT ADVERTISEMENT. PLEASE DO NOT

>> POST ADVERTISEMENTS TO ME. I HAVE ALL THE EQUIPMENT I NEED.

>

>Dear Tesla-2 list members,

>If it is the consensus of list members that you do not want any more

>postings about the H&R Transformers, then I will be glad to stop posting on

>this subject. I hope that no one else out there thinks that I am trying to

>advertise anything. My intent has been to share information that I

>personally thought was of some value to other coilers.

>

>In some of my posts I included rather lengthy sub-posts only because I

>thought they were important to the discussion. I have tried to report on

>both sides of the issue. Experimentation with these transformers is

>currently in progress by myself and several other members, and my intent

>was to keep all interested parties informed as to what we have found out so

>far. I am sorry if my posts have made anyone upset or angry. That surely

>was not their intent.

>

>Fr. Tom McGahee

>

Fr. Tom,

Well I was one of those that was instigating discussion

about these transformers. I wanted to let people know that these

transformers are not a panacea. They have their use but are not

the end all of Tesla power supplies.

I felt your posts were informative and necessary. I know you

went to a lot of trouble collecting it all.

I just about about ordered a couple of them myself but have decided

against it.

TTYL!

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Mon, 17 Mar 1997 15:29:14 -0600

Subject: Re: Safeties and RF Chokes

At 06:37 PM 3/18/97 GMT, you wrote:

>From: Ken Smith <ksmith@ihug.co.nz>

>Date: Tue, 18 Mar 1997 06:37:33 -1200

>Subject: Safeties and RF Chokes

>

>For Mike Hammer :

>

>Mike,

>        you recently answered a question on safety gaps very nicely (in the

>proper sense of the word) and although I have one on my system now I was

>unsure about the gapping or how to handle a ground.  So I was a wally not to

>ask and someone else did - and now I know etc.

>

>So I do have a simple question.  There is discussion about RF Chokes to

>protect the Neon also.  Well now I have coils that are coming upto tune I

>have obviously got to protect the neons some more.  I have seen what happens

>to the voltages when she goes on song and thinking about the secondaries of

>those poor neons makes me shudder.

>

>So please.  Can I (we) have a simple explaination of RF Chokes, how to make

>one and where to put it please.

>

>Thanks in advance

>

>Ken

>

>Ken Smith

>Auckland

Ken,

This is a highly debatable topic. There is no clear indication

of how much inductance is necessary and under what circumstances

chokes can do more harm than good.

Some have claimed success using rather small chokes. Others (myself included)

had success with chokes wound on large ferrite toroids with a value

of around 2 mh. Others have wound large air core chokes with values

as high as 320 mh.

Everyone believes their own plan is the best. My own belief is that a moderate

amount of inductance (2-10 mh) will do a lot in knocking down the

level of the RF kickbacks. You cannot eliminate the RF entirely through

the use of chokes. To do so would require a more extensive filter

system containing bypass caps as well as chokes.

However I don't believe you need to eliminate the RF. I think all that is

necessary is to knock down the level and let the safety gap take care of the

rest.

As to making the chokes well take your pick. Large 2 inch or more diameter

ferrite toroids would with 120 or more turns of magnet wire work well.

Ferrite rods would work as well.

When using ferrite cores put at least 2 layers of good quality tape over the

cores as they are slightly conductive.

Air core chokes can also be made. You can wind chokes on PVC pipe.

About 425 turns of AWG 28 wound on a piece of 2 inch PVC (2.5 inch OD)

would give a winding just under 6 inches long and an inductance of around

4 mh.

Let me include my famous ascii drawing so you can tell where the chokes

belong in the circuit.

TESLA COIL SCHEMATIC                                ---------

|       | T1

---------

O

X1                                 C1              O

RFC           | |             O  L2

O------------------||||----------| |-----        O

||O       |                  |     | |    |   L1   O

||O       |                  |            |    O   O

-----O||O       * SG               * G1         |--->O   O

O||O--Gnd                                       O   O

-----O||O       *                  *                 O   O

120   ||O       |                  |                 O   O

Volts ||O       |                  |                 O   O

O------------------||||----------------------O   O

RFC                           |

|

Gnd

X1 = Neon Sign Transformer, grounded center tap (case)

SG = Safety Gap.

RFC = RF filter chokes

G1 = Main system (tank circuit) Spark Gap.

C1 = Tank circuit capacitor

L1 = Primary Coil.

L2 = Secondary Coil.

T1 = Toroid discharge terminal

Gnd = RF Ground.

Well there you have it.

To recap, to protect your neons you should:

1. Use shunt gap circuit to isolate neon from RF in tank circuit.

2. Use in line RF chokes to knock down level of RF kickbacks

3. Use close spaced safety gap to take of whatever sneaks by the chokes

and to prevent overvoltage.

4. Use the minimum required amount of main spark gap. A wide gap promotes

kickbacks and encourages overvoltage conditions.

5. Keep run times short. Neons are not continous duty Tesla power supplies.

Long run times will deteriorate the insulation due to coronal heating.

Comments, criticism and cash donations welcome.

TTYL!

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Brian Christian <brianchr@tsixroads.com>

Date: Mon, 17 Mar 1997 16:35:38 -0600

Subject: Secondary coil form

List,

I just got my #24 wire in today and got my 3" pvc form.  I have a few

questions.  One, I also got a 3" pvc cap, is it ok to use this?  Two, I

plan on winding the form tomorrow on a lathe, is there any tips, tricks,

of secrets I should know?  I have already "moisture treated" the pvc and

the pvc cap (Thanks to Mike for this info).  Also I have been told not to

drill any holes in the form.  Any other help would be greatly

appreciated.  Thanks,

Brian Christian

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Nathan_Bonsal@intuit.com (Nathan Bonsal)

Date: Mon, 17 Mar 1997 17:53:34 -0800

Subject: Re: Secondary coil form

Man, I tried it on a lathe with #26 AWG and it kept overlapping- could

be my coordination. I wound mine by hand by putting a dowel through

the spool and rolling the form on my lap. mine's 4.5"X20" with 866

turns. it came out very well but took me an hour and a half to wind.

I won't do it with a lathe again, unless it's a metalworking lathe

with really low-speed capability .

Lots of luck!

______________________________ Reply Separator ________________________________

_

Subject: Secondary coil form

Author:  tesla-2@emachine.com (Tesla-2) at Internet

Date:    3/17/97 10:35 PM

From: Brian Christian <brianchr@tsixroads.com>

Date: Mon, 17 Mar 1997 16:35:38 -0600

Subject: Secondary coil form

List,

I just got my #24 wire in today and got my 3" pvc form.  I have a few

questions.  One, I also got a 3" pvc cap, is it ok to use this?  Two, I

plan on winding the form tomorrow on a lathe, is there any tips, tricks,

of secrets I should know?  I have already "moisture treated" the pvc and

the pvc cap(Thanks to Mike for this info).  Also I have been told not to

drill any holes in the form.  Any other help would be greatly

appreciated.  Thanks,

Brian Christian

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Mon, 17 Mar 1997 19:08:28 -0600

Subject: Re: Secondary coil form

At 10:35 PM 3/17/97 GMT, you wrote:

>From: Brian Christian <brianchr@tsixroads.com>

>Date: Mon, 17 Mar 1997 16:35:38 -0600

>Subject: Secondary coil form

>

>List,

>        I just got my #24 wire in today and got my 3" pvc form.  I have a few

>questions.  One, I also got a 3" pvc cap, is it ok to use this?  Two, I

>plan on winding the form tomorrow on a lathe, is there any tips, tricks,

>of secrets I should know?  I have already "moisture treated" the pvc and

>the pvc cap(Thanks to Mike for this info).  Also I have been told not to

>drill any holes in the form.  Any other help would be greatly

>appreciated.  Thanks,

>Brian Christian

>

>

Kevin Nardelle can give you some tips on winding a secondary on a

lathe. He wound several very smart looking coils very quickly that way.

But I did mine the hard way. Sat down on the floor with the form

and spool of wire and wound it by hand. Took forever.

Real men wind their coils by hand. :)

Secure one end of the wire to the form with some tape or

a dab of hot glue. As you wind try to make the turns tight

against one another. No gaps. Avoid kinking the wire. A kink

will be hard to straighten out and wont lay flat on the form.

If you need to stop for any reason put a strip of tape on the winding

so it doesn't unravel.

Once the winding is on start with the polyurethane varnish.

If doing it on a lathe let the lathe turn the form slowly

while applying the varnish. Put it on fairly thick but dont

let it run. Here is where the lathe comes in handy.

Put at least 3 good coats on it and let dry thoroughly.

Below the winding epoxy a small rectangle of copper or brass

sheeting to the form. Solder the bottom end of the secondary winding

to this plate. This becomes your RF ground connection.

You can solder a ground lead to the plate or hold a lead or strap

to the plate with a turn of tape. I prefer to solder.

In this way you place no stress on the secondary winding.

For the top seal the end of the form with a disk of Lexan

or plexiglass. Make it fairly thick. I epoxied 2 disks of

3/32 plexiglass together to make one 3/16th inch thick disk.

Epoxy this into the end of the top of the form.

Take your end cap and mount a 1/4 20 bolt into the end of it.

Place the cap over the end of the form with the bolt sticking up.

This bolt becomes your attachment point for your various discharge

terminals and toroids.

Spiral up the free end of the secondary wire and either wrap the free

end around the bolt or solder a ring terminal on the end of it.

This is my method of secondary construction. There are as many ways

to do it as there are coilers.

The important points are not to drill any holes in the form

and not bring any of the secondary wire inside the form.

Seal the end of the form. This is all to prevent

the secondary discharge from arcing down the inside of the form.

You will realize how important this is once you see a coil

throw sparks twice the length of the secondary form or more.

TTYL!

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Steve Falco <sfalco@worldnet.att.net>

Date: Mon, 17 Mar 1997 20:05:34 -0500

Subject: Re: Secondary coil form

>         I just got my #24 wire in today and got my 3" pvc form.  I have a few

> questions.  One, I also got a 3" pvc cap, is it ok to use this?  Two, I

> plan on winding the form tomorrow on a lathe, is there any tips, tricks,

> of secrets I should know?  I have already "moisture treated" the pvc and

> the pvc cap(Thanks to Mike for this info).  Also I have been told not to

> drill any holes in the form.  Any other help would be greatly

> appreciated.  Thanks,

> Brian Christian

The glue normally used for PVC is flammable.  You may wind up with an

explosive mixture in the tube if you use it.  I have read that epoxy is

best for this application.  I am planning to cut round plexiglas plates,

and epoxy them on the ends of my next coil.

Not drilling holes in the form is correct.  You don't want to put any

metal inside the form either.

Does your lathe have a "back-gear" for very slow running?  I wound my

first coil with my lathe.  Even at 40 or so RPM it was tough to

control.  My next coil was wound by hand and came out much neater.  I

cut some blocks to fit inside the form, with 1/2" center holes.  I ran a

1/2" dowel through them, and into some supports which I C-clamped to my

bench.  Instant coil winder.  I just turned the tube with one hand while

guiding the wire with my other hand.  It went pretty fast, and was much

neater than with the lathe.

--|-@@@@@@@@@@@@@@-|--

C     |                |     C

======|                |======

The C's are C-clamps holding the supports.

The @@@ are the coil form, and the --- are the dowel.

Number 24 wire is fairly thick, so it will not take too many turns.  I

did a coil in #29, which took forever to wind.  One more thing.  I

periodically used my fingernails to tap the turns tightly together (say

after winding each 3/8").  If you look closely, you will see some small

spaces that close up when you do this.  Not essential but makes things

neater.

Steve Falco

sfalco@worldnet.att.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

Hi everyone!

This admin post is being placed in both the TESLA and NEWMAN mailing

lists, as this addresses a common problem to both lists.

As you all are aware, occasionally folks will send encoded files

through the mailing list, either as an attached file or as a file

included in the message.  Many of you have been unable to receive these

files as attachments, or been unable to decode the files when they are

included in the messages.

In general, any files sent as file attachments or included in a

message should be short to as to prevent unnecessary traffic, as well as

avoiding clogging up everyones mailbox.  That's the key word - SHORT.

It is important to understand how files are handled  for transmission

through the internet to understand what is happening.  The internet is

basically a 7-bit world, since it was designed for sending text

messages.  Most files, however, tend to be 8-bit creatures.

Shortly after the first message was sent, someone wanted to be able

to send files as well, but realized that they could not do it since the

files were 8-bit. Sending an 8-bit file through a 7-bit transmission

system is similar to trying to fly an airliner through a railroad

tunnel; something's gonna' get trimmed off around the edges!

Not to be outdone, a method of encoding files into 7-bit format was

devised, called UUENCODE.  This allowed files to be sent through the

internet text based transmission system  and decoded at the other end

by a program called UUDECODE.

There a problem with UUENCODE.  The encoding process makes the

resulting encoded file about 30% bigger than the original file.  Later

methods of encoding files, such as MIME, SHIP, and others, result in a

less bloated encoded file.

Another more serious problem is really a limit of the internet

protocol itself which allows for transmission of messages no larger than

64K. What this means is that if you send a file which encodes out to be

larger then 64K, the system will split the file into segments for

transmission. It's then up to the recipient of the file(s) to

reassemble and decode them.

The server which handles this mailing list runs on Wildcat!

software, and uses the original UUENCODE and UUDECODE protocols to

handle files via the internet.  Although this is the oldest and least

efficient method of handling files, it is also the most compatible

method.  As far as I know, UUENCODE is understood by all computer

platforms available, and versions of it have been ported to just about

every software mailer which runs on the internet on any computer.

If you attach a file to a message sent out by a Wildcat! server, the

message will be UUENCODED and sent as an attached file.  The file will

then be decoded and attached as a separate file to the message at the

receiving end.   Wildcat! systems always do this; other systems may not.

Should the receiving system not know how to handle UUENCODED files, the

file will appear as part of the message, in its UUENCODED form.  It will

be necessary for you to decode it in that case.

Note that this is only true if the encoded file and the message text

total LESS than 64K.  If the total message + file size is LARGER than

64K, the message will be split into segments and will NOT be decoded at

the receiving end in most cases.  It will be up to you to decode it

after reassembling the segments.

So, if you must send a file through this mailing list server, either

as an attached file or included in a message, it should be UUENCODED.

Using other types of encoding will probably result in everyone getting

the file included in your message(s) as many lines of random text

characters.

Ralph Hartwell

List Owner, NEWMAN-L, TESLA-2, DOWNLINE

---

Ever notice how fast Windows runs? Neither did I...

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Roderick Maxwell <major@vicksburg.com>

Date: Mon, 17 Mar 1997 23:12:14 -0800

Subject: Re: Fr. Tom's Posts on H&R

Tesla-2 wrote:

>

> From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

> Date: Mon, 17 Mar 1997 12:59:40 -0500

> Subject: Fr. Tom's Posts on H&R

>

> ----------

> > From: Tesla-2 <tesla-2@emachine.com>

> > To: tom_mcgahee@sigmais.com

> > Subject: Re[2]: 10KV RMS @ 300MA for less than $100

> > Date: Monday, March 17, 1997 12:07 PM

> >

> > From: Nathan_Bonsal@intuit.com (Nathan Bonsal)

> > Date: Mon, 17 Mar 1997 09:07:57 -0800

> > Subject: Re[2]: 10KV RMS @ 300MA for less than $100

> >

> *** super-SNIP ***

> >

> >

> > THIS LIST IS FOR INFORMATION< NOT ADVERTISEMENT. PLEASE DO NOT

> > POST ADVERTISEMENTS TO ME. I HAVE ALL THE EQUIPMENT I NEED.

>

> Dear Tesla-2 list members,

> If it is the consensus of list members that you do not want any more

> postings about the H&R Transformers, then I will be glad to stop posting on

> this subject. I hope that no one else out there thinks that I am trying to

> advertise anything. My intent has been to share information that I

> personally thought was of some value to other coilers.

>

> In some of my posts I included rather lengthy sub-posts only because I

> thought they were important to the discussion. I have tried to report on

> both sides of the issue. Experimentation with these transformers is

> currently in progress by myself and several other members, and my intent

> was to keep all interested parties informed as to what we have found out so

> far. I am sorry if my posts have made anyone upset or angry. That surely

> was not their intent.

>

> Fr. Tom McGahee

>

Please send ME all of the information on these transformers that you

can muster! I've just bought two of them, and I don't want to fry them

for all the tea in China.

Frankensteins Helper

Max

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Bert Hickman <bert.hickman@aquila.com>

Date: Mon, 17 Mar 1997 19:57:37 -0800

Subject: Re: Neons- question on unpotting them

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: Nathan_Bonsal@intuit.com (Nathan Bonsal)

> Date: Mon, 17 Mar 1997 10:52:23 -0800

> Subject: Neons- question on unpotting them

>

>      I got 5 15,000V 30Ma neons from a local neon shop. they are literally

>      FILLED with tar. If I immerse them in kerosene to remove the tar, does

>      anyone know whether it will damage the shellac on the windings?

>

>      I would like to know this because heating them up in my house would be

>      impossible not logistically, but politically. (wife)

>

>      Anybody ever tried this?

>

Nathan,

Kerosene will work just fine and won't mess up the shellac (or whatever

type varnish is used on the windings). If you first freeze the units,

and then chip off the big chunks of tar, the job will go quicker.

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: "J. Bazemore" <jpbazemo@ouray.cudenver.edu>

Date: Tue, 18 Mar 1997 00:16:12 -0700 (MST)

Subject: Re: Neons- question on unpotting them

On Tue, 18 Mar 1997, Tesla-2 wrote:

> From: Bert Hickman <bert.hickman@aquila.com>

> Date: Mon, 17 Mar 1997 19:57:37 -0800

> Subject: Re: Neons- question on unpotting them

> Organization: Stoneridge Engineering

Note: Gentlemen--I already have tried this, and I live

with three roommates, 2 guys and a girl, and it is pretty

bad--I mean, I had a blown 120ma 7200V xformer and I tried

to unpot it in the oven, and it leaked--big, big mistake.

So, I just put it in kerosene, and that does the

trick?  Any comments/tips would be most welcome.

-Jon

>

> Tesla-2 wrote:

> >

> > From: Nathan_Bonsal@intuit.com (Nathan Bonsal)

> > Date: Mon, 17 Mar 1997 10:52:23 -0800

> > Subject: Neons- question on unpotting them

> >

> >      I got 5 15,000V 30Ma neons from a local neon shop. they are literally

> >      FILLED with tar. If I immerse them in kerosene to remove the tar, does

> >      anyone know whether it will damage the shellac on the windings?

> >

> >      I would like to know this because heating them up in my house would be

> >      impossible not logistically, but politically. (wife)

> >

> >      Anybody ever tried this?

> >

>

> Nathan,

>

> Kerosene will work just fine and won't mess up the shellac (or whatever

> type varnish is used on the windings). If you first freeze the units,

> and then chip off the big chunks of tar, the job will go quicker.

>

> -- Bert --

>

>

>

>

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Jochen Kronjaeger <Kronjaeg@Stud-Mailer.Uni-Marburg.DE>

Date: Tue, 18 Mar 1997 22:48:57 +0100 (CET)

Subject: will this sec. work?

Hi all,

This is what I've planned for my first secondary:

- PVC tube 11cm diameter

- 330m of magnet wire (0.3mm diameter)

- this will make 950 turns

- at estimated 34mm coil length per 100 turns (tested), the sec. will be

32 cm long.

According to TESLAC and TESLANS, the res. frequ. will be about 360kHz,

the coil having 7-8pF stray cap. and 28mH inductance.

Questions: Will this work? Are the figures right?

How important is it to match the lambda/4 resonating wavelength

(lamda/4 = total wire length = 330m here => 227kHz)?

I only read about this, what's your experience?

How do I make a toroid (easiest :-)? Does the surface have to

be metal, or can I use graphite spray? What about aluminium air

ducts?

That's it so far.

I won't be able to actually wind the coil in the next few days (or even

weeks), so no hurry (and no quick feedback either).

Thanks!

Jochen

+---------------------------------------------------------------------------+

| Jochen Kronjaeger                     Kronjaeg@stud-mailer.uni-marburg.de |

|                                                                           |

| The High Voltage Page  http://www.mathematik.uni-marburg.de/~kronjaeg/hv/ |

|   info, plans, photos of self-built HV-sources, sparks, HV-experiments    |

+---------------------------------------------------------------------------+

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: "R+S Gordon" <ritchcom@execulink.com>

Date: Tue, 18 Mar 1997 18:26:26 -0500

Subject: Secondary winding

I thought I would share my method of winding coils.  First obtain two end

cap for the size form you are using (the flimsy type are ok too).  Next get

two L-brackets greater in length than the form radius and attach one of

these to the centre point on each end cap (get creative with your hardware

here).  Place the caps on the form and then lay the whole assembly

horizontal and screw the L-brackets down.  Now you have a rolling coil form

and all you need is a high torque dc motor to drive it.  I used an old belt

about 1/4"wide by 16" long sliped over the form (before mounting) and

driven directly off the motor spindle.  I also used a variable DC supply to

control the speed.  Even if your motor needs a little help pulling the wire

at least the motor will provide some back tension on the wire.  I used this

method on my 4.5" coil and it worked great!

Ritch Gordon

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Nathan_Bonsal@intuit.com (Nathan Bonsal)

Date: Tue, 18 Mar 1997 16:37:44 -0800

Subject: Maybe

-

Mathematically looks fine according to my calculator. Don't forget

that even though the coil's resonant frequency is 360Khz, once you put

a terminator sphere or toroid on it the res freq will drop. sounds

possible to get it to 227Khz

Take this with a grain of salt though as theory doesn't always match

practice and my first coil is still under construction.

And to everyone on Tesla-2: Sorry I got snooty yesterday about posts

on the transformers. I was being a prick yesterday and I admit it

freely. My apologies especially to Kevin N. whose posts are really

helpful.

______________________________ Reply Separator ________________________________

_

Subject: will this sec. work?

Author:  tesla-2@emachine.com (Tesla-2) at Internet

Date:    3/18/97 9:48 PM

From: Jochen Kronjaeger <Kronjaeg@Stud-Mailer.Uni-Marburg.DE>

Date: Tue, 18 Mar 1997 22:48:57 +0100 (CET)

Subject: will this sec. work?

Hi all,

This is what I've planned for my first secondary:

- PVC tube 11cm diameter

- 330m of magnet wire (0.3mm diameter)

- this will make 950 turns

- at estimated 34mm coil length per 100 turns (tested), the sec. will be

32 cm long.

According to TESLAC and TESLANS, the res. frequ. will be about 360kHz,

the coil having 7-8pF stray cap. and 28mH inductance.

Questions: Will this work? Are the figures right?

How important is it to match the lambda/4 resonating wavelength

(lamda/4 = total wire length = 330m here => 227kHz)?

I only read about this, what's your experience?

How do I make a toroid (easiest :-)? Does the surface have to

be metal, or can I use graphite spray? What about aluminium air

ducts?

That's it so far.

I won't be able to actually wind the coil in the next few days (or even

weeks), so no hurry (and no quick feedback either).

Thanks!

Jochen

+---------------------------------------------------------------------------+

| Jochen Kronjaeger                     Kronjaeg@stud-mailer.uni-marburg.de |

|                                                                           |

| The High Voltage Page  http://www.mathematik.uni-marburg.de/~kronjaeg/hv/ |

|   info, plans, photos of self-built HV-sources, sparks, HV-experiments    |

+---------------------------------------------------------------------------+

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Tue, 18 Mar 1997 17:47:06 -0600

Subject: Re: will this sec. work?

At 09:48 PM 3/18/97 GMT, you wrote:

>From: Jochen Kronjaeger <Kronjaeg@Stud-Mailer.Uni-Marburg.DE>

>Date: Tue, 18 Mar 1997 22:48:57 +0100 (CET)

>Subject: will this sec. work?

>

>

>Hi all,

>

>This is what I've planned for my first secondary:

>

> - PVC tube 11cm diameter

> - 330m of magnet wire (0.3mm diameter)

> - this will make 950 turns

> - at estimated 34mm coil length per 100 turns (tested), the sec. will be

>   32 cm long.

>

> According to TESLAC and TESLANS, the res. frequ. will be about 360kHz,

> the coil having 7-8pF stray cap. and 28mH inductance.

>

>Questions: Will this work? Are the figures right?

Yea it will probably work but its far from being optimum for that

size of form. The figures you got from Teslac are pretty close to what I got.

Your .3mm wire is fairly close to AWG 28 and thats a bit small for this size

of coil. A better secondary will result from using AWG 24 which is about .51mm.

This will give you a longer coil capable of handling much more power. Go with a

winding about 50 cm long. This will give you around 900 turns. Resonant

frequency about 425 KHZ.

>

>           How important is it to match the lambda/4 resonating wavelength

>           (lamda/4 = total wire length = 330m here => 227kHz)?

>           I only read about this, what's your experience?

Not at all important. I have no idea what the length of wire in my

secondary is. And it doesn't really matter. What is important is that

your primary circuit resonates at the same frequency as your secondary.

>

>           How do I make a toroid (easiest :-)? Does the surface have to

>           be metal, or can I use graphite spray? What about aluminium air

>           ducts?

Should be metal.

Flexible aluminum ducting works well wrapped around two back to back pie pans.

Heres a crude ascii diagram that I stole from somebody.

.                        .

.     .\    Pie Pans    /.     .

\--------------/     Dryer Duct

.     . /--------------\ .     .

.   /                \   .

This shows an edge on cross sectional view.

> Thanks!

>

>

>  Jochen

>

TTYL!

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Ken Smith <ksmith@ihug.co.nz>

Date: Wed, 19 Mar 1997 16:48:01 -1200

Subject: Yet another set of coil piccies

Hi,

Another web page for your tesla bookmarks (perhaps).  I have

documented, roughly, my Mark IV coil and will have the Mark V unit on in a

couple of days.

It may be worth a look since some of lessons I learnt are valuable.  There

is much work to do on the section, but I will get it done, by and by.

All comments are welcome - good or bad.  In fact bad is good - you learn more.

Ken

Ken Smith

Weymouth

Auckland

http://homepages.ihug.co.nz/~ksmith

ksmith@ihug.co.nz

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: "Chad Westbrook" <emu@clarityconnect.com>

Date: Wed, 19 Mar 1997 07:06:06 -0500

Subject: Reciver

What is a tesla coil reciver can you make one?????

How could you make one?

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: rpittman@juno.com

Date: Wed, 19 Mar 1997 08:04:29 PST

Subject: second try

Mike and all,

Well I finally got another chance to play with the coil a little

more yesterday.  I shrunk the toroid so that it is now about 4" x 15".

After running it for a while with the pie plate I decided to try the

toroid.   Interesting thing was I could get a much hotter arc to a

grounded object near by but I could still not get it to spark off into

air.  I measured the distance that I could draw a continuous white arc

at about 8.5 ".  I got rid of the slow starting problem by placing a

jumper cable on the spark gap to loose one gap.

Right now I am running 12 salt watter snapple bottles in

parrallel and another 6 connected to that bank in series.  I also have a

very small 6kv 30 ma neon, and 983 on a 4.27" o.d. pvc drain pipe.  I

tried adding another 6 bottles in both series and parrallel but the

spark decreased (I did not have time to move the tap at all which is on

turn 11 of a 14 turn 10 ga helical primary.)

Any suggestions?  Could I readjust the tap with the extra cap to

get a longer spark?  Interesting thing happened also if I did not put a

grounded object near the toroid, the top end of the primary had major

corona loss, when there was a grounded object it sparked over fine. why?

Once again I really appreciate all the great help I have received.

Adios

Ronnie

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla@america.com (Bob Schumann)

Date: Wed, 19 Mar 1997 11:13:47 -0500 (EST)

Subject: Re: second try

>From: rpittman@juno.com

>Date: Wed, 19 Mar 1997 08:04:29 PST

>Subject: second try

>

>Mike and all,

>        Well I finally got another chance to play with the coil a little

>more yesterday.  I shrunk the toroid so that it is now about 4" x 15".

>After running it for a while with the pie plate I decided to try the

>toroid.   Interesting thing was I could get a much hotter arc to a

>grounded object near by but I could still not get it to spark off into

>air.  I measured the distance that I could draw a continuous white arc

>at about 8.5 ".  I got rid of the slow starting problem by placing a

>jumper cable on the spark gap to loose one gap.

>        Right now I am running 12 salt watter snapple bottles in

>parrallel and another 6 connected to that bank in series.  I also have a

>very small 6kv 30 ma neon, and 983 on a 4.27" o.d. pvc drain pipe.  I

>tried adding another 6 bottles in both series and parrallel but the

>spark decreased (I did not have time to move the tap at all which is on

>turn 11 of a 14 turn 10 ga helical primary.)

>        Any suggestions?  Could I readjust the tap with the extra cap to

>get a longer spark?  Interesting thing happened also if I did not put a

>grounded object near the toroid, the top end of the primary had major

>corona loss, when there was a grounded object it sparked over fine. why?

>        Once again I really appreciate all the great help I havereceived.

>Adios

>Ronnie

Ronnie,

About 8 inches will be your output so I presume you are

close to tune. I had a system that used a 6kv@30ma once. The

transformer was open core type. I don't think that transformer

will provide enough power to cause discharges to break out

freely from your toroid. I ended up using a brass doorknob to

get free breakout. I would not do this anymore because of the

corona losses at the top of the coil. If your toroid is bigger

then your secondary diameter (which it should be) and you still

have corona losses at the top of the secondary then bring your

toroid closer to the top of the secondary if you happen to

have it raised. Since there is not enough power to cause free

breakout, you can force the breakout by placing a nail, pin,

or other sharp object on the toroid to focus the discharge to

breakout from a point. You will always get a brighter arc to

RF ground as the brighter arc is a signature of greater

current flow and RF ground is pretty much max current!

Messing around with components, tuning and coupling can get

better output (thats the fun part!) but I still believe it

will be hard for that transformer to get discharges to freely

breakout of the toroid. Down the road you might want to try

the inverted cone primary type. You will better field distribution

over the secondary then with the helical type. I have seen it

with my own eyes!

Hope this helps some,

Bob Schumann

tesla@america.com

http://www.america.com/~tesla

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Ken Smith <ksmith@ihug.co.nz>

Date: Tue, 18 Mar 1997 20:38:37 -1200

Subject: Re: Secondary coil form

>Does your lathe have a "back-gear" for very slow running?  I wound my

>first coil with my lathe.  Even at 40 or so RPM it was tough to

>control.  My next coil was wound by hand and came out much neater.  I

>cut some blocks to fit inside the form, with 1/2" center holes.  I ran a

>1/2" dowel through them, and into some supports which I C-clamped to my

>bench.  Instant coil winder.  I just turned the tube with one hand while

>guiding the wire with my other hand.  It went pretty fast, and was much

>neater than with the lathe.

>

Hi,

I may have something to add here.  When I was very young (so many years

ago - sigh), my maternal grandfather was an engineer with Pirelli Cables in

Southampton, England.  Back then, in the late 50's they were winding huge

undersea cables with hundreds of cores.  This was a real bitch of a job and

the end result was as thick as your upper arm (unless you are a weight

lifter <g>). One of the things that remains in my mind from these visits are

the strips of tape that were on every winding jig.  These were at an angle

and represented the 'error angles' for a given cable.  Hard to describe -

but I will try - since it has distinct relevance to the current issue :

1       the source of the wire must be a good distance from the winding drum

/ coil.  The more the better - I use 10 ft.

2.      For any given wire guage, drum / coil diameter / wire tension there

is a different set of angles.

3.      The first angle defines where the wire will ride up over the

previously wound.

4.      The second angle defines where the wire will go loose.

You are using the back tension on the wire to provide a tight wind.  In

empirical terms it is about 15 degrees for 24G wire.  But that is very

dependant on tension.  A good tension - I use about an arms worth - maybe 10

lbs.

The big thing is constant tension.  Take time to construct some method of

holding constant tension and a relatively constant angle to the coil.  Put

an intermediate table in place with a couple of indicators to show the

limits of 3 & 4 above.  Then start the mother and let her go.  I use 60-100

RPM and all is good (on a varnish base to provide a slightly tacky surface).

Move the providing source to keep the angle within the limits described above.

It is worth the experimentation to get all this right.  The results are

startlingly pretty and effective....

Hope this helps

Ken

Ken Smith

Weymouth

Auckland

http://homepages.ihug.co.nz/~ksmith

ksmith@ihug.co.nz

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Bert Hickman <bert.hickman@aquila.com>

Date: Tue, 18 Mar 1997 06:39:49 -0800

Subject: Neon Repair (Part 1- R. Quick)

Organization: Stoneridge Engineering

There's recently been a number of questions about neon sign transformer

repair. I'm posting a couple of excellent articles by Richard Quick and

Tim Chandler that should cover the topic quite well. There were

extracted off one of the large archive files on the Tesla Funet site.

Note: the techniques described only work for those transformers potted

in asphalt/tar. Some newer units use other types of potting compounds

which may not be removable. Although Richard does mention the use of

gasoline, I'd recommend using safer, less volatile solvents like mineral

spirits or kerosene. These articles are "keepers" if  you are into

serious repair work!

Good luck and safe unpotting. :^)

-- Bert --

Richard Quick's Excellent Post:

=======================================================================

First call the local neon shop(s) and tell them that your working

with Tesla coils. Ask them to hold all of their failed xfrmrs so

you can pick them up. Make sure you talk to the boss or foreman,

and tell them that you want dead units. I have never had a shop

turn me down for free cores. They are happy to get rid of them.

There are two types of failed neon xfrmrs: warranty units, and

old junk. The local shop must return units that fail within the

two year warranty period back to the distributor for credit. Old

junk (older than two years) you can pick up for free right from

the local shop, but I also scavenge from the distributor. Ask

where the failed warranty units go.

If you can locate the distributor who sells wholesale, and

handles failed warranty units, you have found a gold mine of high

voltage xfrmrs. The distributor removes the PLATE from the xfrmr

for return to the manufacturer, and throws the unit away. The

manufacturer credits the distributor for the plate, as the

shipping is too expensive. The cores go to the dumpster.

After locating your source of failed units, be selective. Try to

bring home the high current units. Ratings commonly used are 9

kv, 12 kv, & 15 kv, with common current ratings of 30 & 60 ma.

Once in awhile you will come across a 120 ma unit. I grab all

of the high current units (60 ma+) I can get in these voltages.

If you can't find any 60s, then grab all the 30s.

First test your units. Use wire with a 15 kv rating or better.

This wire can be obtained where you pick up the transformers.

If you ask they will usually cut you off a few feet for free.

I prefer using the solid polyethylene core from RG-213 or poly

core RG-8 coax, as it will withstand the voltage with gobs of

extra safety margin. Draw an arc from the HV bushing to the case,

one at a time.

About 50% of the "failed" units I pick up are just fine and need

nothing other than a clean up. There is nothing wrong with them.

Often shops get these units from signs they have dismantled, and

they just toss them into the junk pile with the rest. The other

50% are bad. Either one, or both, of the HV windings have broken

down. These units can frequently be repaired:

Remove all hardware, and insulators if possible. Take a hammer

and a chisel and remove the cases by splitting them down the

corners. Break off any stubborn insulators, but try to preserve

the lead wire. You are left with a block of asphalt. Set the unit

outside when it is very cold, and let it freeze solid overnite;

or place in a freezer or deep freeze until frozen solid. The next

morning, short the high voltage lead wires with a clip lead, and

connect 110 volts across the primary. Since the cores on these

transformers are shunted, they may be shorted without harm or

blowing fuses. Let the unit cook for 15-30 minutes.

Disconnect your leads, and with the chisel and hammer, chip a

groove around the block. You want to score a groove lengthwise

that will allow the block to cleave in two. Then starting from

one end of the block, chip until you hit the core, then do the

same with the other end. Pry and chip the asphalt away from the

core until the xfrmr is free. The core may then be disassembled,

and the windings removed and examined. Kerosene and a stiff brush

will clean up the windings and core of any remaining asphalt.

The "cold-cook" method is fast, clean, and works very well. Since

the asphalt is frozen and brittle, it chips away cleanly. The

"cooking" softens up the asphalt around the core, allowing it to

release. The other ways to free the cores are long soaks in sol-

vent such as kero, diesel, or gas, (the nasty waste makes a good

driveway crack filler and roofing patch), or melting out the

asphalt with external heat from a hot plate, oven, or fire.

Solvent soaks are the best done outside in summer. Get a rect-

angular kitchen type trash can. A plywood board may be used for a

lid. Fill the container with about two gallons of solvent (enough

to submerge the xfmr completely). Prepare the transformer as

outlined previously by removing the case and hardware. Submerge

the alphalt block in the solvent for several days. Remove the

unpotted core and clean up with kero and a stiff brush. This

method works best in warm weather; when it gets cold unpotting a

core this way can take a week or more. The solvent can be reused

to unpot three to five cores before it must be replaced.

Melting out the asphalt is best done outside in summer. You will

need two matching galvinized steel pans, such as those used for

catch pans during oil changes. A turkey roasting pan with a lid

will also work. Place a hot plate on some ceramic tiles. Set the

hot plate on high and center a metal pan in the heating coil. Set

the asphalt block in the pan, then cover. It will be necessary to

turn the asphalt block several times. Depending in the wattage of

the hot plate, the temperature may also need adjusting if the

asphalt begins to boil, smoke, and char. You can use a small

charcoal fire in place of a hot plate, but I feel this method is

excessively dangerous: simply set the pans up on bricks, and

build a small charcoal fire under the pan.

There is a new type of neon potting that has emerged recently.

This new potting is a cream colored opaque polymer of some type.

It is not affected by the solvents normally used to unpot neon

cores. It is recommended that this material be removed by the

"cold-cook" method. Freezing makes the material very brittle and

it will chip away easily, the warm core will allow the polymer

to release from the windings without too much difficulty.

Most neons fail when the high voltage breaks down the asphalt

potting. The resulting carbon track bypasses and shorts out the

winding. Simply removing the carbon rich asphalt brings them back

to life. Other times the coils break down internally. In this

case I discard the winding after disassembling the core, and

replace it with a good winding from another unit of the same

model with the same type failure. Sometimes all that is wrong

is a faulty ground connection between the winding and core.

While the core is apart, you can beef up the current output by

removing a few of the shunting plates between windings. Never

take out more than 2 or 3 of these plates per side, as the

additional power output will burn out the secondaries. Generally

I get about 70-75 ma out of 60 ma units after I have finished.

Rebuilt units need a little protection from the high voltage

secondary outputs. The first thing I do is solder on a new lead

wire to the high voltage windings. The HV secondaries are wound

with very fine magnet wire, in the 30 ma units the wire is not

much thicker than a coarse hair. Once a good solder connection

is made, bed the connection and the first 1/2 inch or so of lead

wire to the top of the HV winding with hot glue or clear epoxy.

The lead wire need not be anything special, any thin insulated

stranded wire may be used. Heavy wire increases the chances of a

failed connection due to mechanical stress. When setting the unit

up to fire you simply have to route all high-voltage leads on

insulators.

The windings themselves are wedged against the core to prevent

vibration. I have seen wood, bakelite, and plastic wedges used

commercially. What I like to do is to soften up some 30 mil

polyethylene plastic sheet in boiling water, and heat the core in

a warm oven. I wrap dry softened plastic around the core and

gently force the windings down on it. Once cooled, the windings

have some insulation from the core, and they will not vibrate.

The base wire from the HV windings must be grounded to the core.

Use the original grounding point if possible, if not you may

split the core apart slightly with a thin blade and insert the

base wire into the gap before you clamp the core back up. If

required you may also splice on a small piece of wire for added

length.

Neon sign transformers that have been rebuilt may be fired dry.

The asphalt used to pot the cores for neon use does not really

insulate well against the RF and kickback from the Tesla Tank.

The units last longer when they are freed of the asphalt potting.

The only other choice is to sink rebuilt units in mineral or

xfrmr oil (like Shell Diala-X) which is a very good RF insulator.

I choose to fire them "dry"; it works, and there is no mess.

Neons may be run in parallel to deliver the current required to

fire medium sized coils, and I have run up to 4000 watts with

banked neon power supplies. The general practice is to run these

banks off of 240 volt feeds controlled through a variac. Neons

with matched outputs are run in pairs in these banks. The

primaries are paired up in series, and the secondaries are all

paralleled to the HV buss. Phasing is important here, and each

transformer must be checked as it is added to the bank to ensure

it is in phase with the other units. If an xfrmr draws an arc

from a lead wire brought to the HV buss, the primary or secondary

connections must be reversed.

Neons typically have an efficiency of about 50%, in that they

draw twice as much RMS power as they put out. This problem can be

resolved with the use of power factor correction (pfc) capaci-

tance across the line. The pfc capacitors used are the same as

for alternating current motors. The voltage rating should be at

least twice the line current used, and I like a 4x voltage margin

for long life. The formula used to determine ballpark pfc is as

follows:

9

10^

C = Corrected kVA  ------ 2

2(pi)fe^

This should read C = Corrected kVA times (10 to the ninth power)

over, (2 pi times f times e squared)

C = required capacitance in microfarads

f = frequency of applied voltage

e = applied voltage

CORRECTED kVA is determined by dividing the corrected power

factor output of the neon sign xfrmr (Volt-Amps below) by 1000

Corrected Power Factor

Secondary Rating         Volt-Amps

kV,mA

15,120                 900

15,60                  450

15,30                  250

12,120                 775

12,60                  400

12,30                  200

9,120                 600

9,60                  300

9,30                  150

7.5,120                 500

7.5,60                  250

7.5,30                  125

6,60                  200

6,30                  100

Using a pair of rebuilt 12 kv, 60 ma neons, with 2 shunting

plates removed from the core next to each HV winding, and power

factor correction capacitance, you can get a nice 1.5 KVA Tesla

power supply with over 90% efficiency. Total cost: $5.00 for the

pfc capacitors, and a few hours of time.

I have unpotted dozens of neon transformers from many different

manufacturers. I have tried to make this as informative as

possible, and have checked it over for mistakes. If I have erred,

or was not clear on something, please let me know. Use common

sense whatever you do, and don't expect the first attempt to work

out. On my first attempt I managed to destroy a HV winding during

the unpotting, as I did not know where the windings were located

on the core. But once you see one core unpotted, with minor

differences, you have seen them all.

******FREQUENTLY**ASKED**QUESTIONS*???***************************

> I'm unpotting a neon transformer the way you described. I have

> basically all the tar off except for some dirt that I didn't

> clean off yet. I have a few questions. 1) what are the shunts

> and what are they made of? 2) Is the core the heavy metal band

> that surrounds the windings?

I am going to use your ASCII art as a start for the following

series of diagrams. The simplest, bare (no windings) core is

depicted below. It is made of stamped sections of flat iron

sheet that are coated with a thin layer of enamel. This laminated

core is made to come apart into two "U" or  "L" shaped sections

which is how the pre-wound primary and secondary windings are

fitted on to it. Usually where will be a couple of long screws

w/nuts or a couple of rivets that may be drilled out to separate

the iron core into two open sections. The diagram below shows an

assembled neon core in it's simplest form, that is, without

windings or magnetic leakage shunts.

_____________________________________________

|                                             |

|                                             |

|     -----------------------------------     |

|     |                                 |     |

|     |     OPEN RECTANGLE IN CORE      |     |

|     |                                 |     |

|     -----------------------------------     |

|            STAMPED IRON SHEET               |

|_____________________________________________|

The diagram below shows the simplest core above, but with

the addition of the primary and secondary windings. The

primary winding is in the center, the two secondary windings

are towards the outside. There are two wires leading from the

120 volt primary winding, and one wire each from the outer layer

of the high voltage secondary windings. This gives a total of

four lead wires exiting the transformer. The base wires of the

two secondary windings are grounded to the core laminate.

120 VOLTS

high voltage    v   v     high voltage

__I__        I   I        __I__

|  S  |     __I___I__     |  S  |

|  E  |    |    P    |    |  E  |

_______|  C  |____|    R    |____|  C  |______

|      |  O  |    |    I    |    |  O  |      |

|      |  N  |    |    M    |    |  N  |      |

|     -|  D  |----|    A    |----|  D  |-     |

|     ||  A  |    |    R    |    |  A  ||     |

|     ||  R  |    |    Y    |    |  R  ||     |

|     ||__Y__|     ---------     |__Y__||     |

|     -----------------------------------     |

|             GROUNDED IRON CORE              |

|_____________________________________________|

Now, in order to limit the amount of energy, in the form of

magnetic field flux, that is exchanged between the primary

winding and the two secondary windings: magnetic leakage shunts

are placed in the core on either side of the primary winding.

These shunts are nothing more than small blocks of the same

core laminate that makes up the rest core. The diagram below

shows the simplest neon core from the first diagram, without

windings, but now showing the correct placement for the mag-

netic leakage shunts:

_____________________________________________

|                                             |

|                                             |

|     -----------------------------------     |

|     |     |  |               |  |     |     |

|     |     |  |  < SHUNTS >   |  |     |     |

|     |     |  |               |  |     |     |

|     -----------------------------------     |

|                                             |

|_____________________________________________|

> 3) You say to take softened polethylene sheet and press it

> against the core and then take the windings and press them down

> on it. How would the windings pressed down on the sheet when

> the core is around the windings?

The windings are around the core. When the two pinch bolts (or

rivets) holding the core together are removed, the core may be

disassembled into two sections. Once the core is apart, the

windings may be slipped on and off. To prevent vibration, the

windings are usually wedged into place. Before reassembly of the

core it may be warmed; polyethylene carefully wrapped around the

laminate; and the windings gently but firmly pressed into place.

*****************************************************************

Quoting ed@alumni.caltech.edu (Edward V. Phillips):

Ed> Richard:

> As comprehensive as your writings on neon sign transformers

> have been, there are some points you have either not covered

> or which I have missed. A number of years ago my carpool

> buddy and I were driving home from work and noticed several

> neon sign transformers lying in the iceplant next to a

> freeway on ramp.  We came back to investigate and found a

> half dozen FRANCEFORMER 12 kV, 60 ma power factor corrected

> transformers, which we split down the middle.   There was

> also a 9 kV, 60 ma transformer which I got.  To make a long

> story short, all of these transformers, which had obviously

> fallen off the back of a truck as it went round a curve, had

> more or less damage to the insulators.  I have been able to

> use two of my 12 kV transformers, but am now interested in

> rehabilitating the third, if possible.  Here is the problem:

> One insulator is completely broken and gone, together with

> the stud which ran through it.  When I look into the empty

> hole I see tar with no wire visible.  Questions:

> 1. Is it likely that when the stud broke off it pulled the

> attached lead out of the transformer so that winding is now

> NG?

It is very unlikely the windings are damaged or destroyed. This

same thing frequently happens in the course of a normal un-

potting. With a little bit of experience in unpotting a

particular make and model of transformer, the builder learns the

location and orientation of the spring steel retaining clip used

to hold the HV bushing (or insulator) in place. With this

knowledge it is fairly easy to dig a small hole in the asphalt

with a screwdriver, pop the retaining clip out of the potting,

and remove the insulator without damaging the porcelain or the

lead wire. Without this little bit of key knowledge, the builder

usually ends up cracking the insulators off with a hammer, and

the thin HV lead wire is frequently cut, sheared, or pulled out

of the potting. All is not lost, in fact the setback is minor.

> 2. If there is hope of salvaging the transformer, I am left

> with the alternative of peeling off the case, as you recommend,

> or attempting to melt out the tar so that I can at least get

> access to the winding whose lead no longer gets through to the

> outside world.  I kind of favor melting out the tar, as I would

> like to reuse the case.  Questions with regard to that are:

> 3. What is wrong with melting out the tar?  In some ways it is

> less messy than chiseling off the case and the tar.

As you already suspect there is more than one way to "skin this

cat". Regardless of the particular method actually used to

physically remove the asphalt potting (it is not really tar), I

still advise removal of the case before doing anything. Melting

out the asphalt potting without removal of the case means removal

of the lid and placing the case either on it's side or in an

inverted position in a pan, on a rack, etc., while heating. When

the asphalt begins to flow out of the case it exposes the

delicate windings, then the entire core will suddenly drop out or

roll sideways in the case and damage the exposed windings.

On the other hand, you can split the corners of the case clean

down with a hammer and chisel, beat the asphalt block with a

small sledge even if you desire, and the windings are safely

protected by the potting. Once the block is freed it may be

placed in a pan and heated, and the block will settle gently on

the windings where they will not be damaged.

> 4. Are the windings attached to the case, or is the transformer

> just sitting in the tar?

The entire core is really just sitting on the bottom of the case.

There is a small steel strap which electrically connects the core

to the case at the same location where the grounding lug is

screwed in. Once the grounding lug is removed the only physical

connection that exists between the core and the case is through

the lead wires. Once the potting is removed, the case is just

useless junk. It serves no purpose other than to hold the

potting.

> I am thinking of putting the whole thing in an oven I have,

> and letting the tar melt into a pan.  If this resulted in the

> transformer falling out of the case and ripping off the

> remaining leads the whole operation would be a big and very

> messy waste of time.

> Any comments or suggestions?

Go ahead and bust the insulators to dust with a hammer, then

chisel the case off by splitting the welds/corners from the top

down to the base. If the asphalt block still does not come free

then pry the base of the case away with a hammer and screwdriver.

Don't spare the force and damn the lead wires. Just watch the

fingers and hands, I have gashed myself more than once doing

this. Heavy gloves would be advisable. Someone mentioned using a

cutting wheel on a dremmel tool, I have used air chisels,

whatever works.

Toss the block into a pan and heat at 375 - 400 degrees until the

asphalt begins to soften and run (about 30-40 minutes), then

reduce the temp below 300 or so and cook carefully until done.

There are spliced connections where the windings are wired to the

lead wires. These splices are well protected as they are located

deep in the potting. Once the core is freed from the potting the

old lead wires can be easily replaced.

The reason I am not really enthusiastic about melting the asphalt

out is that the melting temperature of the potting and the

smoking/burning temperature of the transformer fall in a very

narrow range. The temperature range varies from core to core

depending on the age and quality of that particular potting. Some

cores with nylon wedges pinning the windings in place can become

a molten plastic mess if the nylon melts, mixes with asphalt,

then later cools on the sides of the HV windings. The clean-up

requires hours with a dental pick.

By far the cleanest and simplest way to unpott cores bedded in

asphalt is to soak the core (sans case) in a medium sized (not

kitchen size) rectanglar household polyethylene trash can. One

gallon of diesel, kero, or gas will remove all the potting in two

- three days if the solvent is kept at 40 - 50 degrees (min) in a

garage or outside porch (it is winter now, summer is faster). The

unpotted core is then split, the windings removed, and the clean

up can be finished with a few minutes of scrubbing with an old

toothbrush and a cup of clean solvent. The waste can be then be

poured into a couple of one gallon plastic milk containers which

will hold up long enough to take it to the gas station, oil

change station, muni recycling center etc., where it can be

safely disposed of in the waste oil sump.  The windings will dry

thoroughly after about a 7 - 10 days laying out on some clean

newspaper. Turn them every day or two.

Richard Quick

Quoting Ephraim Nussbaum <nussbaum@silly.com>:

> I still can't figure out how to remove the windings. The

> transformer was made by Jefferson Electric; did you by any

> chance unpot one of these? The side view looks like this

__----------------------------------__  |

|  /                                   \ |

| /                                     \|

|/______________________________________\|_

|   |  |          |  |           |  |    |

|   |__|          |__|           |__|    |

|________________________________________|_

> Around that inner core is the windings going through the

> holes(looks like a house).

When you get this figured out you will most likely want to whack

your head a few times with a flat board :-)

The cores are assembled from two mating sections, much like a

tongue and groove construction used in furniture making. Where

the two sections are joined they are drilled through, from the

sides, and either a screw w/nut or a rivet is used for a fastener

to prevent the two sections from coming apart.

I am going to attempt to diagram this with ASCII art, but I am

not an artist, please bear with me:

_____________________________________________

|                                             |

|                                             |

|     -----------------------------------     |

|     |                                 |     |

|     |     OPEN RECTANGLE IN CORE      |     |

|     |                                 |     |

|     -----------------------------------     |

|            STAMPED IRON SHEET               |

|_____________________________________________|

This diagram shows the assembled core without shunts or windings.

If you look carefully, there will be a fastener that is exposed

on the side of the core, even with the shunt plates and windings

installed. Look carefully.

_____________________________________________

|                                             |

|  O            O           O             O   |

|     -----------------------------------     |

|     |                                 |     |

|     |     OPEN RECTANGLE IN CORE      |     |

|     |                                 |     |

|     -----------------------------------     |

|  O            O           O             O   |

|_____________________________________________|

The "O"s above mark _probable_ locations for the fasteners that

pin the two sections of the cores together. These locations would

be in the corners, or between the HV windings and the primary. Of

these locations only two holes will be actually drilled. My guess

would be that rivets are used through the holes in the particular

core you are looking at because I think you would have noticed

screws and nuts by now.

_____________________________________________

|                                             |

|  O <--Rivet Hole                            |

|     -----------------------------------     |

|     |                                 |     |

|     |     OPEN RECTANGLE IN CORE      |     |

|     |                                 |     |

|     -----------------------------------     |

|                          Rivet Hole-->  O   |

|_____________________________________________|

When you locate the fasteners, remove them. If the fastener is a

rivet you will have to block the core carefully with some pieces

of wood and drill or grind the heads off before driving the

shafts out with a nail or small punch. Then separate the two

sections:

____________________________________________

-----   |     |                                      |

|  O  |  | O   |                                      |

|-----|   ---------------------------------------     |

|     |                                         |     |

|     |                                         |     |

|     |                                         |     |

|     ---------------------------------------   |-----|

|                                       | O  |  |  O  |

|_______________________________________|____|   -----

Once the core is apart, you can see how the windings are

removed/installed. The windings simply slide right off of the

ends of the "bars" of the disassembled core. I have also gone

ahead and detailed where the tongue and grooved sections of the

core are located; directly where the fasteners are installed.

I hope this helps, if you are still having difficulties with this

I can recommend a video tape produced Richard Hull that details

the overhaul/rebuild of a neon transformer. Explain your

difficulties in a letter to Richard Hull (he does not have an E-

Mail address) and he will be happy to write you back with the

information about one of his video tapes that details the

rebuilding of a neon sign transformer from beginning to end.

You can contact Richard Hull at:

RICHARD HULL

7103 HERMITAGE ROAD

RICHMOND  VA  23228

His informational and educational videos are extremely detailed.

Though I have not personally seen his video on rebuilding neons,

it is my understanding that Mr. Hull has thoroughly covered

unpotting, disassembly, rebuilding, and testing of neon cores on

a two-hour SP recorded video tape. Given your difficulties, the

purchase of the video from Mr. Hull may be a wise investment.

Quoting Chip Atkinson:

_____________________________________________

|                                             |

|                                             |

|     -----------------------------------     |

|     |                                 |     |

|     |     OPEN RECTANGLE IN CORE      |     |

|     |                                 |     |

|     -----------------------------------     |

|            STAMPED IRON SHEET               |

|_____________________________________________|

> I believe that Jefferson Electric transformers are slightly

> different from RQ's drawings in that the coils are mounted on a

> bar that slides into the core from the side like so:

_____________________________________________

|       /                            \        |

|      | x                           x|       |

|     --\----------------------------/--      |

|     |                                 |     |

|     |     OPEN RECTANGLE IN CORE      |     |

|     |                                 |     |

|     -----------------------------------     |

|            STAMPED IRON SHEET               |

|_____________________________________________|

> You will have to free the core completely from the box and

> drive the bar out with a punch at the x's.  Be patient and

> careful of fingers and coils.  I left the core out in the cold

> in the winter and used a chisel to chip off the remaining

> asphalt when it was brittle as glass.   Chip

Richard Quick

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Bert Hickman <bert.hickman@aquila.com>

Date: Tue, 18 Mar 1997 06:55:41 -0800

Subject: Neon Repair (Part 2 - T. Chandler)

Organization: Stoneridge Engineering

Here's the second part of the neon repair series. This one's by Tim

Chandler and describes some non-standard core and coil configurations.

Safe unpottin' to ya!

-- Bert --

Requoted from Tim Chandler:

========================================================================

Hi All,

Well its the begining of a new year and pretty much snowed in, so

decided to unpot some transformers I had collected over the past year.

Upon doing so I ran into some rather interesting designs, thought I

would share with any who were interested.  Here it goes...

------------------------------------------------------------------------

First of all when I unpot transformers, like RQ, I do 3 or more at a

time.  I also have a tendency of doing all the transformers from a

particular manufactorer at the same time.  Well this time I have 7

transformers of the 30mA range from Jefferson Electric (MagneTek).

There are rated as follows:

(i)   5000V@30mA

(ii)  6000V@30mA

(iii) 7500V@30mA

(iv)  9000V@30mA

(v)   12000V@30mA

(vi)  12000V@30mA

I unpotted the above transformers using RQ's cold-cook method, I then

cleaned the core and windings as best as possible with kerosene.  Then I

proceeded to disassemble the core and windings, PROBLEM.  The problem is

that in all the transformers, with the exception of the 9000V@30mA, the

core had no bolts, rivets, or pins holding it together.  Also I noticed

it had the abnormal configuration, which I had before in other

transformers:

+----------------------------------------------------------------------+

|                                                                     |

|

+------------------------------------------------------------------|*

|   |                                                             |   |

|   |                                                             |   |

|   |                                                             |   |

*|-----------------------------------------------------------------+   |

|                                                                     |

*|-----------------------------------------------------------------+   |

|   |                                                             |   |

|   |                                                             |   |

|   |                                                             |   |

|

------------------------------------------------------------------|*

|                                                                     |

----------------------------------------------------------------------+

Figure 1:  Stamped Core

--------

+----------------------------------------------------------------------+

|                                                                    |

|------------|---|-----------------------|---------------------------|*

|   |        |///|                       |                       |   |

|   |        |///|                       |                       |   |

|   |        |///|                       |                       |   |

*|---|        |///|                       |                       |   |

|   |  P1    |///|          S1           |          S2           |   |

*|---|        |///|                       |                       |   |

|   |        |///|                       |                       |   |

|   |        |///|                       |                       |   |

|   |        |///|                       |                       |   |

|------------|---|-----------------------|---------------------------|*

|                                                                    |

+----------------------------------------------------------------------+

Figure 2:  Core shown with Windings

---------

P1:  Primary Winding (input)

S1:  Secondary Winding 1 (output)

S2:  Secondary Winding 2 (output)

NOTE:  The area between P1 and S1 (///) are the shunts, which can be

removed to increase current, but REMOVE NO MORE THAN 2,

otherwise the TRANSFORMER WILL BURN UP, I say this from

experience...

In both figures above there are "*"'s at certain points on side of the

core.  These are the problem.  They are what holds this 3 piece core

together. At first I thought maybe it was a high temp solder, wishful

thinking, but instead it is a strip-weld.  I contacted Jefferson

Electric in New York and talked to one of the guys there and he

confirmed this.

Now I use three means to remove these strip-weld(s):

(1)  If you have some type of welder, then just unweld it (SOME

WHAT        EASY)

(2)  Another way is to cut through the strip-weld with a

hack-saw             (EASIER SAID THEN DONE)

(3)  My preferred method is using a Dremel Tool with a metal

cutting/grinding tool on it, cuts through it like hot butter

(SIMPLE)

Now you can take the core apart, in 3 peices, and remove the windings.

IMPORTANT NOTE, WITH MOST TRANSFORMERS OF THE ABOVE CONFIGURATION (both

secondary windings on the same side of the primary) IT IS IMPORTANT TO

MAKE SURE THE WINDINGS GO BACK ON THE SAME WAY YOU TOOK THEM OFF. (the

transformer may not work properly otherwise and might burn up)...also

from experience.

I usually remove the windings from the core, use kerosene to remove the

remainder of the potting compound, and then let dry for a few days.

Then I rebuild/rewire the windings and recondition the core if necessary

(as to RQ spec's).  Now they are ready to go back together.  But how

will you reconnect the core???

+-----------------------------------------------------------+

|                                                           |

|   +-------------------------------------------------------+  +---+

|   |                                                          |   |

|   |                                                          |   |

+---+  +---+---------------------------------------------------+   |

|                                                           |

+---+  +---+---------------------------------------------------+   |

|   |                                                          |   |

|   |                                                          |   |

|   +-------------------------------------------------------+  +---+

|                                                           |

+-----------------------------------------------------------+

left                                                              right

Top View

--------

Welding this back together would be the best thing to do, but I

frequently disassemble the transformers and it becomes quite bothersome,

plus most people don't have access to a welder...

I have used several methods to fasten the core assembly back together,

but the most reliable I have found so far is as follows:

(1)Find a material suitable a as support.  I generally use very thin

plexi-glass (1/16") or aluminum flashing wrapped in electrical

tape. Plexi-glass is generally better if you have it.

(2)Cut the material for the supports.  You will need 3 for the top

and 3 for the bottom.  My design is usually like this:

+---+                     +---+

| o |                     | o |

|   |                     |   |

|   |                     | o |

| o |                     +---+

|   |

|   |                    Brace B

| o |

+---+

Brace A

You will need 2 of Brace A type and 4 of Brace B type.

NOTE:Brace A should be as long as the core is wide (NOT THICK!!)

(3) Next you need to find bolts that will be long enough to go throught

the core yet are relativily small in diameter, you don't want to

drill a large hole in the core.  After you have 6 bolts w/nuts and

washers select the proper sized drill bit and get ready to drill

holes in the core.  I use long skinny bolts (5"L x 1/16"D) that

I get at a local motor repair shop but I think (not sure) that

Home Depot carries them as well.  You don't need the bolts that

long, I use them that long because when I am finished connecting

everthing back together, assuming everthing is still uniform, the longer

bolts create a nice little stand to support the transformer, and with a

little work you can secure it to just about anything.

4) Drilling is the hard part, unless you have a drill press.  Support

Brace A is for the top & bottom of the core's left side (as

labeled in the above core diagram) and Support Brace B is for the top &

bottom of the core's right side corners.  It should be fairly obvious

where they go.  If you luck out and do it correctly you only have to

drill 3 holes because there are usually holes drilled in all 4 corners

(but not always).  All the drilling is usually done on the center or

T-shaped core piece.  Place the core together (WITHOUT THE WINDINGS,

MIGHT DAMAGE THEM WHEN DRILLING!!!) and mark where the Support Braces

go and where the holes need to be drilled. Take it apart and drill

the holes.

NOTE:  If the core on your transformer has the 4 pre-drilled holes

in the corners already, by all means use them (its alot less

less work).  Remember though, those holes are generally

going to be alot bigger in diameter than the holes you will

drill so do not use the small diameter bolts.  I suggest

finding a bolt with the exact (or as close as possible)

diameter as the hole, if you use the smaller diameter

bolts, the stability is decreased dramatically.

(5)  Reassemble the core and the windings, attach the Support Braces,

tighten the bolts and thats it, the transformer is back together.

Make sure you check the windings and make sure they are not lose

otherwise they may rattle when the transformer is in use.

Although this procedure is not exactly easy and alittle time consuming,

it is for the most part worth it, the transformer last longer and

operates better when reconditioned and set up to fire-dry.  I don't have

as much experience and time in reconditioning neon transformers as most

on this listserv do but I thought I would share my method with everyone.

If any one has a better method of doing any of the above, by all means

please let me know.

I am getting ready to start unpotting my Jefferson Electric (MagneTek)

60mA and 120mA next, then I will move onto the FRACEformers, if I notice

any differences or experience any problems I will do another write up on

it, assuming I can find

a solution to the problem...:)!

Happy Coiling,

Tim

+---------------------------------++---------------------------------+

] Timothy Alan Chandler           []   M.S.Physics/B.S.Chmeistry     [

+---------------------------------++---------------------------------+

| NASA-Langley Reasearch Center   /\   George Mason University       |

| Department of Energy            ||   Department of Physics         |

| Fusion Research Team- Alpha     ||   Department of Chemistry       |

| CHOCT Fusion Reactor Design     \/   OPC-EFC                       |

+---------------------------------++---------------------------------+

)            Private Email Address:  tchand@slip.net                 (

+--------------------------------------------------------------------+

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Wed, 19 Mar 1997 10:44:51 -0600

Subject: Re: Reciver

At 12:06 PM 3/19/97 GMT, you wrote:

>From: "Chad Westbrook" <emu@clarityconnect.com>

>Date: Wed, 19 Mar 1997 07:06:06 -0500

>Subject: Reciver

>

>What is a tesla coil receiver can you make one?????

>

This is essentially a free standing secondary coil tuned

to the same frequency as your operating coil.

If your operating coil has a large toroid that prevents breakout

it becomes a RF transmitter. The RF ground currents

become very heavy so a good RF ground is important.

The free standing coil which is also connected to ground

will breakout with sparks while the other coil is running provided

its tuned to the same frequency.

TTYL!

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Wed, 19 Mar 1997 10:44:49 -0600

Subject: Re: second try

At 04:04 PM 3/19/97 GMT, you wrote:

>From: rpittman@juno.com

>Date: Wed, 19 Mar 1997 08:04:29 PST

>Subject: second try

>

>Mike and all,

>        Well I finally got another chance to play with the coil a little

>more yesterday.  I shrunk the toroid so that it is now about 4" x 15".

>After running it for a while with the pie plate I decided to try the

>toroid.   Interesting thing was I could get a much hotter arc to a

>grounded object near by but I could still not get it to spark off into

>air.  I measured the distance that I could draw a continuous white arc

>at about 8.5 ".  I got rid of the slow starting problem by placing a

>jumper cable on the spark gap to loose one gap.

That toroid is probably still a little large for the power level

you are running. Still 8.5 inches of spark for only 180VA isn't

too bad. Try placing a thumbtack on the edge of the toroid. Or place

a piece of wire sticking out over the edge. This will provide

a sharp point to spark from.

>        Right now I am running 12 salt watter snapple bottles in

>parrallel and another 6 connected to that bank in series.  I also have a

>very small 6kv 30 ma neon, and 983 on a 4.27" o.d. pvc drain pipe.  I

>tried adding another 6 bottles in both series and parrallel but the

>spark decreased (I did not have time to move the tap at all which is on

>turn 11 of a 14 turn 10 ga helical primary.)

>        Any suggestions?  Could I readjust the tap with the extra cap to

>get a longer spark?

When you change the amount of capacitance in the primary circuit

you are changing the resonant frequency. You then MUST change

the tap position on the primary to retune to the original frequency.

Remember the primary circuit must be tuned to the same frequency

as the self resonant frequency of the secondary.

> Interesting thing happened also if I did not put a

>grounded object near the toroid, the top end of the primary had major

>corona loss, when there was a grounded object it sparked over fine. why?

If you are having lots of corona and sparking from the primary then you

are overcoupled. You need to raise up your secondary a bit away from the

primary to reduce the coupling. Try moving your secondary up about an

inch and see if that helps. You want to get the coupling as high as

possible without corona or sparking from the primary.

>Once again I really appreciate all the great help I have recieved.

>Adios

>Ronnie

>

TTYL!

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: "Chad Westbrook" <emu@clarityconnect.com>

Date: Wed, 19 Mar 1997 15:21:43 -0500

Subject: Avoid a bad misteak

If you touched the toroid of a small coil would it hurt or kill???

How does that guy get the sparks to come out of his fingers is it safe to

try??

Do you need any special equipment???

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Maciszonek Artur <ummacius@cc.UManitoba.CA>

Date: Wed, 19 Mar 1997 16:47:26 -0600 (CST)

Subject: Capacitor construction

Hi all,

Does anyone know of a good website or document which will outline the

constuction of a home-brew capacitor?

How would you measure it's capacitance? I know the easiest way to find out

is to measure it after you have built it, besides that is there a formula I

can use?

I have not found a transformer yet so I cannot provide any

specifications on my coil yet, so I don't know what value of capacitor I need.

But I am still interested in how to make it.

Thanks in advance .

^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^^

"Ignorance is not an excuse for stupidity...." M.A.

Maciuszonek Artur

e-mail: ummacius@ccu.umanitoba.ca

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Wed, 19 Mar 1997 16:50:46 -0600

Subject: Re: Avoid a bad mistake

At 08:21 PM 3/19/97 GMT, you wrote:

>From: "Chad Westbrook" <emu@clarityconnect.com>

>Date: Wed, 19 Mar 1997 15:21:43 -0500

>Subject: Avoid a bad misteak

>

>If you touched the toroid of a small coil would it hurt or kill???

>

>How dose that guy get the sparks to come out of his fingers is it safe to

>try??

>

>Do you need any special equipment???

>

Yea it hurts like hell! For the most part it will not kill

you being RF. Rf flows along the outside of conductors (your body)

due to skin effect.

But streamers often contain a 60 hz component that can kill.

Consider that the primary coil is alive with 60hz high voltage.

Often the primary will take hits from the secondary.

If your body is attached to a streamer when another contacts

the primary coil you are then essentially connected to the

primary through the arc channel.

A stage performer by the name of Henry Transtrom often took

streamers to his body during his electrical shows. He was

finally killed doing so during his "last" performance.

If you insist on doing this place yourself on a insulated

platform. Don't even think of doing it standing in your sneakers

on the basement floor.

Live long and prosper!

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Nathan_Bonsal@intuit.com (Nathan Bonsal)

Date: Wed, 19 Mar 1997 16:46:33 -0800

Subject: Re: Avoid a bad misteak

I wouldn't advise touching the toroid at all unless you are ABSOLUTELY

sure that the coil is tuned properly. I've heard the white sparks can

kill you easily, and don't forget that an arc to you from the

secondary could be indirectly connected to the primary through the

corona or secondary arcs.

Don't do it, man!

______________________________ Reply Separator ________________________________

_

Subject: Avoid a bad misteak

Author:  tesla-2@emachine.com (Tesla-2) at Internet

Date:    3/19/97 8:21 PM

From: "Chad Westbrook" <emu@clarityconnect.com>

Date: Wed, 19 Mar 1997 15:21:43 -0500

Subject: Avoid a bad misteak

If you touched the toroid of a small coil would it hurt or kill???

How dose that guy get the sparks to come out of his fingers is it safe to

try??

Do you need any special equipment???

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Wed, 19 Mar 1997 19:02:20 -0600

Subject: Re: Capacitor construction

At 10:47 PM 3/19/97 GMT, you wrote:

>From: Maciszonek Artur <ummacius@cc.UManitoba.CA>

>Date: Wed, 19 Mar 1997 16:47:26 -0600 (CST)

>Subject: Capacitor construction

>

>

>Hi all,

>Does anyone know of a good website or document which will outline the

>constuction of a home-brew capacitor?

Kevin Nardelle's site has both the Richard Hull rolled cap file

and Gary Weavers adaptation of the Hull cap.

Kevin's URL is:

http://www.icorp.net/users/kev/tesla

>

>How would you measure it's capacitence? I know the easiest way to find out

>is to measure it after you have built it, besides that is there a formula I

>can use?

Formula for Plate Type Capacitors

Capacitance (in microfarads) = (0.224 KA / d) (N-1)

0.224 x Dielectric Constant x Area of plate

C = -----------------------------------------  x (Number of plates - 1)

distance between plates in inches

NOTE: For two plate rolled capacitors the value derived from this

formula must be multiplied by two, or, the value must be computed

with the number of plates (N) equal to three (3), so (N-1 = 2)

The dielectric constant (K) for polyethylene is 2.

TTYL!

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

T>>If you touched the toroid of a small coil would it hurt or kill???

T>Yea it hurts like hell! For the most part it will not kill

T>you being RF. Rf flows along the outside of conductors (your body)

T>due to skin effect.

T>But streamers often contain a 60 hz component that can kill.

T>Consider that the primary coil is alive with 60hz high voltage.

T>Often the primary will take hits from the secondary.

T>If your body is attached to a streamer when another contacts

T>the primary coil you are then essentially connected to the

T>primary through the arc channel.

Reminds me of the day looong ago when I was a young'un... (immortal,

and all that goes with being a teenager...)

I was demonstrating my coil, when on the spur of the moment, I

demonstrated how you could take a big spark from the secondary of a

Tesla coil to your hand and remain alive. <G>  The result was something

I have never forgotten.  It took a couple of moments for me to get my

eyeballs pointed in the same direction, and all my limbs to function

properly.

My audience said afterwards that my demonstration was quite stunning.

Believe me, I QUITE agreed!!

Ralph

---

Famous Last Words: "Watch me goose that sleeping dragon."

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Bert Hickman <bert.hickman@aquila.com>

Date: Wed, 19 Mar 1997 23:26:31 -0800

Subject: Re: Avoid a bad mistake

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> T>>If you touched the toroid of a small coil would it hurt or kill???

>

> T>Yea it hurts like hell! For the most part it will not kill

> T>you being RF. Rf flows along the outside of conductors (your body)

> T>due to skin effect.

>

> T>But streamers often contain a 60 hz component that can kill.

> T>Consider that the primary coil is alive with 60hz high voltage.

> T>Often the primary will take hits from the secondary.

>

> T>If your body is attached to a streamer when another contacts

> T>the primary coil you are then essentially connected to the

> T>primary through the arc channel.

>

>    Reminds me of the day looong ago when I was a young'un... (immortal,

> and all that goes with being a teenager...)

>

>    I was demonstrating my coil, when on the spur of the moment, I

> demonstrated how you could take a big spark from the secondary of a

> Tesla coil to your hand and remain alive. <G>  The result was something

> I have never forgotten.  It took a couple of moments for me to get my

> eyeballs pointed in the same direction, and all my limbs to function

> properly.

>

>    My audience said afterwards that my demonstration was quite stunning.

> Believe me, I QUITE agreed!!

>

>    Ralph

> ---

>   Famous Last Words: "Watch me goose that sleeping dragon."

>

All,

When you add a toroid, you're actually adding lumped capacitance to the

top of your coil. Whenever you charge a capacitor to a high voltage, and

then attempt to take the full discharge current through your body,

you're risking your life! The fact that the toroid voltage is actually

oscillating at an RF frequency IS NOT RELAVENT to the fact that, at each

voltage peak, the sudden surge that will "zap" you will look _just_ like

a capacitor charged to a high DC potential.

A moderately sized coil can deliver very large peak currents during

heavy ground arc discharges, amounting to many hundreds of amps. These

ground discharges are hot, bright, loud, and potentially lethal! Don't

make yourself part of this circuit if you wish to continue coiling!

SAFE coiling to you!

-- Bert --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: "Chad Westbrook" <emu@clarityconnect.com>

Date: Thu, 20 Mar 1997 06:59:06 -0500

Subject: To Do or Not To Do

My cousin is in a electronics course in schoool and they are going to build

a receiver so when you stand on the booth you get plasma all over your body

How are they doing that???

He also told me that Tesla stood in such a booth and then came out and

flung his arm at a women with a white dress and you could see little plasma

balls on her

That is cool

What is the safe frequency and other things to make this so you come out of

the booth yer still alive????

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Jochen Kronjaeger <Kronjaeg@Stud-Mailer.Uni-Marburg.DE>

Date: Thu, 20 Mar 1997 23:12:55 +0100 (CET)

Subject: Re: will this sec. work?

> >Questions: Will this work? Are the figures right?

>

>

> Your .3mm wire is fairly close to AWG 28 and thats a bit small for this size

> of coil. A better secondary will result from using AWG 24 which is about .51mm.

wire gauge is fixed, i'm afraid, as I have this wire :-( Anyway, for a

first try it's o.k., I guess.

> >           How important is it to match the lambda/4 resonating wavelength

> >           (lamda/4 = total wire length = 330m here => 227kHz)?

> >           I only read about this, what's your experience?

>

> Not at all important. I have no idea what the length of wire in my

> secondaries is. I it doesn't really matter. What is important is that

> your primary circuit resonate at the same frequency as your secondary.

This should be rather easy. Only one thing to adjust. To match the wire

length, I'd had to adjust the toroid too, which didn't seem easy to me :-)

Kind of inflatable toroid.

>

> >

> >           How do I make a toroid (easiest :-)? Does the surface have to

> >           be metal, or can I use graphite spray? What about aluminium air

> >           ducts?

>

> Should be metal.

> Flexible aluminum ducting works well wrapped around two back to back pie pans

.

I'll try this next time I come home. Thanks so far!

Jochen

+---------------------------------------------------------------------------+

| Jochen Kronjaeger                     Kronjaeg@stud-mailer.uni-marburg.de |

|                                                                           |

| The High Voltage Page  http://www.mathematik.uni-marburg.de/~kronjaeg/hv/ |

|   info, plans, photos of self-built HV-sources, sparks, HV-experiments    |

+---------------------------------------------------------------------------+

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Thu, 20 Mar 1997 18:28:55 -0600

Subject: Re: will this sec. work?

At 10:12 PM 3/20/97 GMT, you wrote:

>From: Jochen Kronjaeger <Kronjaeg@Stud-Mailer.Uni-Marburg.DE>

>Date: Thu, 20 Mar 1997 23:12:55 +0100 (CET)

>Subject: Re: will this sec. work?

>

>> >Questions: Will this work? Are the figures right?

>>

>>

>> Your .3mm wire is fairly close to AWG 28 and thats a bit small for this size

>> of coil. A better secondary will result from using AWG 24 which is about

.51mm.

>wire gauge is fixed, i'm afraid, as I have this wire :-( Anyway, for a

>first try it's o.k., I guess.

Ok if you must use that guage of wire I suggest a smaller diameter secondary.

3 inch PVC in the US is 3 1/2 inches OD. Thats about 89 mm.

Make a winding about 13 inches long.(about 33cm)  That will be about 925

turns or so.

Resonant frequency about 516 KHZ. A small toroid should bring that down to

350 khz or so.

I made a small coil wound on small 2 inch (51mm) PVC pipe. I used AWG 32

wire and wound a winding 6 inches long. (15.2 cm)

I got 12 inch long sparks out of it that were twice as long as the secondary

coil.

But it would not handle any more than 270 VA of power before breaking down.

You will get the best results using the largest guage of wire that will

allow for 700-900 turns in the available space.

TTYL!

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Jochen Kronjaeger <Kronjaeg@Stud-Mailer.Uni-Marburg.DE>

Date: Fri, 21 Mar 1997 23:09:36 +0100 (CET)

Subject: Re: will this sec. work?

> From: Mike Hammer <mhammer@misslink.net>

> Date: Thu, 20 Mar 1997 18:28:55 -0600

> Subject: Re: will this sec. work?

>

> Ok if you must use that guage of wire I suggest a smaller diameter secondary.

> 3 inch PVC in the US is 3 1/2 inches OD. Thats about 89 mm.

O.K., I'll try that. Not sure whether I'll find a suitable pipe. What are

the pipes you use made for originally? Sewage?

By the way, what's the polyurethane varnish someone mentioned usually made

for (i.e. in what kind of shop can I find it)?

Jochen

+---------------------------------------------------------------------------+

| Jochen Kronjaeger                     Kronjaeg@stud-mailer.uni-marburg.de |

|                                                                           |

| The High Voltage Page  http://www.mathematik.uni-marburg.de/~kronjaeg/hv/ |

|   info, plans, photos of self-built HV-sources, sparks, HV-experiments    |

+---------------------------------------------------------------------------+

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Fri, 21 Mar 1997 19:06:56 -0600

Subject: Re: will this sec. work?

At 10:09 PM 3/21/97 GMT, you wrote:

>From: Jochen Kronjaeger <Kronjaeg@Stud-Mailer.Uni-Marburg.DE>

>Date: Fri, 21 Mar 1997 23:09:36 +0100 (CET)

>Subject: Re: will this sec. work?

>

>

>> From: Mike Hammer <mhammer@misslink.net>

>> Date: Thu, 20 Mar 1997 18:28:55 -0600

>> Subject: Re: will this sec. work?

>>

>> Ok if you must use that guage of wire I suggest a smaller diameter secondary

.

>> 3 inch PVC in the US is 3 1/2 inches OD. Thats about 89 mm.

>O.K., I'll try that. Not sure whether I'll find a suitable pipe. What are

>the pipes you use made for originally? Sewage?

In various parts of the US PVC pipe can be used for most indoor

plumbing. The larger PVC pipe is usually used for sewer lines and

drains.

Use only white PVC pipe. Avoid colored pipe especially black.

The black pipe has carbon in it for coloring.

>

>By the way, what's th polyurethane varnish someone mentioned usually made

>for (i.e. in what kind of shop can I find it)?

Its commonly used for floors and woodwork and furniture.

Its being replaced by latex varnishes in some areas of the US

due to environmental concerns. But the latex varnish is no good

for our intended use. Real polyurethane is a must.

TTYL!

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Ken Smith <ksmith@ihug.co.nz>

Date: Sat, 22 Mar 1997 15:56:22 -1200

Subject: Re: will this sec. work?

>>

>>By the way, what's the polyurethane varnish someone mentioned usually made

>>for (i.e. in what kind of shop can I find it)?

>

>Its commonly used for floors and woodwork and furniture.

>Its being replaced by latex varnishes in some areas of the US

>due to environmental concerns. But the latex varnish is no good

>for our intended use. Real polyurethane is a must.

>

I use the marine polyurethane single part (since I have a lot of it around

now my boat is sold <sg>)  I have some two part epoxy type poly varnish also

which is spectacular in finish and ability to lay on.  However I don't have

much and I can't find a replacement so I am saving it for the secondary

design that will be saved for posterity. Covered in that stuff - posterity

will be the word.  It's tuff stuff...

Ken

Ken Smith

Auckland

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Ken Smith <ksmith@ihug.co.nz>

Date: Sat, 22 Mar 1997 22:51:30 -1200

Subject: Re: Your message to TESLA-2

Good Evening fellow coilers,

I have a problem wrapped up in an enigma.

Without going into long and boring details.  I have been restoring my MkIV

coil by reworking the primary to a 45 degree unit rather than the straight

helix (which gave me so many problems with flashover).

Playing with the numbers in NTESLA and TESLA it was apparent that the

secondary was far to tall and skinny (2.5 x 26 inches).  I am using (I

think, 24g wire) and I get 52 turns / inch close wound).  Anyway - I chopped

10 inches off the secondary. Did it work - did it hell.  I tried every turn

on the primary and every combination of toriod / top load that I could think

of.  All I got was a pathetic buzz from the spark gap.  At once point I

thought I had blown the neon, so I put the MkV primary/secondary plate back

on the rig.  Wrummmm all was well.

At a total loss for what was happening, I put the two bits of the secondary

back together with masking tape and soldered up the wire.  This was the act

of a desperate coiler....

Wruuummmm.  Off she went, tuning nicely at 8 of 10.  Without going into

details about the new primary and things.  The only item that I can think of

is that my caps may be totally over / under valued.  I have them at .02 each

and they are in series for .01uf. Details are 45cm x 190 cm twin plate

rolled on 1.25cm polyethelyne sheet - tight rolled in mineral oil.

The MkV tuned right out of the box on the numbers using NTESLA using my

assumed cap figures.

If the caps are not the problem - I have a question.  When we use the term

HEIGHT in regards to secondary - what are we using it for.  Is it to get the

number of turns available or something else.  Likewise is that the height

above the bottom of the primary or height of wire and how is all this

affected as to wether the primary is flat / angles or a straight helix ?

Ken

Ken Smith

Weymouth

Auckland

http://homepages.ihug.co.nz/~ksmith

ksmith@ihug.co.nz

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Bert Hickman <bert.hickman@aquila.com>

Date: Sat, 22 Mar 1997 08:45:27 -0800

Subject: Re: Your message to TESLA-2

Organization: Stoneridge Engineering

Tesla-2 wrote:

>

> From: Ken Smith <ksmith@ihug.co.nz>

> Date: Sat, 22 Mar 1997 22:51:30 -1200

> Subject: Re: Your message to TESLA-2

>

> Good Evening fellow coilers,

>

>                                 I have a problem wrapped up in an enigma.

>

> Without going into long and boring details.  I have been restoring my MkIV

> coil by reworking the primary to a 45 degree unit rather than the straight

> helix (which gave me so many problems with flashover).

>

> Playing with the numbers in NTESLA and TESLA it was apparent that the

> secondary was far to tall and skinny (2.5 x 26 inches).  I am using (I

> think, 24g wire) and I get 52 turns / inch close wound).  Anyway - I chopped

> 10 inches off the secondary. Did it work - did it hell.  I tried every turn

> on the primary and every combination of toriod / top load that I could think

> of.  All I got was a pathetic buzz from the spark gap.  At once point I

> thought I had blown the neon, so I put the MkV primary/secondary plate back

> on the rig.  Wrummmm all was well.

>

> At a total loss for what was happening, I put the two bits of the secondary

> back together with masking tape and soldered up the wire.  This was the act

> of a desperate coiler....

>

> Wruuummmm.  Off she went, tuning nicely at 8 of 10.  Without going into

> details about the new primary and things.  The only item that I can think of

> is that my caps may be totally over / under valued.  I have them at .02 each

> and they are in series for .01uf. Details are 45cm x 190 cm twin plate

> rolled on 1.25cm polyethelyne sheet - tight rolled in mineral oil.

>

> The MkV tuned right out of the box on the numbers using NTESLA using my

> assumed cap figures.

>

> If the caps are not the problem - I have a question.  When we use the term

> HEIGHT in regards to secondary - what are we using it for.  Is it to get the

> number of turns available or something else.  Likewise is that the height

> above the bottom of the primary or height of wire and how is all this

> affected as to wether the primary is flat / angles or a straight helix ?

>

> Ken

>

> Ken Smith

> Weymouth

> Auckland

> http://homepages.ihug.co.nz/~ksmith

> ksmith@ihug.co.nz

>

> * Send list Replies to: Tesla-2@emachine.com

>

> * To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

>   in the body of your message to: Listserv@Emachine.com

>

> -----

> Processed with Listserv v2.93 for Wildcat v4

Ken,

Welcome to the wonderful world of coiling. You apparently have a large

enough toroid so that it won't break out when you're somewhat out of

tune. Tape a small piece of foil, nail, or other object to the side of

the toroid so that you have a point of nice, consistent, breakout. This

will let you tune the system for best operation.

It turns out that with small diameter coils, the length-height ratio

"rule of thumb" (i.e., no more that 6X H:D ratio) doesn't really doesn't

hold up very well. The reason is that you need to have relatively large

inductance in the secondary. With a small diameter coilform, there's

only one way to get this - more turns of smaller wire, or a longer

length. In your case, you started out with a secondary with about 8000

uH and chopped it down to 4800 uH. If all things stayed the same, and

you were in tune at about turn 8 before, you should be in tune at about

turn 6 or so now.

However, all things don't stay the same - by reducing primary

inductance, you've also increased the primary current when the gap

fires, which may have made your gap less efficient in quenching. If your

toroid was just barely breaking out before, the combination of mistuning

and gap performance may be just enought to stop it from breaking out

now.

Longer term, I'd suggest going to a larger 4" diameter coilform so that

you can get relatively high inductance with relatively short coil

length. For example, a 4" diameter coilform with 16" of #24 would bring

you up to almost 12000 uH. While this may require you to add a couple

more primary turns or more tank capacitance to bring it back into

optimal tune, it will make for a better running coil.

Safe coilin' to you, Ken!

-- Bert H --

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Sat, 22 Mar 1997 10:34:13 -0600

Subject: Re: Your message to TESLA-2

At 10:51 AM 3/23/97 GMT, you wrote:

>From: Ken Smith <ksmith@ihug.co.nz>

>Date: Sat, 22 Mar 1997 22:51:30 -1200

>Subject: Re: Your message to TESLA-2

>

>Good Evening fellow coilers,

>

>Playing with the numbers in NTESLA and TESLA it was apparent that the

>secondary was far to tall and skinny (2.5 x 26 inches).  I am using (I

>think, 24g wire) and I get 52 turns / inch close wound).  Anyway - I chopped

>10 inches off the secondary. Did it work - did it hell.  I tried every turn

>on the primary and every combination of toriod / top load that I could think

>of.  All I got was a pathetic buzz from the spark gap.  At once point I

>thought I had blown the neon, so I put the MkV primary/secondary plate back

>on the rig.  Wrummmm all was well.

>

>At a total loss for what was happening, I put the two bits of the secondary

>back together with masking tape and soldered up the wire.  This was the act

>of a desperate coiler....

Ken

Chopping 10 inches off of your secondary would drasticly change its resonant

frequency. Perhaps nearly out of range of your primary circuit considering

the large size of your primary cap.

Lets work a few numbers working with the info you gave us above.

Assuming 52 turns per inch that would be AWG 25.

So a 26 inch winding of AWG 25 on a 2.5 inch form would have about 1350

turns and a resonant frequency of around 650 KHZ.

Chopping off 10 inches so our winding is now 16 inches gives us 830 turns

and a resonant frequency of 915 KHZ.

You didn't supply details about your primary but you would need

far fewer turns tapped in to tune this shortened secondary.

A large toroid will get the resonant frequency back down and allow

you to tap in more primary turns. But too large a toroid may not break

out at your power level. Lay a piece of bare wire hanging

over the edge of the toroid to provide a point for spark to breakout or bring

a grounded wire close to the toroid to promote a spark.

A flourescent light tube can be a help in tuning when you are not generating

much spark. Tune for brightest glow.

The combination of a large toroid and large tank capacitance should make

for a potent performing coil. You just need to get the tuning right.

>

>Wruuummmm.  Off she went, tuning nicely at 8 of 10.  Without going into

>details about the new primary and things.  The only item that I can think of

>is that my caps may be totally over / under valued.  I have them at .02 each

>and they are in series for .01uf. Details are 45cm x 190 cm twin plate

>rolled on 1.25cm polyethelyne sheet - tight rolled in mineral oil.

Caps should be fine if a bit large. Your problem is that your primary circuit

and secondary coil are no longer resonant at the same frequency.

>

>The MkV tuned right out of the box on the numbers using NTESLA using my

>assumed cap figures.

>

>If the caps are not the problem - I have a question.  When we use the term

>HEIGHT in regards to secondary - what are we using it for.  Is it to get the

>number of turns available or something else.

The height is usually meant as the secondary winding length. The most efficient

secondaries have height/diameter (h/d) ratios of between 6 to 1 and 3 to 1.

Pick a ratio appropriate for your secondary diameter and get your winding

length.

Coil Form Diameter   H/D Ratio     Winding Length

3 inches           6:1              18 inches

4 inches           5:1              20 inches

5 inches         4.5:1            22.5 inches

6 inches           4:1              24 inches

7 inches         3.5:1            24.5 inches

8 inches           3:1              24 inches

Use the actual OD of your form. (3 inch PVC is actually 3 1/2 inches OD)

Pick the wire guage to give you from 700-900 turns the space allotted.

These are general guidlines and you can vary considerably from this and

still have a functional coil.

>Likewise is that the height above the bottom of the primary or height

>of wire and how is all this affected as to wether the primary is

>flat / angles or a straight helix ?

The primary geometry affects only the coupling between the primary and

secondary.

The helical (solenoid) has the greatest coupling while the flat spiral

primary has the least. The inverted conical type is somewhere between the two.

TTYL!

Comments criticism and cash donations (US Currency) welcome.

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Sat, 22 Mar 1997 14:23:24 -0500

Subject: Re: 10KV RMS @ 300MA for less than $100

>             Tue, 18 Mar 1997 14:12:17 -0800

>             Skip Greiner <sgreiner@wwnet.com>

>         To: Tesla List <tesla@pupman.com>

***SNIP***

>

> Hi Fr. McGahee and All

>

>

> Today I received 2 of the infamous trannies from H&R. They appear to

> have arrived in good shape. They do have the wire leads which is

> apparently a plus. Good. They arrived without a wiring diagram. Can you

> help? The terminals are marked 1 thru 7. 6 and 7 should be the HV leads

> since they have the HV connector on them. 1 and 2 go to what I assume is

> the resonator winding which is totally separate from the second winding

> which contains the HV winding. 3, 4 and 5 I assume are the primary

> 115vac inputs. If all of the above is true, do you know how 3,4 and 5

> are to be connected?

*** INCORRECT! PLEASE SEE BELOW! ***

>

> Before I blow one of these up, there is one thing in the discussion so

> far which I do not understand. What is the problem with running these

> things in a TC application *with* the resonator cap connected? I'd

> really appreciate some inputs on this.

>

> Thanks in advance

>

> Skip

Skip, Huffman, and others interested in the H&R transformers,

At the end of this post I have appended the relevant portions of Barry

Benson's latest response to my queries concerning his successful use of the

H&R transformer. Please read it in its entirety.

In the meantime I will attempt an appropriate response to Skip's questions,

and a few other questions that have been raised.

Different versions of this transformer have the wires *numbered somewhat

differently*. For this reason I will refrain from talking about them by

number, as that might lead to someone misunderstanding and assuming that

the same numbers apply to their particular transformer. However, it is easy

in practice to figure out what each wire or terminal is, so that is what we

will do!

It appears that the C&H transformer is for all intents and purposes the

same transformer as the H&R transformer (but the C&H costs a few bucks

more).

1) The High Voltage Secondary is the largest winding on the transformer. It

has two connections. If you have the wire-lead style transformer, then you

will find two Black High Voltage Silicone wires that have a breakdown

rating of 20KV marked on them. If you have the quick-connect style

transformer, then the terminals are mounted about 1.5" apart. (This close

spacing is the real weak point of the H&R transformers. The ones with wire

leads are less prone to arc-over in Tesla coil use).

Examination of the wire used in the secondary shows it to be #28 gauge,

which is excellent. HV insulation is impregnated paper between winding

layers, and cloth tape that has been heavily varnished as a protective

outer covering. This appears to be more than adequate. The weak spot, as

already noted, is the spacing between the inner and outer HV leads in the

case of the quick-connect variety. If you have the quick-connect variety

you can modify it *without* disassembling the whole transformer.

2) The secondary is physically wound over the Resonating Winding. This

winding is center-tapped. In the wire-lead style transformer, these wires

are red. In the quick-connect style the three terminals are located on the

side of the transformer opposite the HV connections.

Barry mentions that he does *NOT* use the Resonating Windings at all. I

have experimented with using them, and have come to the conclusion that it

is best NOT to use them. When used, the *open circuit* voltage increases

from 2,700 volts to 5,000 volts. But in Tesla coil use this leads to an

undesireable charging pattern that shows itself as a sort of thumping noise

(the noise is generated in my system by the vibrations of the capacitor

plates. The noise is merely an indicator of the uneven charging pattern). I

agree with Barry that we do well to just leave the Resonating Windings

alone.

Yeah, well, OK, but for the curious among you, The open circuit voltage

produced (according to *my* meter) is 2,700 volts with *NO* capacitance

used. Applying the supplied 12 MFD @ 600 VAC oil-filled cap to the two

outside Resonating Winding Terminals resulted in a reading of 5,200 volts.

Using the center-tap and either outside lead resulted in an output of 3,300

volts. The wire used for the Resonating Winding is positively wimpy

compared to the actual primary. Do *NOT* attempt to run the transformer

using the Resonating Winding(s) as the primary! The current drawn through

the Resonating Winding would exceed the current carrying capacity of the

wire!

PLEASE, don't go trying to run this thing in resonant mode on your Tesla

coil. Not only would it be a bumpy ride, but you may destroy the

transformer.

3) The Primary is the totally separate winding that looks like a pancake.

It is wound with heavy-duty wire and held together with varnish. On the

quick-connect version you can see the wire. It is quite robust!

Between the primary and the secondary there are two current-limiting

shunts. Removal of these shunts is not adviseable, as then you would have

to provide external current limiting.

Some people have remarked that they tried to use these things for tube

Tesla coils and found them unsatisfactory. If using the resonant windings,

I am *sure* they were lousey! They probably jumped all over the place

voltage-wise! If used without the resonant windings then they should be OK.

By the way, if I were to use these to run tube Tesla coils, I would

probably want to remove the shunts.

***** Some additional comments *****

Safety gaps across *each* transformer are a must. If you have the

quick-connect style transformer, please modify it, or it may not survive.

Properly modified and protected, the transformers are excellent.

Yes, at 2.7KV RMS the voltage is a bit on the low side. However, the

current capacity is large, and I have found that two of these delivering

5.4KV at 300 ma can do an excellent job in Tesla coil service. I had to

change to a .05 mfd capacitor to get everything matched, and I am going to

have to fabricate a new spark gap for this sucker, but it is already

producing better output than I used to get with a 15KV 60 ma transformer.

As a consolation prize, we all get to use capacitors with lower voltage

ratings if we want to.

*** Barry Benson's Latest Reply ***

Fellow coilers,

I finally got a second reply from Barry Benson on Chip's Tesla list

regarding some of the questions about using the H&R transformers. I am

reposting it to the Tesla-2 list since some Tesla-2 members may not be

Tesla list members. For the sake of brevity I have cut out all but the

relevant response material. Hope this proves useful.

Fr. Tom McGahee

>         Fri, 21 Mar 1997 12:04:00 -0500 (EST)

>   From:

>         Benson_Barry%PAX5@mr.nawcad.navy.mil

>

> Benson_Barry%PAX5@mr.nawcad.navy.mil

>

*** Barry replied as follows: ***

> 1) Do you use the resonating winding at all? The capacitor?

>

> Never used it.  Never found a need.  I figured that since the

> transformer had enough voltage from TC kickback to arc over

> itself that it might be dangerous to try.

>

> 2) Did you make *any* modifications at all to the transformer itself,

> such

> as removing the shunts? If so, could you explain what you did and how

> you

> did it? What were the practical results? Any "gotchas"?

>

> Never touched it except for repositioning and putting some teflon

> spaghetti tubing on the HV leadout wire. I mentioned this mod

> a while back on the list.  If past posts could be organized for

> encyclopaedic perusal it would save a lot of people from the

> past mistakes of others such as myself.

> The transformers that I purchased 10 years ago had an E core with

> a flat plate on the E.  No shunts.

> I wouldn't remove the shunts on the new ones without external current

> limiting.  It might make it more efficient but at the expense of the

> wire melting from the huge current these things put out.

>

> 3) You mentioned repairing one of the transformers. What is the quality

> of

> their original construction in your opinion? Do you remember

> approximately

> what size wire was used for the secondary? Can they really handle the

> rated

> 300 ma continuously in your opinion?

>

> It appears to have (polyurethane?) impregnated paper between the

> windings.  It burns very fast.  Not the best insulation.  Mine are still

> good

> though after 10 years of use.

>

> 4) Of the failure modes you have experienced, what were they, and how

> can

> they be reduced?

>

> Arc over incinerates the insulation.  I protect mine with 1/8" needle

> sharp

> tungsten points set to break over at about 9 kV.  Every couple of hours

> they have to be resharpened because of the current.  I have thought of

> using a horn gap for future tests because of the high currents.  Maybe

> someone can try that out.

>

> 5) Have you added any auxiliary stuff such as external current limiting?

> If

> so, be as explicit as you can as to what you have added, and why.

>

> I originally tried the two variac method.  One variac to adjust voltage

> and one in series to adjust the current.  The current limiting variac

> didn't because it didn't have the slice out of it as Richard Hull

> pointed out in one of his earlier tapes.  As soon as the core saturated

> the breaker poped.  I just run the system above that point.

> I also have MOVS across the primary of both transformers.

> One between hot and neutral, one between hot and ground, and one

> between neutral and ground.

>

*** Note: the newer transformers have built-in current limiting at 300 ma

and do not require any external current limiting ***

> 6) The common mode choke you mentioned: is this homemade, or can we buy

> one

> somewhere? Details?

> Noise.

> Homemade.  Use an old transformer core with about 1" square cross

> section.  My core isn't gapped.  Strip the braid off of a couple hundred

> feet of RG58 solid polyethylene (A/U maybe) cable (fun!).  Wind about 50

> to 100 turns around each transformer core leg.  One winding is connected

> in series with the output lead of one H&R transformer.  The other

> winding

> is connected in series with the output lead from the other H&R

> transformer.

> The ends of these windings go to the Tesla coil rotary gap.  The sense

> of the winding connections is such that the current flow through one

> winding

> cancels the current flow through the other i.e. "common mode".

> The whole choke is floated electrically by placing it on a 1/8" sheet

> of plexiglas with 3" margins.

> Any current surge that tries to feed into both high voltage leads

> in the same direction at the same time creates magnetic fields that

> add.  This makes the common mode choke appear as a very high reactive

> impedance to Tesla coil noise "common" in phase to each line.

> That is how an electrical engineer friend of mine explained it to me.

> He said that the same kind of common mode choke is used in switching

> converters for light weight power supplies to filter niose out of the DC

> output.

>

*** Since the purpose of this choke is to remove noise, it appears to be

purely optional ***

> Current limiting.

> He also said that if the windings were in the same direction then the

> current from the transformers could be limited depending upon the

> number of turns and size of the core (> saturation).  He said that the

> much smaller current out of the high voltage side of the transformers

> would be a lot easier to limit this way than with variacs and arc

> welders

> on the multiampere primary side.  I've always been intrigued by this but

> never tried it.  I will try it on my 10 kw next system.

>

> Cleaning.

> I would recommend dusting off your transformer coils before a run to

> prevent an accident from arc over due to invertebrates and dust.

>

*** Yes, Dead Bug Removal is a must when using open frame transformers that

are not in any protective enclosures. By the way, what some coilers thought

were "burn marks" on their transformers simply turned out to be very black

sooty dust that had been attracted to the transformers when they were being

used in the electrostatic copiers they were designed for. I use a paint

brush and a little compressed air to clean up such dust. The paint brush

helps to break the stuff loose if it is held on by oil or moisture.

> One more thing.

> The safety gap settings are for dry weather or indoors with cool

> temperatures.  High humidity or temperatures (inadvisable) will

> drastically change this.

> Barry

>

My thanks to Barry for taking the time to share with us his knowledge about

these transformers.

Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: "Thomas McGahee" <tom_mcgahee@sigmais.com>

Date: Tue, 25 Mar 1997 08:52:23 -0500

Subject: Re: H&R transformers by the numbers

----------

> From: Tesla-2 <tesla-2@emachine.com>

> To: tom_mcgahee@sigmais.com

> Subject: H&R transformers by the numbers

> Date: Tuesday, March 25, 1997 12:08 AM

>

> From: Roderick Maxwell <major@vicksburg.com>

> Date: Mon, 24 Mar 1997 21:08:50 -0800

> Subject: H&R transformers by the numbers

>

> Well I got those big suckers today! Now the only question is how do you

> connect them in antiparallel? Since all of the terminals are numbered

> can anyone describe the way you connected them? (By the numbers of

> course).I know which wires are which but I,ve never connected

> transformers in series or in antiparallel,only in parallel.

>

>

>

>

>                               Frankensteins Helper

>                                      Max

>

Max,

Hope the stuff appended at the end of this post helps you determine the

wiring numbers for your particular transformer(s).

Once you have determined the  wiring of each transformer, you need to wire

them together in an anti-parallel arrangement. That is just a fancy term

for winding the transformer primaries in such a way that they are paralled,

but the phasing is reversed. That allows us to wire the secondaries in a

special series arrangement in which the outer HV leads produce opposite

polarities at any given instant. One reason for doing that is that it

allows us to ground the center tapped secondaries to the core and to AC

ground so that the electrical stresses on the transformer are reduced. It

is important that it is the *INNER* HV leads that get tied together and

connected to ground!

Some stupid ASCII drawings:

A1----p s----C1

p s

p s

p s

B1----p s----D1

A2----p s----C2

p s

p s

p s

B2----p s----D2

Assume:

(p) is primary

(s) is secondary

A/B are primary wires

C/D are secondary wires

D1 and D2 are the inner HV

C1 and C2 are the outer HV

Wire the primary wires together as follows:

First AC input wire goes to A1 and B2.

Second AC input wire goes to B1 and A2.

That puts the primary in anti-parallel.

Wire the secondary wires as follows:

D1 connects to D2.

(That connects the inner HV wires together).

Ground the connection just made to the core.

Ground the core to the AC ground (Green wire on line cord).

(You MUST use a three wire line cord).

The C1 and D1 wires are now the HV output wires.

You *MUST* have a safety gap for EACH transformer. Adjust each gap for

breakdown at about 5KV. The safety gaps each connect to the core ground

(which connects to AC ground and the inner HV wires). The other side of

each safety gap goes to one of the outer HV leads.

Fr. Tom McGahee

****** Appended Stuff

> Subject: Re: 10KV RMS @ 300MA for less than $100

> Date: Saturday, March 22, 1997 2:23 PM

>

> >             Tue, 18 Mar 1997 14:12:17 -0800

> >             Skip Greiner <sgreiner@wwnet.com>

> >         To: Tesla List <tesla@pupman.com>

>

> ***SNIP***

>

> >

> > Hi Fr. McGahee and All

> >

> >

> > Today I received 2 of the infamous trannies from H&R. They appear to

> > have arrived in good shape. They do have the wire leads which is

> > apparently a plus. Good. They arrived without a wiring diagram. Can you

> > help? The terminals are marked 1 thru 7. 6 and 7 should be the HV leads

> > since they have the HV connector on them. 1 and 2 go to what I assume

is

> > the resonator winding which is totally separate from the second winding

> > which contains the HV winding. 3, 4 and 5 I assume are the primary

> > 115vac inputs. If all of the above is true, do you know how 3,4 and 5

> > are to be connected?

>

> *** INCORRECT! PLEASE SEE BELOW! ***

> >

> > Before I blow one of these up, there is one thing in the discussion so

> > far which I do not understand. What is the problem with running these

> > things in a TC application *with* the resonator cap connected? I'd

> > really appreciate some inputs on this.

> >

> > Thanks in advance

> >

> > Skip

>

> Skip, Huffman, and others interested in the H&R transformers,

> At the end of this post I have appended the relevant portions of Barry

> Benson's latest response to my queries concerning his successful use of

the

> H&R transformer. Please read it in its entirety.

>

> In the meantime I will attempt an appropriate response to Skip's

questions,

> and a few other questions that have been raised.

>

> Different versions of this transformer have the wires *numbered somewhat

> differently*. For this reason I will refrain from talking about them by

> number, as that might lead to someone misunderstanding and assuming that

> the same numbers apply to their particular transformer. However, it is

easy

> in practice to figure out what each wire or terminal is, so that is what

we

> will do!

>

> It appears that the C&H transformer is for all intents and purposes the

> same transformer as the H&R transformer (but the C&H costs a few bucks

> more).

>

> 1) The High Voltage Secondary is the largest winding on the transformer.

It

> has two connections. If you have the wire-lead style transformer, then

you

> will find two Black High Voltage Silicone wires that have a breakdown

> rating of 20KV marked on them. If you have the quick-connect style

> transformer, then the terminals are mounted about 1.5" apart. (This close

> spacing is the real weak point of the H&R transformers. The ones with

wire

> leads are less prone to arc-over in Tesla coil use).

>

> Examination of the wire used in the secondary shows it to be #28 gauge,

> which is excellent. HV insulation is impregnated paper between winding

> layers, and cloth tape that has been heavily varnished as a protective

> outer covering. This appears to be more than adequate. The weak spot, as

> already noted, is the spacing between the inner and outer HV leads in the

> case of the quick-connect variety. If you have the quick-connect variety

> you can modify it *without* disassembling the whole transformer.

>

>

> 2) The secondary is physically wound over the Resonating Winding. This

> winding is center-tapped. In the wire-lead style transformer, these wires

> are red. In the quick-connect style the three terminals are located on

the

> side of the transformer opposite the HV connections.

>

> Barry mentions that he does *NOT* use the Resonating Windings at all. I

> have experimented with using them, and have come to the conclusion that

it

> is best NOT to use them. When used, the *open circuit* voltage increases

> from 2,700 volts to 5,000 volts. But in Tesla coil use this leads to an

> undesireable charging pattern that shows itself as a sort of thumping

noise

> (the noise is generated in my system by the vibrations of the capacitor

> plates. The noise is merely an indicator of the uneven charging pattern).

I

> agree with Barry that we do well to just leave the Resonating Windings

> alone.

>

> Yeah, well, OK, but for the curious among you, The open circuit voltage

> produced (according to *my* meter) is 2,700 volts with *NO* capacitance

> used. Applying the supplied 12 MFD @ 600 VAC oil-filled cap to the two

> outside Resonating Winding Terminals resulted in a reading of 5,200

volts.

> Using the center-tap and either outside lead resulted in an output of

3,300

> volts. The wire used for the Resonating Winding is positively wimpy

> compared to the actual primary. Do *NOT* attempt to run the transformer

> using the Resonating Winding(s) as the primary! The current drawn through

> the Resonating Winding would exceed the current carrying capacity of the

> wire!

>

> PLEASE, don't go trying to run this thing in resonant mode on your Tesla

> coil. Not only would it be a bumpy ride, but you may destroy the

> transformer.

>

> 3) The Primary is the totally separate winding that looks like a pancake.

> It is wound with heavy-duty wire and held together with varnish. On the

> quick-connect version you can see the wire. It is quite robust!

>

> Between the primary and the secondary there are two current-limiting

> shunts. Removal of these shunts is not adviseable, as then you would have

> to provide external current limiting.

>

> Some people have remarked that they tried to use these things for tube

> Tesla coils and found them unsatisfactory. If using the resonant

windings,

> I am *sure* they were lousey! They probably jumped all over the place

> voltage-wise! If used without the resonant windings then they should be

OK.

> By the way, if I were to use these to run tube Tesla coils, I would

> probably want to remove the shunts.

>

> ***** Some additional comments *****

>

> Safety gaps across *each* transformer are a must. If you have the

> quick-connect style transformer, please modify it, or it may not survive.

> Properly modified and protected, the transformers are excellent.

>

> Yes, at 2.7KV RMS the voltage is a bit on the low side. However, the

> current capacity is large, and I have found that two of these delivering

> 5.4KV at 300 ma can do an excellent job in Tesla coil service. I had to

> change to a .05 mfd capacitor to get everything matched, and I am going

to

> have to fabricate a new spark gap for this sucker, but it is already

> producing better output than I used to get with a 15KV 60 ma transformer.

> As a consolation prize, we all get to use capacitors with lower voltage

> ratings if we want to.

>

> *** Barry Benson's Latest Reply ***

>

> Fellow coilers,

> I finally got a second reply from Barry Benson on Chip's Tesla list

> regarding some of the questions about using the H&R transformers. I am

> reposting it to the Tesla-2 list since some Tesla-2 members may not be

> Tesla list members. For the sake of brevity I have cut out all but the

> relevant response material. Hope this proves useful.

>

>

> Fr. Tom McGahee

>

> >         Fri, 21 Mar 1997 12:04:00 -0500 (EST)

> >   From:

> >         Benson_Barry%PAX5@mr.nawcad.navy.mil

> >

> > Benson_Barry%PAX5@mr.nawcad.navy.mil

> >

>

> *** Barry replied as follows: ***

>

> > 1) Do you use the resonating winding at all? The capacitor?

> >

> > Never used it.  Never found a need.  I figured that since the

> > transformer had enough voltage from TC kickback to arc over

> > itself that it might be dangerous to try.

> >

> > 2) Did you make *any* modifications at all to the transformer itself,

> > such

> > as removing the shunts? If so, could you explain what you did and how

> > you

> > did it? What were the practical results? Any "gotchas"?

> >

> > Never touched it except for repositioning and putting some teflon

> > spaghetti tubing on the HV leadout wire. I mentioned this mod

> > a while back on the list.  If past posts could be organized for

> > encyclopaedic perusal it would save a lot of people from the

> > past mistakes of others such as myself.

> > The transformers that I purchased 10 years ago had an E core with

> > a flat plate on the E.  No shunts.

> > I wouldn't remove the shunts on the new ones without external current

> > limiting.  It might make it more efficient but at the expense of the

> > wire melting from the huge current these things put out.

> >

> > 3) You mentioned repairing one of the transformers. What is the quality

> > of

> > their original construction in your opinion? Do you remember

> > approximately

> > what size wire was used for the secondary? Can they really handle the

> > rated

> > 300 ma continuously in your opinion?

> >

> > It appears to have (polyurethane?) impregnated paper between the

> > windings.  It burns very fast.  Not the best insulation.  Mine are

still

> > good

> > though after 10 years of use.

> >

> > 4) Of the failure modes you have experienced, what were they, and how

> > can

> > they be reduced?

> >

> > Arc over incinerates the insulation.  I protect mine with 1/8" needle

> > sharp

> > tungsten points set to break over at about 9 kV.  Every couple of hours

> > they have to be resharpened because of the current.

ADDENDUM-----------------------------------------------------

That is 2 separate

safety gaps (set to break at 4.5kv each-approx. 100v on variac) with

one side grounded to the inner high voltage leads of the transformers which

are in turn grounded to the case (core) which is grounded to the 115

VAC ground from the line cord.

----- END ADDENDUM--------------------

>> I have thought of

> > using a horn gap for future tests because of the high currents.  Maybe

> > someone can try that out.

> >

Note: A Horn Gap is sort of like a mini Jacob's ladder. The bottom of the

gap is set to fire at 4.5KV, but then the gap material bends away from the

starting gap. When an arc develops at the bottom of the gap, the hot air

rising makes the arc rise. Since the gap now diverges and gets further

apart, the arc will eventually extinguish. Without this feature the safety

gap might continue to flame for an excessive amount of time after an arc is

initiated.

Keep in mind that these transformers produce fairly high currents, and the

arcs can be quite massive once they initiate. Make sure that the safety gap

eventually turns off!

> > 5) Have you added any auxiliary stuff such as external current

limiting?

> > If

> > so, be as explicit as you can as to what you have added, and why.

> >

> > I originally tried the two variac method.  One variac to adjust voltage

> > and one in series to adjust the current.  The current limiting variac

> > didn't because it didn't have the slice out of it as Richard Hull

> > pointed out in one of his earlier tapes.  As soon as the core saturated

> > the breaker poped.  I just run the system above that point.

> > I also have MOVS across the primary of both transformers.

> > One between hot and neutral, one between hot and ground, and one

> > between neutral and ground.

> >

>

> *** Note: the newer transformers have built-in current limiting at 300 ma

> and do not require any external current limiting ***

>

> > 6) The common mode choke you mentioned: is this homemade, or can we buy

> > one

> > somewhere? Details?

> > Noise.

> > Homemade.  Use an old transformer core with about 1" square cross

> > section.  My core isn't gapped.  Strip the braid off of a couple

hundred

> > feet of RG58 solid polyethylene (A/U maybe) cable (fun!).  Wind about

50

> > to 100 turns around each transformer core leg.  One winding is

connected

> > in series with the output lead of one H&R transformer.  The other

> > winding

> > is connected in series with the output lead from the other H&R

> > transformer.

> > The ends of these windings go to the Tesla coil rotary gap.  The sense

> > of the winding connections is such that the current flow through one

> > winding

> > cancels the current flow through the other i.e. "common mode".

> > The whole choke is floated electrically by placing it on a 1/8" sheet

> > of plexiglas with 3" margins.

> > Any current surge that tries to feed into both high voltage leads

> > in the same direction at the same time creates magnetic fields that

> > add.  This makes the common mode choke appear as a very high reactive

> > impedance to Tesla coil noise "common" in phase to each line.

> > That is how an electrical engineer friend of mine explained it to me.

> > He said that the same kind of common mode choke is used in switching

> > converters for light weight power supplies to filter niose out of the

DC

> > output.

> >

>

> *** Since the purpose of this choke is to remove noise, it appears to be

> purely optional ***

>

> > Current limiting.

> > He also said that if the windings were in the same direction then the

> > current from the transformers could be limited depending upon the

> > number of turns and size of the core (> saturation).  He said that the

> > much smaller current out of the high voltage side of the transformers

> > would be a lot easier to limit this way than with variacs and arc

> > welders

> > on the multiampere primary side.  I've always been intrigued by this

but

> > never tried it.  I will try it on my 10 kw next system.

> >

> > Cleaning.

> > I would recommend dusting off your transformer coils before a run to

> > prevent an accident from arc over due to invertebrates and dust.

> >

>

> *** Yes, Dead Bug Removal is a must when using open frame transformers

that

> are not in any protective enclosures. By the way, what some coilers

thought

> were "burn marks" on their transformers simply turned out to be very

black

> sooty dust that had been attracted to the transformers when they were

being

> used in the electrostatic copiers they were designed for. I use a paint

> brush and a little compressed air to clean up such dust. The paint brush

> helps to break the stuff loose if it is held on by oil or moisture.

>

> > One more thing.

> > The safety gap settings are for dry weather or indoors with cool

> > temperatures.  High humidity or temperatures (inadvisable) will

> > drastically change this.

> > Barry

> >

>

> My thanks to Barry for taking the time to share with us his knowledge

about

> these transformers.

>

> Fr. Tom McGahee

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Fri, 28 Mar 1997 16:55:58 -0600

Subject: wire from Electronic Goldmine

For those of you that were intereted in taking

advantage of the good price on magnet wire from Electronic

Goldmine you're too late.

They have already sold out of the #21 and #24 wire.

I guess I waited too long. Figures.

They still have some of the other sizes left but they tell me

its going fast.

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Roderick Maxwell <major@vicksburg.com>

Date: Mon, 24 Mar 1997 21:08:50 -0800

Subject: H&R transformers by the numbers

Well I got those big suckers today! Now the only question is how do you

connect them in antiparallel? Since all of the terminals are numbered

can anyone describe the way you connected them? (By the numbers of

course).I know which wires are which but I,ve never connected

transformers in series or in antiparallel,only in parallel.

Frankensteins Helper

Max

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Fri, 28 Mar 1997 19:17:35 -0600

Subject: small disaster

I finally got around to firing my 4 inch dia

coil yesterday for the first time in almost 3 months.

I have my new vacuum blown gap almost finished

and wanted to get some runs on the old gap

so I had a reference to compare them by.

I set the coil up with a 6 inch diameter sphere terminal, 12kv/90ma neon,

and set the tuning up from my notes. My notes showed

this configuration gave 26 inch long streamers before.

I fired her up and all went well as I dialed up the variac.

I was getting 2 foot long streamers with 100 VAC into the neon.

When I got up to 110 volts my neon blew up. This was a Jefferson Electric

12kv/60ma neon I had unpotted and rebuilt. I had removed 5 shunts

from it to boost cuurent to 90 ma. I had beat this transformer

pretty hard last year without a whimper.

I quickly killed the power and a quick ohmeter check showed the secondary

to still be intact. So I disassembled it and found where the arc over had

occurred. I cleaned away all the carbonized insulation I could and patched

the area with epoxy.

This transformer had previously been insulated from the core using

hot glue and it seemed to work well. But the arc blew right through

nearly a 1/4 inch of hot glue.

I reinsulated it with soft rope caulking weatherstrip.

This stuff is cheap and seems to have great insulating properties.

Use like HV putty. I molded it around the secondaries and reassembled the

transformer.

It seems to be just fine and survived the incident intact.

Just goes to show you that despite taking all the precautions

including inline chokes and shunt gap circuit and safety gaps

that neons still can and will blow up on you.

Fortunately by killing the power quickly the secondary winding

wasn't damaged and I was able to repair it.

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Roderick Maxwell <major@vicksburg.com>

Date: Sun, 30 Mar 1997 18:00:27 -0800

Subject: neon secondaries

I have finally cleaned up the 15kv neon transformers that I got way back

in january. Out of four transformers I may be able to get three of them

going again! Two of the secondaries are toast for sure though. the other

six have varying resistances from 8.7k to 7.5k . My question is are

these the correct readings for 15kv secondaries, or are some or all

shorted?

Frankensteins Helper

Max

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Sun, 30 Mar 1997 19:29:55 -0600

Subject: Re: neon secondaries

At 02:00 AM 3/31/97 GMT, you wrote:

>From: Roderick Maxwell <major@vicksburg.com>

>Date: Sun, 30 Mar 1997 18:00:27 -0800

>Subject: neon secondaries

>

>I have finally cleaned up the 15kv neon transformers that I got way back

>in january. Out of four transformers I may be able to get three of them

>going again! Two of the secondaries are toast for sure though. the other

>six have varying resistances from 8.7k to 7.5k . My question is are

>these the correct readings for 15kv secondaries, or are some or all

>shorted?

>

>

>

Roderick,

The DC resistance of neon secondaries varies quite a bit

from model to model and manufacturer to manufacturer. Even the

2 secondaries on the same transformer may show different values.

60ma neons usually show lower resistance than the 30s do.

Just for grins I measured the resistance of one of my

good France 15kv/30s for you. I got 7.6 K on one and

7.64 K on the other. I would say your secondaries are OK.

What had you planned to reinsulate the secondaries from the core with?

I have tried lots of various things and the best so far has

been the soft pliable rope caulking. This stuff is used just like

HV putty. Mold it around the secondary and try to fill up the

space between the core and secondary with the stuff. It has

worked very well on the 2 neons I have used it on. It can also be

easily removed if need be.

TTYL!

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Roderick Maxwell <major@vicksburg.com>

Date: Sun, 30 Mar 1997 23:22:42 -0800

Subject: Re: neon secondaries

Tesla-2 wrote:

>

> From: Mike Hammer <mhammer@misslink.net>

> Date: Sun, 30 Mar 1997 19:29:55 -0600

> Subject: Re: neon secondaries

>

> At 02:00 AM 3/31/97 GMT, you wrote:

> >From: Roderick Maxwell <major@vicksburg.com>

> >Date: Sun, 30 Mar 1997 18:00:27 -0800

> >Subject: neon secondaries

> >

> >I have finally cleaned up the 15kv neon transformers that I got way back

> >in january. Out of four transformers I may be able to get three of them

> >going again! Two of the secondaries are toast for sure though. the other

> >six have varying resistances from 8.7k to 7.5k . My question is are

> >these the correct readings for 15kv secondaries, or are some or all

> >shorted?

> >

> >

> >

>

> Roderick,

>

> The DC resistance of neon secondaries varies quite a bit

> from model to model and manufacturer to manufacturer. Even the

> 2 secondaries on the same transformer show different values.

>

> 60ma neons usually show lower resistance than the 30s do.

>

> Just for grins I measured the resistance of one of my

> good France 15kv/30s for you. I got 7.6 K on one and

> 7.64 K on the other. I would say your secondaries are OK.

>

> What had you planned to reinsulate the secondaries from the core with?

>

> I have tried lots of various things and the best so far has

> been the soft pliable rope caulking. This stuff is used just like

> HV putty. Mold it around the secondary and try to fill up the

> space between the core and secondary with the stuff. It has

> worked very well on the 2 neons I have used it on. It can also be

> easily removed if need be.

>

> TTYL!

> Mike Hammer

> mhammer@misslink.net

Thank you for the information! I'm in kind of a toss up between

varnishing the daylights out of them with polyurathane, putting them in

a oil tank ,coating them in rtv rubber, or coating them in a compound

called liquid electrical tape which has a dialectric strength of 1700vac

per mil.

Another question, how many shunts do you suggest I remove? My core

configuration is almost like the one in neon txt. except the core is an

E core and the windings are on the center post of the transformer. And

there are four sets of shunts, two on top and two on the bottom.

120 VOLTS

high voltage    v   v     high voltage

__I__        I   I        __I__

|  S  |     __I___I__     |  S  |

|  E  |    |    P    |    |  E  |

_______|  C  |____|    R    |____|  C  |______

|      |  O  |    |    I    |    |  O  |      |

|      |  N  |    |    M    |    |  N  |      |

|     -|  D  |----|    A    |----|  D  |-     |

|     ||  A  |    |    R    |    |  A  ||     |

|     ||  R  |    |    Y    |    |  R  ||     |

|     ||__Y__|     ---------     |__Y__||     |

|     -----------------------------------     |

|             GROUNDED IRON CORE              |

|_____________________________________________|

_____________________________________________

|                                             |

|                                             |

|     -----------------------------------     |

|     |     |  |               |  |     |     |

|     |     |  |  < SHUNTS >   |  |     |     |

|     |     |  |               |  |     |     |

|     -----------------------------------     |

|                                             |

|_____________________________________________|

Frankensteins Helper

Max

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Mon, 31 Mar 1997 00:44:12 -0600

Subject: Re: neon secondaries

At 07:22 AM 3/31/97 GMT, you wrote:

>From: Roderick Maxwell <major@vicksburg.com>

>Date: Sun, 30 Mar 1997 23:22:42 -0800

>Subject: Re: neon secondaries

>

>

>  Thank you for the information! I'm in kind of a toss up between

>varnishing the daylights out of them with polyurathane, putting them in

>a oil tank ,coating them in rtv rubber, or coating them in a compound

>called liquid electrical tape which has a dialectric strength of 1700vac

>per mil.

My best results have been from coating the secondaries with a layer

of epoxy to harden and protect the windings and then mold that rope

caulking compound around the secondaries filling up the space between

them and the core.

I also used polyester resin on one transformer and it worked OK

but not as well as epoxy.

>  Another question, how many shunts do you suggest I remove? My core

>configuration is almost like the one in neon txt. except the core is an

>E core and the windings are on the center post of the transformer. And

>there are four sets of shunts, two on top and two on the bottom.

>

Sounds like you are doing a France unit. If its a 30ma neon then

take out only 2 or 3 plates from each shunt. Be sure to remove an

equal number from each shunt. The France 12kv/30 I rebuilt gave 42 ma with

3 plates from each shunt removed. Thats about all you can boost the

30ma neons. However the 60s can take much more. I have removed as

many as 6 shunts from 60ma neons and pushed them to 90 ma.

I recommend reassembling your neons and testing first with

all shunts in place. The remove 1 plate from each shunt at a

time and measure the current output.

If you have an AC ammeter you can connect it directly across

the neon output to measure the short circuit current.

Make sure your meter will read AC milliamps and you have good

solid connections. If a lead falls off under test you could have an

arc and do bad things to your meter.

TTYL!

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Nathan_Bonsal@intuit.com (Nathan Bonsal)

Date: Mon, 31 Mar 1997 10:21:14 -0800

Subject: Re[2]: neon secondaries

Does anyone know where to get good Corona Dope Cheap? I use SHOO-GOO

right now- it's OK but smells god-awful when it's still liquid. I

think Corona Dope has a pretty high Dielectric constant. Still on a

toss up about Mineral Oil-- after all it would be self-healing...

______________________________ Reply Separator ________________________________

_

Subject: Re: neon secondaries

Author:  tesla-2@emachine.com (Tesla-2) at Internet

Date:    3/31/97 7:22 AM

From: Roderick Maxwell <major@vicksburg.com>

Date: Sun, 30 Mar 1997 23:22:42 -0800

Subject: Re: neon secondaries

Tesla-2 wrote:

>

> From: Mike Hammer <mhammer@misslink.net>

> Date: Sun, 30 Mar 1997 19:29:55 -0600

> Subject: Re: neon secondaries

>

> At 02:00 AM 3/31/97 GMT, you wrote:

> >From: Roderick Maxwell <major@vicksburg.com>

> >Date: Sun, 30 Mar 1997 18:00:27 -0800

> >Subject: neon secondaries

> >

> >I have finally cleaned up the 15kv neon transformers that I got way back

> >in january. Out of four transformers I may be able to get three of them

> >going again! Two of the secondaries are toast for sure though. the other

> >six have varying resistances from 8.7k to 7.5k . My question is are

> >these the correct readings for 15kv secondaries, or are some or all

> >shorted?

> >

> >

> >

>

> Roderick,

>

> The DC resistance of neon secondaries varies quite a bit

> from model to model and manufacturer to manufacturer. Even the

> 2 secondaries on the same transformer show different values.

>

> 60ma neons usually show lower resistance than the 30s do.

>

> Just for grins I measured the resistance of one of my

> good France 15kv/30s for you. I got 7.6 K on one and

> 7.64 K on the other. I would say your secondaries are OK.

>

> What had you planned to reinsulate the secondaries from the core with?

>

> I have tried lots of various things and the best so far has

> been the soft pliable rope caulking. This stuff is used just like

> HV putty. Mold it around the secondary and try to fill up the

> space between the core and secondary with the stuff. It has

> worked very well on the 2 neons I have used it on. It can also be

> easily removed if need be.

>

> TTYL!

> Mike Hammer

> mhammer@misslink.net

Thank you for the information! I'm in kind of a toss up between

varnishing the daylights out of them with polyurathane, putting them in

a oil tank ,coating them in rtv rubber, or coating them in a compound

called liquid electrical tape which has a dialectric strength of 1700vac

per mil.

Another question, how many shunts do you suggest I remove? My core

configuration is almost like the one in neon txt. except the core is an

E core and the windings are on the center post of the transformer. And

there are four sets of shunts, two on top and two on the bottom.

120 VOLTS

high voltage    v   v     high voltage

__I__        I   I        __I__

|  S  |     __I___I__     |  S  |

|  E  |    |    P    |    |  E  |

_______|  C  |____|    R    |____|  C  |______

|      |  O  |    |    I    |    |  O  |      |

|      |  N  |    |    M    |    |  N  |      |

|     -|  D  |----|    A    |----|  D  |-     |

|     ||  A  |    |    R    |    |  A  ||     |

|     ||  R  |    |    Y    |    |  R  ||     |

|     ||__Y__|     ---------     |__Y__||     |

|     -----------------------------------     |

|             GROUNDED IRON CORE              |

|_____________________________________________|

_____________________________________________

|                                             |

|                                             |

|     -----------------------------------     |

|     |     |  |               |  |     |     |

|     |     |  |  < SHUNTS >   |  |     |     |

|     |     |  |               |  |     |     |

|     -----------------------------------     |

|                                             |

|_____________________________________________|

Frankensteins Helper

Max

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

Processed with Listserv v2.93 for Wildcat v4

From: Nathan_Bonsal@intuit.com (Nathan Bonsal)

Date: Mon, 31 Mar 1997 11:21:09 -0800

Subject: Re: small disaster

--IMA.Boundary.939238958

Content-Type: text/plain; charset=US-ASCII

Content-Transfer-Encoding: 7bit

Content-Description: cc:Mail note part

I notice that in this post the author is using Hot glue around his

secondaries in the neon. Doesn't the glue melt when the Xformer gets

hot? Also, what is the dielectric constant of the average hot glue? I

would imagine this constant changes when the glue liquifies. Wouldn't

another substance be preferable for this reason? I am NOT judging the

material or its use by others but I would like your thoughts on

this,as I am rebuilding 7 neons myself and I am in the mood to have

them last as close to forever as I can make them. I'll probably remove

1-2 shunts at the MOST from my france 15KV 30ma 's . I can always run

them in parallel for more current.

______________________________ Reply Separator ________________________________

_

Subject: small disaster

Author:  tesla-2@emachine.com (Tesla-2) at Internet

Date:    3/29/97 1:17 AM

From: Mike Hammer <mhammer@misslink.net>

Date: Fri, 28 Mar 1997 19:17:35 -0600

Subject: small disaster

I finally got around to firing my 4 inch dia

coil yesterday for the first time in almost 3 months.

I have my new vacuum blown gap almost finished

and wanted to get some runs on the old gap

so I had a reference to compare them by.

I set the coil up with a 6 inch diameter sphere terminal, 12kv/90ma neon,

and set the tuning up from my notes. My notes showed

this configuration gave 26 inch long streamers before.

I fired her up and all went well as I dialed up the variac.

I was getting 2 foot long streamers with 100 VAC into the neon.

When I got up to 110 volts my neon blew up. This was a Jefferson Electric

12kv/60ma neon I had unpotted and rebuilt. I had removed 5 shunts

from it to boost cuurent to 90 ma. I had beat this transformer

pretty hard last year without a whimper.

I quickly killed the power and a quick ohmeter check showed the secondary

to still be intact. So I disassembled it and found where the arc over had

occurred. I cleaned away all the carbonized insulation I could and patched

the area with epoxy.

This transformer had previously been insulated from the core using

hot glue and it seemed to work well. But the arc blew right through

nearly a 1/4 inch of hot glue.

I reinsulated it with soft rope caulking weatherstrip.

This stuff is cheap and seems to have great insulating properties.

Use like HV putty. I molded it around the secondaries and reassembled the

transformer.

It seems to be just fine and survived the incident intact.

Just goes to show you that despite taking all the precautions

including inline chokes and shunt gap circuit and safety gaps

that neons still can and will blow up on you.

Fortunately by killing the power quickly the secondary winding

wasn't damaged and I was able to repair it.

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Mon, 31 Mar 1997 13:21:10 -0600

Subject: Re: Re[2]: neon secondaries

At 06:21 PM 3/31/97 GMT, you wrote:

>From: Nathan_Bonsal@intuit.com (Nathan Bonsal)

>Date: Mon, 31 Mar 1997 10:21:14 -0800

>Subject: Re[2]: neon secondaries

>

>--IMA.Boundary.204928958

>Content-Type: text/plain; charset=US-ASCII

>Content-Transfer-Encoding: 7bit

>Content-Description: cc:Mail note part

>

>     Does anyone know where to get good Corona Dope Cheap? I use SHOO-GOO

>     right now- it's OK but smells god-awful when it's still liquid. I

>     think Corona Dope has a pretty high Dielectric constant. Still on a

>     toss up about Mineral Oil-- after all it would be self-healing...

>

>

Nathan,

Please don't quote back entire message threads when replying.

Take a minute to cut out the unnecessary material.

(This goes for everybody in general)

GC Electronics makes a good polystyrene Q dope. Most any electronics

distributor carries GC products. I got mine from Newark Electronics.

Its not cheap.

I was not pleased with corona dope on neon secondaries. I had better

results with epoxy or hot glue. Hot glue is mainly polyethylene

and is cheap and you can lay down a nice thick layer.

The best stuff I've used has been soft rope caulking. This stuff

is ultra cheap. I got enough to do 2 transformers for $1.75 from

a local Menards. Ace hardware also carries this. Its soft and doesn't

harden. And you can remove it if you need to and reuse it.

Mineral or transformer oil would be the ultimate but if your transformer

were to break down or develop a problem it sure would be messy working

on it. I fire mine dry and they work that way just fine.

TTYL!

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Mon, 31 Mar 1997 13:34:51 -0600

Subject: Re: small disaster

At 07:21 PM 3/31/97 GMT, you wrote:

>From: Nathan_Bonsal@intuit.com (Nathan Bonsal)

>Date: Mon, 31 Mar 1997 11:21:09 -0800

>Subject: Re: small disaster

>

>--IMA.Boundary.939238958

>Content-Type: text/plain; charset=US-ASCII

>Content-Transfer-Encoding: 7bit

>Content-Description: cc:Mail note part

>

>     I notice that in this post the author is using Hot glue around his

>     secondaries in the neon. Doesn't the glue melt when the Xformer gets

>     hot? Also, what is the dielectric constant of the average hot glue? I

>     would imagine this constant changes when the glue liquifies. Wouldn't

>     another substance be preferable for this reason? I am NOT judging the

>     material or its use by others but I would like your thoughts on

>     this,as I am rebuilding 7 neons myself and I am in the mood to have

>     them last as close to forever as I can make them. I'll probably remove

>     1-2 shunts at the MOST from my france 15KV 30ma 's . I can always run

>     them in parallel for more current.

>

Nathan,

Hot glue is polyethylene and if your neon is getting hot enough

to melt this then boy have you got a problem. :)

Even after considerable beating your neon secondaries should only be

warm to the touch. The primary will get considerable warmer but

still should not be hot.

Hot glue works well as insulation provided its backed up with

something like the rope caulking I've been using lately.

I have a France 12kv/30 I rebuilt and it uses hot glue only for

insulation and it has done well.

The best combination I've found so far is to coat the secondaries

with epoxy and insulate then from the core with the rope caulking.

With your France 30ma units you can most likely get away with 3 plates

removed from each shunt. That will put you around 40 - 45 ma.

Thats about as much as you can get without risk.

TTYL!

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Roderick Maxwell <major@vicksburg.com>

Date: Mon, 31 Mar 1997 18:34:20 -0800

Subject: Re: neon secondaries

Tesla-2 wrote:

>

> From: Mike Hammer <mhammer@misslink.net>

> Date: Mon, 31 Mar 1997 00:44:12 -0600

> Subject: Re: neon secondaries

>

> At 07:22 AM 3/31/97 GMT, you wrote:

> >From: Roderick Maxwell <major@vicksburg.com>

> >Date: Sun, 30 Mar 1997 23:22:42 -0800

> >Subject: Re: neon secondaries

> >

>

> >

> >  Thank you for the information! I'm in kind of a toss up between

> >varnishing the daylights out of them with polyurathane, putting them in

> >a oil tank ,coating them in rtv rubber, or coating them in a compound

> >called liquid electrical tape which has a dialectric strength of 1700vac

> >per mil.

>

> My best results have been from coating the secondaries with a layer

> of epoxy to harden and protect the windings and then mold that rope

> caulking compound around the secondaries filling up the space between

> them and the core.

>

> I also used polyester resin on one transformer and it worked OK

> but not as well as epoxy.

>

> >  Another question, how many shunts do you suggest I remove? My core

> >configuration is almost like the one in neon txt. except the core is an

> >E core and the windings are on the center post of the transformer. And

> >there are four sets of shunts, two on top and two on the bottom.

> >

>

> Sounds like you are doing a France unit. If its a 30ma neon then

> take out only 2 or 3 plates from each shunt. Be sure to remove an

> equal number from each. The France 12kv/30 I rebuilt gave 42 ma with

> 3 plates from each shunt removed. Thats about all you can boost the

> 30ma neons. However the 60s can take much more. I have removed as

> many as 6 shunts from 60ma neons and pushed them to 90 ma.

>

> I recommend reassembling your neons and testing first with

> all shunts in place. The remove 1 plate from each shunt at a

> time and measure the current output.

>

> If you have an AC ammeter you can connect it directly across

> the neon output to measure the short circuit current.

> Make sure your meter will read AC milliamps and you have good

> solid connections. If a lead falls off under test you could have an

> arc and do bad things to your meter.

>

> TTYL!

> Mike Hammer

> mhammer@misslink.net

>

Thanks for your help!

Frankensteins Helper

Max

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: ChrisJHill@aol.com

Date: Tue, 1 Apr 1997 14:42:50 -0500 (EST)

Subject: xfmr 'shunts', mineral oil, rf filters

Before I ask some questions, I'd like to introduce myself to the list as this

is my first posting (though I've posted on usa-tesla before - thanks go to

Mike Hammer for introducing me to Tesla 2). I live in Cardiff, Wales, U.K.

and I've just completed my first Tesla coil...

Details are:

4.25" PVC secondary, polyurethane coated, 22swg 20" winding, 18" dia toroid

5 turn 10" dia helical primary (wound on PVC former)

solid state car ignition coil based PSU (until I can afford something more

powerful) giving about 10 discharges per second on the spark gap.

36nF oil insulated alu. foil \ polythene tank cap.

(Temporary) Spark gap of ~ 4mm between two M4 (metric) bolts.

9 to 10" secondary discharge.

I'd be very happy to hear of anyone else in the U.K. who builds tc's.

First Question. I've been listening in for some time and have noted the

conversations about removing shunts from neons. I'm still a bit new to this -

what are the shunts for in the first place? Is it for short circuit

protection of the secondary? How do they work? Are they some sort of 'bypass'

so that some primary magnetic flux doesn't go through the secondary?

I've just got hold of a transformer from an old microwave oven - there

doesn't appear to be any kind of insulation on the secondary. The transformer

has what I think are 'shunts'. I don't think that the transformer is suitable

for Tesla work (the cap. it was connected to was only a 2100 VAC rated) but I

would be interested to know if removing the shunts would increase the

secondary voltage. Is it safe to run a microwave transformer with no load?

My second question is regarding oil. Most of the capacitor construction info

I have obtained suggests the use of mineral oil. I have used multigrade oil

for automobile engines - is this the same? (my caps seem to work fine at 20kV

on multigrade, and seem to stay charged for a very long time. I've not been

brave enough to push them further).

Finally, I have seen references to using shorted arc welders in tesla coil

building. What are they used for? (I may be able to get one !)

Sorry if that's a bit long! (shorter next time)

Chris Hill.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Tue, 01 Apr 1997 15:17:26 -0600

Subject: Re: xfmr 'shunts', mineral oil, rf filters

At 07:42 PM 4/1/97 GMT, you wrote:

>From: ChrisJHill@aol.com

>Date: Tue, 1 Apr 1997 14:42:50 -0500 (EST)

>Subject: xfmr 'shunts', mineral oil, rf filters

>

>Before I ask some questions, I'd like to introduce myself to the list as this

>is my first posting (though I've posted on usa-tesla before - thanks go to

>Mike Hammer for introducing me to Tesla 2). I live in Cardiff, Wales, U.K.

>and I've just completed my first Tesla coil...

>

>Details are:

>

>4.25" PVC secondary, polyurethane coated, 22swg 20" winding, 18" dia toroid

>5 turn 10" dia helical primary (wound on PVC former)

>solid state car ignition coil based PSU (until I can afford something more

>powerful) giving about 10 discharges per second on the spark gap.

>36nF oil insulated alu. foil \ polythene tank cap.

>(Temporary) Spark gap of ~ 4mm between two M4 (metric) bolts.

>9 to 10" secondary discharge.

>

>I'd be very happy to hear of anyone else in the U.K. who builds tc's.

>

>First Question. I've been listening in for some time and have noted the

>conversations about removing shunts from neons. I'm still a bit new to this -

>what are the shunts for in the first place? Is it for short circuit

>protection of the secondary? How do they work? Are they some sort of 'bypass'

>so that some primary magnetic flux doesn't go through the secondary?

Neon transformers have internal current limiting in the form of magnetic shunts.

You can completely short circuit a neon indefinitely without harm. Even short

circuited only the rated current will flow.

A few of the shunts can be removed to allow more magnetic flux to reach the

secondaries allowing for greater cuurent flow.

Neons have very fine wire in the secondaries and allowing too much current flow

will easily burn them up.

>

>I've just got hold of a transformer from an old microwave oven - there

>doesn't appear to be any kind of insulation on the secondary. The transformer

>has what I think are 'shunts'. I don't think that the transformer is suitable

>for Tesla work (the cap. it was connected to was only a 2100 VAC rated) but I

>would be interested to know if removing the shunts would increase the

>secondary voltage. Is it safe to run a microwave transformer with no load?

You don't want to remove the shunts from a microwave oven transformer.

They have considerable current capability already. However they don't have

enough voltage to be useful for TC use.  You really need 6kv or more more

decent spark gap operation.

I once powered a TC with 2 series connected oven transformers and had

marginal results. It did provide some decent sparks but it was hard to get

reliable gap operation.

Even a small neon transformer will provide better results.

>

>My second question is regarding oil. Most of the capacitor construction info

>I have obtained suggests the use of mineral oil. I have used multigrade oil

>for automobile engines - is this the same? (my caps seem to work fine at 20kV

>on multigrade, and seem to stay charged for a very long time. I've not been

>brave enough to push them further).

Automotive type oils have anti-wear and anti-foam additives that make them

unsuitable for high voltage use.

Mineral or transformer oils have much better dielectric strength and

are necessary for high voltage operation.

>

>Finally, I have seen references to using shorted arc welders in tesla coil

>building. What are they used for? (I may be able to get one !)

This is often used to provide external current limiting for large transformers

that have no current limiting of their own.

Since neons are internally current limited we don't have to worry about such

things with neon powered coils.

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Roderick Maxwell <major@vicksburg.com>

Date: Tue, 01 Apr 1997 18:16:19 -0800

Subject: How clean is clean enough?

I have clean my 15kv neon secondaries so well you can see the stamped

serial numbers on the paper covering. But when you look at the side of

the secondaries you can still see tar between the winding layers in

places. The question is do you have to remove all of the tar? The only

way I believe you could do it would be to soak, and soak ,and soak ,them

in solvent solvent until they came clean.

Frankensteins Helper

Max

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Roderick Maxwell <major@vicksburg.com>

Date: Wed, 02 Apr 1997 20:36:43 -0800

Subject: neon plunder!

I have to admit that I got jeleous of Nathan Bonsal when he said he was

rebuilding 7 neons. So I went to Jackson today which is about 80 miles

from where I live. The nearest town is vicksburg which is 30 miles

away(I,m a hick I live in the country). Vicksburg has only one small

neon shop, and I already paid him a visit, that is how I got the other 4

transformers. I found another much larger shop in Jackson, one of

several others. I asked the man in charge could I have some neon

transformers for a science project, and I told him I wanted only the bad

ones. He looked sort of puzzled for a moment then he asked me how many I

wanted. I told him how many you got? Well I've got a whole pile of them

out back and I'll charge you a dollar each. I told him okay. So one of

the other men that works in the back showed me a pile about three foot

high! He tells me if the boss don't see ya take as many as you want! So

I grabbed about 23 total. I could have gotten more but the minivans

springs were sitting kind of low! Heres what I grabbed.

13  MAGNATEK    15,000    30ma

4   FRANCEFORMER  15,000  30ma

1   MAGNATEK     15,000   60ma

1   JEFFERSON ELECTRIC 15,0000  30ma

1  TRANSCO  12,000  30ma

3   ?????   12,000  60ma

The last 3 transformers I can't identify but they have two large glass

socketed holes with springs inside on one end to connect to the neon

tubing.

And get this he said come back and get some more if you want( cross

my heart and hope to die)!

Frankensteins Helper

Max

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Roderick Maxwell <major@vicksburg.com>

Date: Wed, 02 Apr 1997 18:15:48 -0800

Subject: How clean is clean enough?

I have cleaned my 15kv neon secondaries so well you can see the

stamped serial numbers on the paper covering. But when you look at the side of

the secondaries you can still see tar between the winding layers in

places. The question is do you have to remove all of the tar? The only

way I believe you could do it would be to soak, and soak ,and

soak, them in solvent solvent until they came clean.

Frankensteins Helper

Max

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Roderick Maxwell <major@vicksburg.com>

Date: Wed, 02 Apr 1997 21:34:47 -0800

Subject: Re: How clean is clean enough?

Tesla-2 wrote:

>

> From: Mike Hammer <mhammer@misslink.net>

> Date: Wed, 02 Apr 1997 10:38:12 -0600

> Subject: Re: How clean is clean enough?

>

> At 02:16 AM 4/2/97 GMT, you wrote:

> >From: Roderick Maxwell <major@vicksburg.com>

> >Date: Tue, 01 Apr 1997 18:16:19 -0800

> >Subject: How clean is clean enough?

> >

> >I have clean my 15kv neon secondaries so well you can see the stamped

> >serial numbers on the paper covering. But when you look at the side of

> >the secondaries you can still see tar between the winding layers in

> >places. The question is do you have to remove all of the tar? The only

> >way I believe you could do it would be to soak, and soak ,and soak ,them

> >in solvent solvent until they came clean.

> >

> >

> >

>

> I would say you have your secondaries cleaner than any of mine.

> If there are no large pieces hanging on then you're OK. Whats left

> along the sides and between layers isn't worth bothering with.

>

> Soaking the secondaries usually results in the outer layers of

> paper insulation coming loose.

Thank you again for your help! I guess that means I can start putting

them back together.

Frankensteins Helper

Max

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Wed, 02 Apr 1997 23:33:56 -0600

Subject: Re: neon plunder!

At 04:36 AM 4/3/97 GMT, you wrote:

>From: Roderick Maxwell <major@vicksburg.com>

>Date: Wed, 02 Apr 1997 20:36:43 -0800

>Subject: neon plunder!

>

>I have to admit that I got jeleous of Nathan Bonsal when he said he was

>rebuilding 7 neons. So I went to Jackson today which is about 80 miles

>from where I live. The nearest town is vicksburg which is 30 miles

>away(I,m a hick I live in the country). Vicksburg has only one small

>neon shop, and I already paid him a visit, that is how I got the other 4

>transformers. I found another much larger shop in Jackson, one of

>several others. I asked the man in charge could I have some neon

>transformers for a science project, and I told him I wanted only the bad

>ones. He looked sort of puzzled for a moment then he asked me how many I

>wanted. I told him how many you got? Well I've got a whole pile of them

>out back and I'll charge you a dollar each. I told him okay. So one of

>the other men that works in the back showed me a pile about three foot

>high! He tells me if the boss don't see ya take as many as you want! So

>I grabbed about 23 total. I could have gotten more but the minivans

>springs were sitting kind of low! Heres what I grabbed.

>

>13  MAGNATEK    15,000    30ma

>

>4   FRANCEFORMER  15,000  30ma

>

>1   MAGNATEK     15,000   60ma

>

>1   JEFFERSON ELECTRIC 15,0000  30ma

>

>1  TRANSCO  12,000  30ma

>

>3   ?????   12,000  60ma

>

>

>  The last 3 transformers I can't identify but they have two large glass

>socketed holes with springs inside on one end to connect to the neon

>tubing.

>

>   And get this he said come back and get some more if you want( cross

>my heart and hope to die)!

>

>

>

>

>

>                             Frankensteins Helper

>                                      Max

You hit the mother lode! Well done. I keep telling people

they are out there you just have to go look.

I would bet you will find some of those transformers to be just fine.

I usually get about 50 percent good transformers out of a pile of

"dead" ones.

Now that you have a good source you can get selective. Go for the

high current models. The 60 ma neons are much more rugged.

If you find a 12kv 120ma unit don't use it! Send it immediately

to me. :)

TTYL!

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Ken Smith <ksmith@ihug.co.nz>

Date: Thu, 03 Apr 1997 19:24:06 -1200

Subject: Re: neon plunder!

>>  The last 3 transformers I can't identify but they have two large glass

>>socketed holes with springs inside on one end to connect to the neon

>>tubing.

>>

>>   And get this he said come back and get some more if you want( cross

>>my heart and hope to die)!

>You hit the mother lode! Well done. I keep telling people

>they are out there you just have to go look.

>

>I would bet you will find some of those transformers to be just fine.

>I usually get about 50 percent good transformers out of a pile of

>"dead" ones.

>

>Now that you have a good source you can get selective. Go for the

>high current models. The 60 ma neons are much more rugged.

>If you find a 12kv 120ma unit don't use it! Send it immediately

>to me. :)

>

Damn, Mike hit the button before I did.  I was going to say "and send the

ones you don't want to ME"  I will pay the DHL <g>

Seriously - you have hit a good source, and when there are that many you can

afford to be seriously selective.  You will soon be taking your ohmmeter

with you to the yard.  I have found some of the F.A.R.T. (really) from Italy

(where it obviously means something else <g>) but they are encased in resin

and totally impossible to unpot.  There a few of the 'inside' neons around,

but they are few and far between.....

Ken

Ken Smith

Weymouth

Auckland

http://homepages.ihug.co.nz/~ksmith

ksmith@ihug.co.nz

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Nathan_Bonsal@intuit.com (Nathan Bonsal)

Date: Thu, 3 Apr 1997 09:38:13 -0800

Subject: Re: neon plunder!

Lucky lucky. Now I'M jealous. Did you ohm them out?

______________________________ Reply Separator ________________________________

_

Subject: neon plunder!

Author:  tesla-2@emachine.com (Tesla-2) at Internet

Date:    4/3/97 4:36 AM

From: Roderick Maxwell <major@vicksburg.com>

Date: Wed, 02 Apr 1997 20:36:43 -0800

Subject: neon plunder!

I have to admit that I got jeleous of Nathan Bonsal when he said he was

rebuilding 7 neons. So I went to Jackson today which is about 80 miles

from where I live. The nearest town is vicksburg which is 30 miles

away(I,m a hick I live in the country). Vicksburg has only one small

neon shop, and I already paid him a visit, that is how I got the other 4

transformers. I found another much larger shop in Jackson, one of

several others. I asked the man in charge could I have some neon

transformers for a science project, and I told him I wanted only the bad

ones. He looked sort of puzzled for a moment then he asked me how many I

wanted. I told him how many you got? Well I've got a whole pile of them

out back and I'll charge you a dollar each. I told him okay. So one of

the other men that works in the back showed me a pile about three foot

high! He tells me if the boss don't see ya take as many as you want! So

I grabbed about 23 total. I could have gotten more but the minivans

springs were sitting kind of low! Heres what I grabbed.

13  MAGNATEK    15,000    30ma

4   FRANCEFORMER  15,000  30ma

1   MAGNATEK     15,000   60ma

1   JEFFERSON ELECTRIC 15,0000  30ma

1  TRANSCO  12,000  30ma

3   ?????   12,000  60ma

The last 3 transformers I can't identify but they have two large glass

socketed holes with springs inside on one end to connect to the neon

tubing.

And get this he said come back and get some more if you want( cross

my heart and hope to die)!

Frankensteins Helper

Max

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Wed, 02 Apr 1997 10:38:12 -0600

Subject: Re: How clean is clean enough?

At 02:16 AM 4/2/97 GMT, you wrote:

>From: Roderick Maxwell <major@vicksburg.com>

>Date: Tue, 01 Apr 1997 18:16:19 -0800

>Subject: How clean is clean enough?

>

>I have clean my 15kv neon secondaries so well you can see the stamped

>serial numbers on the paper covering. But when you look at the side of

>the secondaries you can still see tar between the winding layers in

>places. The question is do you have to remove all of the tar? The only

>way I believe you could do it would be to soak, and soak ,and soak ,them

>in solvent solvent until they came clean.

>

>

>

I would say you have your secondaries cleaner than any of mine.

If there are no large pieces hanging on then you're OK. Whats left

along the sides and between layers isn't worth bothering with.

Soaking the secondaries usually results in the outer layers of

paper insulation coming loose.

TTYL!

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Roderick Maxwell <major@vicksburg.com>

Date: Thu, 03 Apr 1997 19:46:51 -0800

Subject: Re: neon plunder!

> You hit the mother lode! Well done. I keep telling people

> they are out there you just have to go look.

>

> I would bet you will find some of those transformers to be just fine.

> I usually get about 50 percent good transformers out of a pile of

> "dead" ones.

>

> Now that you have a good source you can get selective. Go for the

> high current models. The 60 ma neons are much more rugged.

> If you find a 12kv 120ma unit don't use it! Send it immediately

> to me. :)

>

> TTYL!

> Mike Hammer

> mhammer@misslink.net

>

I hope I can get as many to work as I can! And don't think I did'nt

look for a 120ma job. Believe me If I had found one it would have been

the first one I would have snatched! There are a couple of other neon

shops in Jackson next time I'm over there I'm going to pay them a call

as well. One of which states in the telephone book to have been in

business since 1927. I wonder what he has lurking around in his scrap

pile!

Frankensteins Helper

Max

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: "Erik Hennix" <mr_white@hotmail.com>

Date: Fri, 04 Apr 1997 07:51:10 PST

Subject: Running TCs indoors

Hi everyone!

Just a few quick questions:

When running a TC indoors on a concrete floor how do you accomplish

a good RF ground?

Just how dangerous are the magnetic fields generated by a TC to a computer?

What is the minimum safe distance, assuming the computer is unplugged?

Cheers,

Erik.

---------------------------------------------------------

Get Your *Web-Based* Free Email at http://www.hotmail.com

---------------------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Fri, 04 Apr 1997 11:58:19 -0600

Subject: Re: Running TCs indoors

At 03:51 PM 4/4/97 GMT, you wrote:

>From: "Erik Hennix" <mr_white@hotmail.com>

>Date: Fri, 04 Apr 1997 07:51:10 PST

>Subject: Running TCs indoors

>

>Hi everyone!

>

>Just a few quick questions:

>

>When running a TC indoors on a concrete floor how do you accomplish

>a good RF ground?

Drill a hole in the floor! :)

Seriously I did just that. I run my coil in the basement and have

my ground rods driven through the basement floor into the ground.

I didn't have to drill the holes as they were already drilled

years ago when the house was treated for termites.

I end up with a good ground and a very short 5 foot long ground lead.

If that wont fly then run a HEAVY ground lead out a window

to a system of ground rods. You can use some heavy ring terminals

and a bolt to connect up the ground lead when you want to run the coil.

Try to get your ground lead as short and as straight as possible.

>

>Just how dangerous are the magnetic fields generated by a TC to a computer?

Very.

>What is the minimum safe distance, assuming the computer is unplugged?

I wouldn't have my computer running while the coil is running.

In fact I unplug my computer from the wall when coiling.

Don't forget the phone line to your modem. Voltages

can be induced in the line and damage your modem.

Better safe than sorry.

TTYL!

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Mon, 07 Apr 1997 23:21:56 -0500

Subject: Re: capacitors

At 06:14 PM 4/6/97 GMT, you wrote:

>From: Krumins  Aaron <krumins@bvsd.k12.co.us>

>Date: Sun, 6 Apr 1997 12:14:15 -0600 (MDT)

>Subject: capacitors

>

>I'm still in the process of collecting parts for my first Tesla coil and

>recently a friend told me I could pick up a .005 uF capacitor for about

>$.25 at any electrical shop. I was shocked to hear this for everyone who I

>had previously talked to said a .005 capacitor for a Tesla coil would be

>super expensive. My first thought was that it had to do with the high

>voltage required by the capacitor but after talking to the same friend he

>told me as long as the voltage of the capacitor was less then the output

>voltage of the transformer it would work. This didn't make a whole lot of

>sense to me because I would think it would be the other way around and the

>capacitor would need to have a higher voltage reading then the output of

>the transformer. Could someone clarify this for me?

>

>

Unfortunately your friend is incorrect.

Sure you can buy .005 caps for a quarter a piece or less

but these will not be AC rated pulse capable caps.

Small disc ceramic or mylar caps are plentiful and cheap

but these have DC voltage ratings and will not work with

Tesla coils.

Your primary capacitor should have an AC rating of at least

TWICE your transformer output voltage.

The way to go is to build your own cap using polyethylene

film and aluminum flashing or foil.

They are lots of work but worth it.

TTYL!

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: richard@anduril.demon.co.uk

Date: Mon, 07 Apr 1997 19:12:31 GMT

Subject: New member

Greetings,

As it seems to be customary to introduce yourself as a new member to

this list, here goes:

I live in the UK and I've been  interested in Tesla for a number of

years, though I cannot really say for certain what prompted my

interest.  Though seeing "plasma spheres" probably had something to do

with it - and wanting to find out how they worked.

I'm interested in making my own (very small) Tesla coil and I believe

I have the necessary expertise, though not the plans and practicval

knowledge.  I am a licenced radio amateur so I do have the theoretical

knowledge - I hope!

Most of the information I've found on Tesla coils is from the WWW and

is US in origin, is the fact I'm in the UK with its different mains

power pose any problems to building coils here?

At the moment I'm a complete beginner so I'm looking for something

very simple to get me started.

Richard

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: "Chad Westbrook" <emu@clarityconnect.com>

Date: Mon, 7 Apr 1997 16:00:18 -0400

Subject: capacitors

How do you make a cap out of that poly stuff and aluminum foil???

will a 1000v x-former work for any tesla related stuff jacobs ladders

plasme ect.

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Sat, 05 Apr 1997 19:21:28 -0600

Subject: Importance of RF grounding

Hello All

Several people have asked me recently about

RF grounds and why they are needed and why they are important.

I thought I would share a couple of thoughts with the group.

Here is a recent question.

>Here's a question that I have been wondering about for sometime.

>The question is why is a good ground so essential for Tesla coil work?

>Why won't a Tesla coil produce nearly as good a spark if it

>doesnt have a good ground?

OK, your secondary coil is generating a very large voltage at the

top of the coil. This is the location of the 1/4 wave voltage peak.

There is not a whole lot of current at the 1/4 wave location but

plenty of voltage. (we hope)

The base of the coil however is a "null" voltage point, or ground

potential. There is not a whole lot of voltage here, but RF current

is plentiful and heavy.

The top and bottom of the coil have inverse current/voltage relation-

ships. Top end high voltage, low current. Bottom end high current, low

voltage. Total energy output ( V*A ) must be equal from both ends.

The ability of the coil to process power requires that the current

at the base wire be removed as quickly and efficiently as possible.

If this current is not removed and a high impedance ground path

exists, then the coils ability to produce very high voltage is limited.

Also if this RF current is not removed via the shortest lowest

impedance path possible it will crawl along other wiring and conductors.

This "crawling" RF is a significant source of RF interference/contamination

as it will get into the house wiring via the 60 cycle ground path.

Comments criticism and cash donations welcome.

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Krumins  Aaron <krumins@bvsd.k12.co.us>

Date: Sun, 6 Apr 1997 12:14:15 -0600 (MDT)

Subject: capacitors

I'm still in the process of collecting parts for my first Tesla coil and

recently a friend told me I could pick up a .005 uF capacitor for about

$.25 at any electrical shop. I was shocked to hear this for everyone who I

had previously talked to said a .005 capacitor for a Tesla coil would be

super expensive. My first thought was that it had to do with the high

voltage required by the capacitor but after talking to the same friend he

told me as long as the voltage of the capacitor was less then the output

voltage of the transformer it would work. This didn't make a whole lot of

sense to me because I would think it would be the other way around and the

capacitor would need to have a higher voltage reading then the output of

the transformer. Could someone clarify this for me?

_____________________________________________

|Aaron Krumins                              |

|Devoted learner of computers i.e internet  |

|Krumins@bvsd.k12.co.us                     |

|                                           |

|"Silicone Valley or bust"                  |

--------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Tesla79@aol.com

Date: Mon, 7 Apr 1997 16:59:41 -0400 (EDT)

Subject: Re: capacitors

In a message dated 97-04-07 09:43:06 EDT, you write:

<< This didn't make a whole lot of sense to me because I would think it would

be the other way around and the capacitor would need to have a higher voltage

reading then the output of the transformer. Could someone clarify this for

me? >>

I am a beginner in Tesla coil building but I know some things about Tesla

coils.  First of all, the capicitor should always be at least three times the

voltage reading than the output of the transformer.  Why?  It's simple, the

Tesla coil operates by alternating the AC current electricity between the

capacitor and the transformer.  The only way a Tesla can operate efficiently

is if the capacitor can handle at least three times the voltage of the

transformer.  If you want a better, understanding of this phenomenon, go to

this hyperlink:  <A HREF="http://home.earthlink.net/~jimmypaul/descrpt.htm">Te

sla Coil Components & Theory</A>

I hope that cleared up your misconceptions of Tesla coils.  If you have more

questions, don't be shy to e-mail me.

A Fellow Tesla Coiler,

Joey

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Tue, 08 Apr 1997 08:31:51 -0500

Subject: Re: capacitors

At 08:59 PM 4/7/97 GMT, you wrote:

>From: Tesla79@aol.com

>Date: Mon, 7 Apr 1997 16:59:41 -0400 (EDT)

>Subject: Re: capacitors

>

>In a message dated 97-04-07 09:43:06 EDT, you write:

>

><< This didn't make a whole lot of sense to me because I would think it would

>be the other way around and the capacitor would need to have a higher voltage

>reading then the output of the transformer. Could someone clarify this for

>me? >>

>

>I am a beginner in Tesla coil building but I know some things about Tesla

>coils.  First of all, the capicitor should always be at least three times the

>voltage reading than the output of the transformer.  Why?  It's simple, the

>Tesla coil operates by alternating the AC current electricity between the

>capacitor and the transformer.  The only way a Tesla can operate efficiently

>is if the capacitor can handle at least three times the voltage of the

>transformer.

Not really true. It has really nothing to do with efficiency

but rather survivability.

The voltage rating on your transformer is RMS volts AC.

A capacitor charging on this transformer could charge

to 1.414 times this value. This is the peak value of the AC

sine wave.  And sometimes there can be voltage spikes above this.

The voltage to which your main cap will charge depends on the spacing

of your spark gap. The wider the gap the higher the voltage your

cap will charge. Very wide gaps will over stress your cap

and transformers. Especially neons.

The 3 times transformer cap rating just insures adequate safety margin

and is probably more than is necessary.

I usually recommend to use a cap rated at least twice the voltage

from your transformer.

I run only 12kv neons and my cap is rated conservatively at 24 KVAC.

A higher rating on the cap wont affect the coil output but may

add to the longevity of your cap especially if you open your gap too wide.

TTYL!

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: "steve pelet" <stevepel@axs2000.net>

Date: Tue, 8 Apr 1997 06:41:28 -0400

Subject: re:transformers

can someone tell me where I can get a 15kv neon sign transformer?

must be old and cheap

thanks

stevepel@axs2000.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: tesla@america.com (Bob Schumann)

Date: Tue, 8 Apr 1997 07:38:11 -0400 (EDT)

Subject: re:transformers

>From: "steve pelet" <stevepel@axs2000.net>

>Date: Tue, 8 Apr 1997 06:41:28 -0400

>Subject: re:transformers

>

>can someone tell me where I can get a 15kv neon sign transformer?

>must be old and cheap

>

>

>

>

>thanks

>stevepel@axs2000.net

>

Steve,

Look in the phone book's yellow pages under sign companies.

Mark down the ones that do neon. Go to the shops and ask if they

have any old transformers and let them know what you are using

them for. Being 'up' on the lingo seems to help for me. Know names

like 'Magnatek' (formerly Jefferson) and 'Franceformer'. This way

they will know that you have knowledge of transformers. Some shops

are hesitant to give a lethal output transformer to people who

do not know what they are doing. I find that if I tell them just

exactly what I am doing, that they most of the time will GIVE me a

few transformers or in worse case, charge a cheap price. After

you do this a few times and get friendly with the sign company

people, you will have 'contacts' to get free transformers. I have

so many stacked up on my porch now that I havent got time to get

to them and check them!!!!

Bob Schumann

tesla@america.com

http://www.america.com/~tesla

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Tue, 08 Apr 1997 23:54:36 -0500

Subject: re:transformers

At 11:38 AM 4/8/97 GMT, you wrote:

>From: tesla@america.com (Bob Schumann)

>Date: Tue, 8 Apr 1997 07:38:11 -0400 (EDT)

>Subject: re:transformers

>

>>From: "steve pelet" <stevepel@axs2000.net>

>>Date: Tue, 8 Apr 1997 06:41:28 -0400

>>Subject: re:transformers

>>

>>can someone tell me where I can get a 15kv neon sign transformer?

>>must be old and cheap

>>

>>

>>

>>

>>thanks

>>stevepel@axs2000.net

>>

>Steve,

>        Look in the phone book's yellow pages under sign companies.

>Mark down the ones that do neon. Go to the shops and ask if they

>have any old transformers and let them know what you are using

>them for. Being 'up' on the lingo seems to help for me. Know names

>like 'Magnatek' (formerly Jefferson) and 'Franceformer'. This way

>they will know that you have knowledge of transformers. Some shops

>are hesitant to give a lethal output transformer to people who

>do not know what they are doing. I find that if I tell them just

>exactly what I am doing, that they most of the time will GIVE me a

>few transformers or in worse case, charge a cheap price. After

>you do this a few times and get friendly with the sign company

>people, you will have 'contacts' to get free transformers. I have

>so many stacked up on my porch now that I havent got time to get

>to them and check them!!!!

>

>Bob Schumann

>

Steve,

Bob is exactly correct. This is the way to get transformers.

Even if some of the places don't have any old used transformers

around they will usually start saving them for you if you ask.

I have 4 neons shops within a 75 mile radius. I visit them all

periodically and take along a picture of my coil in operation.

You would be suprised the interest it creates. All 4 places

are saving any bad transformers they get for me.

One other thing is that you do not want a 15kv neon for a first coil.

The high voltage from these is hard on caps and gaps.

I recommend starting with 9kv neons. With the smaller neon you

won't need as high a rated cap. Some of the neon shops have told

me they don't even use 15kv neons anymore because its hard to keep

the wiring from arcing. Same goes for TC use.

When you start building that BIG coil and want all the power

you can get then the 15kv neons can be used to good advantage.

But stick with the lower voltage neons on a first coil. It will

make the whole thing easier to handle.

I use only 12kv neons myself and have all the power my coil can handle.

Best results so far are 38 inch streamers with 12kv/90ma neon.

TTYL!

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Tue, 08 Apr 1997 05:44:46 -0500

Subject: Re: capacitors

At 08:00 PM 4/7/97 GMT, you wrote:

>From: "Chad Westbrook" <emu@clarityconnect.com>

>Date: Mon, 7 Apr 1997 16:00:18 -0400

>Subject: capacitors

>

>

>How do you make a cap out of that poly stuff and aluminum foil???

Surf (I hate that term) on over to Kevin Nardell's

web site and have a look at both the Richard Hull and

Gary Weaver rolled cap files. They are basically the same

design with just a different approach. Many have made these

rolled caps and they are excellent performers.

An alternative design is to stack flat plates of poly and aluminum

and orient them vertically in a tank of oil.

Flat plate caps have very low inductance and better pulse

capability than rolled caps. Also the vertically oriented

plates allow the air to rise out of the stack.

I will attach a short file on my flat plate cap on the end

of this post.

>

>

>will a 1000v x-former work for any tesla related stuff jacobs ladders

>plasme ect.

>

No not really. Voltage is far too low for spark gap operation.

You need a minimum of 4500 volts or so and 6000 is much better.

Here is the file on my homebuilt flat plate cap.

Its not a step by step construction article. Just a general

guideline showing how I built my cap.

<begin include>

Flat Plate stacked Capacitor

Mike Hammer

mhammer@misslink.net

My first workable capacitor for a Tesla coil was a salt water

Leyden jar built with 12 oz beer bottles. While this did work

and I was able to get 14 inch arcs with a single 12kv 30ma neon

I could tell lots of energy was going to waste. Sometimes the corona

coming off the bottles was greater that that coming off the coil.

And the arcs from the coil were thin and purplish.

I read the files on building a rolled poly cap but could not find

a source of the 60 mil polyethylene. The best I could do locally

was 4 mil. Mail ordering the plastic from a distributor was way

too expensive.

I decided to use the 4 mil poly in a plate type cap.

What worked for me was to cut out the plates with a

sharp pair of scissors and include an extended tab to use for the

buss connection:

___

|   |

|   |

|   |

/---------------------|   |

/                           |

|                           |

|      ALUMINUM FOIL        |

|     CAPACITOR PLATE       |

|                           |

|                           |

|                           |

|                           |

|                           |

|                           |

\                         /

\----------------------/

These plates were 6 x 8 inches and the tab was created by cutting

a 5 x 2 rectangle from the top of the plate. This created a tab that

was 1 inch wide by 2 inches long. The capacitive area of the plate was

6 x 6 inches. I chose these dimensions to make best use of the available

foil which came in a 12 inch wide roll. I used Reynolds Heavy duty foil as

it is easier to work with than the standard variey.

Multiple plates can be cut out at the same time by stacking

sheets of foil one atop another and cutting several plates at

once with the scissors. The plates are then separated and each

plate is inspected for flaws. All the plates may be cut with the

same template. Be sure to round off the sharp corners to help prevent

corona. When the capacitor stack is built, you simply

reverse the orientation of every other plate in the stack

so that the tabs extend out from opposite sides of the capacitor.

All the left hand side tabs are then bundled together to form one

buss, the right hand tabs get the same treatment:

___                     ___

|   |                   |   |

BUSS -->  |   |                   |   |  <--BUSS

|   |                   |   |

| /-|-------------------|\  |

/                          \|

|                           |

|         STACK OF          |

|       ALUMINUM FOIL       |

|     CAPACITOR PLATES      |

|                           |

|                           |

|                           |

|                           |

|                           |

\                         /

\----------------------/

For the dielectric I used 7x7 sheets of 4 mil polyethylene. I bought

a roll of this from a local hardware outlet quite cheaply. (it was on sale)

This is the plastic sold for landscaping purposes. Be aware that inexpensive

poly like this is often made with reclaimed and or scrap material. I found

numerous imperfections and pieces of grit imbedded in the plastic. Inspect

it carefully and work around the flaws. Reject any pieces with a visible flaw.

I used two pieces for each dielectric layer for a total of 8 mils.

I created a stack of alternating layers of foil and poly with a total of 24

plates of foil and 48 sheets of poly. The most difficult part here is trying

to keep all the plates and plastic sheets lined up. The poly is slick and

wants to

slide around a lot.This made a high value cap of approximately 0.048 mfd.

I created 6 of this units. I sandwiched each one between 7 x7 sheets of

plexiglass. Bind each unit up with some strips of strong packing tape. Do

not block

up the edges as they need to be open so the oil can penetrate the layers.

I wired up the 6 individual units in series for a higher working voltage. This

gave a total capacitance of approximately .008 mfd. This was put into a 1.25

gallon

rubbermaid container. Wedge some foam material into the tank to hold to plates

firmly in position. Better to use some type of closed cell foam so as not

to introduce too much air into the unit. I also cut some wood spacers to wedge

the plates in. I mounted a 1/4 20 bolt in each end of the tank for the

electrical

connections. I sealed the holes with discs of plexiglass and RTV. Fill the work

s

with mineral oil or transformer oil. You must allow the completed cap to sit

and bubble out the trapped air. This can take a while. I had planned on waiting

a whole week but got impatient. After 4 days I fired it up at very low power on

my coil.

I used a 7500 volt neon and kept the gap very narrow. I was getting 4 inch

sparks

to a grounded rod held close to the terminal at this point. I ran it for 2 days

at this power level. After every run (short 30 sec runs) more air would bubble

out. The pulsing from the Tesla tank circuit helps to dislodge air bubbles.

In succeeding days I gradually upped the power. I used next a 9kv neon then

a 12.

I also slowly opened up the gap. Once fully peaked I was getting 16 inches with

a single 12kv 30 ma neon.

I have really put the bricks this cap over the past few months using up to

12kv at 100ma worth of neons. Best discharges so far have been 34 inches.

I have not used this with any 15kv transformers as I feel that would be in

excess

of the rating. It will take it all at 12 kv. Using thicker dielectric (maybe 3

layers of 4 mil) or more units in series would be required for 15 kv operation.

I have included some capacitor formulas that may be useful.

=======================================================================

CAPACITORS IN PARALLEL:

|      |

|__C1__|

|      |

|__C2__|

|      |

|__C3__|

|      |

Capacitance = C1 + C2 + C3, etc...

Maximum voltage rating will be equal to the voltage rating of the

lowest voltage capacitor of the group.

=======================================================================

CAPACITORS IN SERIES:

|              |

|__C1__C2__C3__|

|              |

|              |

1       1       1

Capacitance = 1 /[---] + [---] + [---], etc...

C1      C2      C3

The total capacitance of several capacitors in series will always be

LESS than that of the smallest capacitor.

Total voltage rating increases with number of capacitors in series.

When capacitors are placed in series to increase voltage rating they

should have the same capacitance and voltage rating else voltages will

divide unevenly, most likely causing failure.

=======================================================================

PLATE TYPE CAPACITORS

Capacitance (in picofarads) = (0.224 KA / d) (N-1)

0.224 x Dielectric Strength x Area of plate

C = -----------------------------------------  x (Number of plates - 1)

distance between plates in inches

=======================================================================

Note:  Dielectric strength of polyethylene is 2.2. It will usually end up

closer to 2.0 in homebuilt caps.

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Krumins  Aaron <krumins@bvsd.k12.co.us>

Date: Tue, 8 Apr 1997 18:21:40 -0600 (MDT)

Subject: Transformers

I am working on my first tesla coil and recently found a 12KV 30Ma neon

sign transformer for $18 and a 9KV 30Ma neon transformer for $20. Are these

good deals and if so which of them would you suggest for powering my first

coil.

_____________________________________________

|Aaron Krumins                              |

|Devoted learner of computers & the iternet |

|Krumins@bvsd.k12.co.us                     |

|                                           |

|"Silicone Valley or bust"                  |

--------------------------------------------

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Roderick Maxwell <major@vicksburg.com>

Date: Tue, 08 Apr 1997 18:47:57 -0700

Subject: pow!

That new neon I just rebuilt gave me a scare last night! I hooked it up

to a piece of neon tubing and decided to give a extended run. It ran

about 5 min. and then let out a loud pop accompanied by an arc. But the

neon continued to operate. I quickly unplugged it and began to inspect

it for damage, finding none I disconected the tubing and plugged it back

in. Instantly arcs shot from both secodaries to the core! I quickly

unplugged it again.

Then I realized what happened: As long as the transformer was

connected to the neon tube it had a load, and the voltage dropped. But

when there was no load (neon tube disconnected) the voltage was at

maximum and the secondaries arced to the core which is at ground

potential. To correct this problem I have cut spacers made of the same

1/8" poly that I made the caps from, and gooped on silicone rubber to

hold the spacers in place as well as add further insulation. The next

one I rebuild will have these spacers as well But I will skip the rubber

and sink the whole thing in an oil tank. I put the spacers on both sides

of each secondary.

POLY SPACER

_____________________

|                   |

|      ______       |

|     |       |     |

|     | core  |     |

|     | hole  |     |

|     |       |     |

|     | ______|     |

|                   |

|                   |

|___________________|

Frankensteins Helper

Max

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Wed, 09 Apr 1997 08:33:49 -0500

Subject: Re: Transformers

At 12:21 AM 4/9/97 GMT, you wrote:

>From: Krumins  Aaron <krumins@bvsd.k12.co.us>

>Date: Tue, 8 Apr 1997 18:21:40 -0600 (MDT)

>Subject: Transformers

>

>I am working on my first tesla coil and recently found a 12KV 30Ma neon

>sign transformer for $18 and a 9KV 30Ma neon transformer for $20. Are these

>good deals and if so which of them would you suggest for powering my first

>coil.

>

>

>

Those are OK deals. Not great deals but typical prices.

The 9kv/30 will be the best bet for a first coil.

If you can get a pair of these that would be best.

Thats gives you a spare and you can parallel them for more

power later.

If they only have those 2 transformers then take them both

if you can afford it. Start with the 9kv neon and move

up to the 12kv neon after things are running well.

You cannot parallel different voltage neons.

TTYL!

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: "steve pelet" <stevepel@axs2000.net>

Date: Tue, 8 Apr 1997 21:42:29 -0400

Subject: Re: re:transformers

could someone please send one to me I realy need 15kv. I am aware it is

alot of ware and tear on capacitors but.........

I if anyone could send one to me I would gladly pay a resonable price

I have check in a 50 mi. radius and the shops either don't have them or

don't have the old ones. I talked to a shop that tried to sell one to me

for $180.00 for 15kv. And I am not going to pay that much.

so if anyone could send one to me or give me the phone # to a place that

you got yours (they could mail it to me) I would be delighted

thanks

stevepel@axs2000.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Mike Hammer <mhammer@misslink.net>

Date: Wed, 09 Apr 1997 08:43:33 -0500

Subject: Re: pow!

At 01:47 AM 4/9/97 GMT, you wrote:

>From: Roderick Maxwell <major@vicksburg.com>

>Date: Tue, 08 Apr 1997 18:47:57 -0700

>Subject: pow!

>

>That new neon I just rebuilt gave me a scare last night! I hooked it up

>to a piece of neon tubing and decided to give a extended run. It ran

>about 5 min. and then let out a loud pop accompanied by an arc. But the

>neon continued to operate. I quickly unplugged it and began to inspect

>it for damage, finding none I disconected the tubing and plugged it back

>in. Instantly arcs shot from both secodaries to the core! I quickly

>unplugged it again.

>   Then I realized what happened: As long as the transformer was

>connected to the neon tube it had a load, and the voltage dropped. But

>when there was no load (neon tube disconnected) the voltage was at

>maximum and the secondaries arced to the core which is at ground

>potential. To correct this problem I have cut spacers made of the same

>1/8" poly that I made the caps from, and gooped on silicone rubber to

>hold the spacers in place as well as add further insulation. The next

>one I rebuild will have these spacers as well But I will skip the rubber

>and sink the whole thing in an oil tank. I put the spacers on both sides

>of each secondary.

>

>

>

>                                   POLY SPACER

>                                _____________________

>                                |                   |

>                                |      ______       |

>                                |     |       |     |

>                                |     | core  |     |

>                                |     | hole  |     |

>                                |     |       |     |

>                                |     | ______|     |

>                                |                   |

>                                |                   |

>                                |___________________|

>

>

>

>                               Frankensteins Helper

>                                       Max

Thats how I test my rebuilt neons. I crank in 140 VAC into the primary

and leave the secondary open. This creates maximum voltage

on the secondary and if its gonna blow up that will usually do it.

Try some of that rope caulking compound as insulation.

I have had good luck with it. Just fill up the space between the

core and secondary with the stuff. And this stuff is cheap.

A box with enough for 2 transformers cost me $1.75. The 2 neons I've

used this on have not failed me so far despite being beat pretty hard.

I overvolt all my rebuilt neons with 140 VAC input. Make em sweat.

TTYL!

Mike Hammer

mhammer@misslink.net

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: "R+S Gordon" <ritchcom@execulink.com>

Date: Wed, 9 Apr 1997 21:46:06 -0400

Subject: mylar film

Does anyone know if mylar film is acceptable for capacitor construction?

I realize that it has sufficient voltage withstandind properties but I

wondered about rf dissipation and dielectric constant.  I have a lead on

some fairly inexpensive mylar which is <$40 for a roll 42" by 50 feet.

There are two types available known as A-type and B-type.  Any help on this

matter would be greatly appreciated.

Ritch Gordon

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4

From: Kevin Batchelor <KBATCHELOR@arslimited.com>

Date: Thu, 10 Apr 1997 12:58:34 -0400

Subject: mylar film -Reply

I would stay with the polyethylene sheeting.  You can get a large roll for

less than $10 and it will work fine.  I'll admit that building my cap took

forever, but it should hold up to some beating.  Hope to have my coil

online by this weekend.

Kevin

KBATCHELOR@ARSLIMITED.COM

* Send list Replies to: Tesla-2@emachine.com

* To UNSUBSCRIBE from this list, put the command UNSUBSCRIBE TESLA-2

in the body of your message to: Listserv@Emachine.com

-----

Processed with Listserv v2.93 for Wildcat v4