Technical description of the Topographic Database

Some extra features also on 1:50 000 and 1:100 000 databases.

Features included: transport network, transmission networks, terrain1, terrain2, man-made structures, elevation, special-purpose areas, conservation areas and administrative division.

| Feature | attribute | name in export file | value | legend |
| --- | --- | --- | --- | --- |
| transport line | feature group | RYHMÄ | 25 | transport lines |
|  | class | LUOKKA | values |  |
|  | attributes common to all lines |  |  |  |
| transport point | feature group | RYHMÄ | 45 | transport points |
|  | class | LUOKKA | values |  |
|  | attributes common to all points |  |  |  |
| transport text | Feature group | RYHMÄ | 55 | transport texts |
|  | class | LUOKKA | values |  |
|  | attributes common to all texts |  |  |  |
| transmission line | Feature group | RYHMÄ | 28 | transmission lines |
|  | class | LUOKKA | values |  |
|  | attributes common to all lines |  |  |  |
| transmission point | Feature group | RYHMÄ | 48 | transmission points |
|  | class | LUOKKA | values |  |
|  | attributes common to all points |  |  |  |
| transmission text | Feature group | RYHMÄ | 58 | transmission texts |
|  | class | LUOKKA | 26202 | legend for water intake plant |
|  | attributes common to all texts |  |  |  |
| terrain1-area | Feature group | RYHMÄ | 64 | terrain1-areas |
|  | class | LUOKKA | values |  |
|  | attributes common to all areas |  |  |  |
| terrain1-line | Feature group | RYHMÄ | 19 | terrain1-lines |
|  | class | LUOKKA | values |  |
|  | attributes common to all lines |  |  |  |
| terrain1-point | Feature group | RYHMÄ | 13 | terrain1-points |
|  | class | LUOKKA | values |  |
|  | attributes common to all points |  |  |  |
| terrain1-text | Feature group | RYHMÄ | 16 | terrain1-texts |
|  | class | LUOKKA | values |  |
|  | attributes common to all texts |  |  |  |
| terrain2-area | Feature group | RYHMÄ | 70 | terrain2-areas |
|  | class | LUOKKA | values |  |
|  | attributes common to all areas |  |  |  |
| terrain2-line | Feature group | RYHMÄ | 22 | terrain2-lines |
|  | class | LUOKKA | values |  |
|  | attributes common to all lines |  |  |  |
| terrain2-point | Feature group | RYHMÄ | 42 | terrain2-points |
|  | class | LUOKKA | values |  |
|  | attributes common to all points |  |  |  |
| terrain2-text | Feature group | RYHMÄ | 52 | terrain2-texts |
|  | class | LUOKKA | values |  |
|  | attributes common to all texts |  |  |  |
| man-made structures as areas | Feature group | RYHMÄ | 75 | man-made structure-areas |
|  | class | LUOKKA | values |  |
|  | attributes common to all areas |  |  |  |
| man-made structures as lines | Feature group | RYHMÄ | 27 | man-made structure-lines |
|  | class | LUOKKA | values |  |
|  | attributes common to all lines |  |  |  |
| man-made structure-point | Feature group | RYHMÄ | 47 | man-made structure-points |
|  | class | LUOKKA | values |  |
|  | attributes common to all points |  |  |  |
| man-made structure-text | Feature group | RYHMÄ | 57 | man-made structure-texts |
|  | class | LUOKKA | values |  |
|  | attributes common to all texts |  |  |  |
| elevation line | Feature group | RYHMÄ | 20 | elevation lines |
|  | class | LUOKKA | 52100 | contour |
|  |  |  | 54100 | depth contour |
|  | attributes common to all lines |  |  |  |
| elevation point | Feature group | RYHMÄ | 14 | elevation points |
|  | class | LUOKKA | 52192 | contour gradient |
|  |  |  | 54192 | depth contour gradient |
|  | attributes common to all points |  |  |  |
| elevation text | Feature group | RYHMÄ | 17 | elevation texts |
|  | class | LUOKKA | values |  |
|  | attributes common to all texts |  |  |  |
| special-purpose areas | Feature group | RYHMÄ | 77 | special-purpose areas |
|  | class | LUOKKA | 62200 | restricted area |
|  | attributes common to all areas |  |  |  |
| special-purpose area line | Feature group | RYHMÄ | 29 | special-purpose area lines |
|  | class | LUOKKA | 62100 | military command area |
|  |  |  | 62200 | restricted area boundary line |
|  | attributes common to all lines |  |  |  |
| special-purpose area text | Feature group | RYHMÄ | 59 | special-purpose area texts |
|  | class | LUOKKA | 62102 | legend for military command areas |
|  | class | LUOKKA | 62202 | legend for restricted area |
|  | attributes common to all texts |  |  |  |
| conservation area | Feature group | RYHMÄ | 74 | conservation areas |
|  | class | LUOKKA | values |  |
|  | attributes common to all areas |  |  |  |
| conservation area line | Feature group | RYHMÄ | 26 | conservation area lines |
|  | class | LUOKKA | values |  |
|  | attributes common to all lines |  |  |  |
| conservation area point | Feature group | RYHMÄ | 46 | conservation area points |
|  | class | LUOKKA | values |  |
|  | attributes common to all points |  |  |  |
| conservation area text | Feature group | RYHMÄ | 56 | conservation area texts |
|  | class | LUOKKA | values |  |
|  | attributes common to all texts |  |  |  |
| administrative division area | Feature group | RYHMÄ | 71 | administrative division areas |
|  | class | LUOKKA | 84200 | municipality |
|  | attributes common to all areas |  |  |  |
| administrative division line | Feature group | RYHMÄ | 23 | administrative division lines |
|  | class | LUOKKA | values |  |
|  | attributes common to all lines |  |  |  |
| administrative division point | Feature group | RYHMÄ | 43 | administrative division points |
|  | class | LUOKKA | 82500 | state boundary mark |
|  | attributes common to all points |  |  |  |
| administrative division text | Feature group | RYHMÄ | 53 | administrative division texts |
|  | class | LUOKKA | values |  |
|  | attributes common to all texts |  |  |  |

Assignment formats and nomination

| format: | nomination: |
| --- | --- |
| maagisxl | {k¦a}xxxxxx.mxl |
| maagis | kxxxxxx.mgs |
| maagis2 | kxxxxxx.mg2 |
| fingis | kxxxxxx.fgs |
| Mapinfo Interchange Format | w{k¦a}xxxxxx.mif |
|  | w{k¦a}xxxxxx.mid |
| ESRI Shape | lxxxxxx{v¦s¦t}.dbf |
|  | lxxxxxx{v¦s¦t}.shp |
|  | lxxxxxx{v¦s¦t}.shx |
|  | jxxxxxx{v¦s¦t}.dbf |
|  | jxxxxxx{v¦s¦t}.shp |
|  | jxxxxxx{v¦s¦t}.shx |
|  | mxxxxxx{v¦s¦t¦p}.dbf |
|  | mxxxxxx{v¦s¦t¦p}.shp |
|  | mxxxxxx{v¦s¦t¦p}.shx |
|  | nxxxxxx{v¦s¦t¦p}.dbf |
|  | nxxxxxx{v¦s¦t¦p}.shp |
|  | nxxxxxx{v¦s¦t¦p}.shx |
|  | rxxxxxx{v¦s¦t¦p}.dbf |
|  | rxxxxxx{v¦s¦t¦p}.shp |
|  | rxxxxxx{v¦s¦t¦p}.shx |
|  | kxxxxxx{v¦s¦t}.dbf |
|  | kxxxxxx{v¦s¦t}.shp |
|  | kxxxxxx{v¦s¦t}.shx |
|  | sxxxxxx{v¦s¦t¦p}.dbf |
|  | sxxxxxx{v¦s¦t¦p}.shp |
|  | sxxxxxx{v¦s¦t¦p}.shx |
|  | exxxxxx{v¦s¦t¦p}.dbf |
|  | exxxxxx{v¦s¦t¦p}.shp |
|  | exxxxxx{v¦s¦t¦p}.shx |
|  | hxxxxxx{v¦s¦t¦p}.dbf |
|  | hxxxxxx{v¦s¦t¦p}.shp |
|  | hxxxxxx{v¦s¦t¦p}.shx |

| xxxxxx | map sheet number (if demarcation is by sheet number) |
| --- | --- |
|  | municipality number (3 symbols) (if demarcation is by municipality) |
|  | TRAIL (if some other means of demarcation) |
| a | by area |
| k | by feature (lines, area boundaries, symbols and texts) |
| v | lines |
| s | (symbol) points |
| t | texts |
| p | polygons |
| l | Transport networks |
| j | Transmission networks |
| m | Terrain/1 |
| n | Terrain/2 |
| r | Man-made structures |
| k | Elevation |
| s | Protected areas |
| e | Special-purpose areas |
| h | Administrative division |

### Attributes common to all lines

| attribute | name in export file | value | legend |
| --- | --- | --- | --- |
| Positional accuracy class code | TASTAR | level precision |  |
| Vertical positioning accuracy class code | KORTAR | vertical positioning accuracy |  |
| Elevation value | KORARV |  |  |
| Course of the line | KULKUTAPA | 1 | Broken line |
|  |  | 2 | Curve |
|  |  | 4 | Un-thickened curve |
| rwo_id | KOHDEOSO |  |  |
| Data source code | AINLAHDE | 0 | Other |
|  |  | 1 | National Land Survey |
| Start date | SYNTYHETKI |  |  |
| End date | KUOLHETKI |  |  |
| Cartographic classing code | KARTOGLK |  |  |
| Regional division code | ALUEJAKOON | 0 | Included in regional division |
|  |  | 1 | Is not included in regional division |
| Vertical status code | VERSUH | -1 | Below the surface |
|  |  | 0 | On the surface |
|  |  | 1 | Above the surface |
|  |  | 10 | Unspecified |

### Attributes common to all points

| attribute | Name in the export file | value | legend |
| --- | --- | --- | --- |
| Positional accuracy class code | TASTAR | level precision |  |
| Vertical positioning accuracy class code | KORTAR | vertical positioning accuracy |  |
| rwo_id | KOHDEOSO |  |  |
| Data source code | AINLAHDE | 0 | Other |
|  |  | 1 | National Land Survey |
| Start date | SYNTYHETKI |  |  |
| End date | KUOLHETKI |  |  |
| Bearing of symbol (1/10000-part as radians) | SUUNTA |  |  |
| Eastward displacement of symbol (mm) | SIIRT_DX |  |  |
| Northward displacement of symbol (mm) | SIIRT_DY |  |  |

### Attributes common to all texts

| attribute | Name in the export file | value | Legend |
| --- | --- | --- | --- |
| Text | TEKSTI |  | max. 80 symbols |
| Positional accuracy class code | TASTAR | level precision |  |
| rwo_id | KOHDEOSO |  |  |
| Data source code | AINLAHDE | 0 | Other |
|  |  | 1 | National Land Survey |
| Start date | SYNTYHETKI |  |  |
| End date | KUOLHETKI |  |  |
| Bearing of text (1/10000-part as radians) | SUUNTA |  |  |
| Eastward displacement of text (mm) | SIIRT_DX |  |  |
| Northward displacement of text (mm) | SIIRT_DY |  |  |
| Typesetting code | ATTR1 | typesetting codes |  |

### Attributes common to all areas

| attribute | Name in export file | value | Legend |
| --- | --- | --- | --- |
| Positional accuracy class code | TASTAR | level precision |  |
| rwo_id | KOHDEOSO |  |  |
| Data source code | AINLAHDE | 0 | Other |
|  |  | 1 | National Land Survey |
| Start date | SYNTYHETKI |  |  |
| End date | KUOLHETKI |  |  |

### Values for transport lines

| Value | Legend |
| --- | --- |
| 12111 | road Ia = a motorway roadway with two or more lanes |
| 12112 | road Ib = a roadway with two or more lanes of other way than motorway or a a road with one roadway with two or more lanes, width of the roadway more than 8 meters |
| 12121 | road IIa = a road with one roadway with two lanes, width of the roadway 6,5 - 8 meters |
| 12122 | road IIb = a road with one roadway with two lanes, width of the roadway 5 - 6,5 meters |
| 12131 | road IIIa = a road with one roadway with one lane, width of the roadway 4 - 5 meters |
| 12132 | road IIIb = a road with one roadway with one lane, width of the roadway 4 - 5 meters |
| 12141 | roadway = a road with one roadway with one lane, width of the roadway < 3 meters |
| 12151 | ferry |
| 12152 | small ferry |
| 12311 | old track |
| 12312 | winter road |
| 12314 | pedestrian and bicycle way |
| 12316 | track |
| 14100 | railway, unclassified |
| 14111 | electrified railway |
| 14112 | non-electrified railway |
| 14121 | narrow-gauge railway |
| 14131 | metro |
| 14141 | 1:50 000 railway, 1 track |
| 14142 | 1:50 000 railway, 2 tracks |
| 14143 | 1:50 000 railway, 3 tracks |
| 14144 | 1:50 000 railway, 4 tracks |
| 14151 | 1:50 000 electrified railway, 1track |
| 14152 | 1:50 000 electrified railway, 2 tracks |
| 14153 | 1:50 000 electrified railway, 3 tracks |
| 14154 | 1:50 000 electrified railway, 4 tracks |
| 14161 | 1:50 000 railway, short track |
| 16500 | waterway, unclassified |
| 16511 | channel |
| 16512 | boating route |
| 16521 | channel line |
| 16522 | boating route line |

### Values for transport points

| Value | Legend |
| --- | --- |
| 12192 | ferry |
| 12193 | small ferry |
| 12200 | obstruction |
| 12391 | light traffic route underpass |
| 12500 | motor traffic site |
| 14191 | railway electrification symbol |
| 14192 | symbol for closed railway |
| 14193 | 1:50 000 railway electrification symbol, 1 track |
| 14194 | 1:50 000 railway electrification symbol, 2 tracks |
| 14195 | 1:50 000 railway electrification symbol, 3 tracks |
| 14196 | 1:50 000 railway electrification symbol, 4 tracks |
| 14300 | railway kilometre post |
| 14200 | railway operating point |
| 14210 | 1:100 000 main railway station |
| 14211 | 1:100 000 other railway station |
| 14212 | 1:100 000 operating railway point |
| 16800 | tunnel opening |
| 16600 | berth |
| 16700 | wreck, unclassified |
| 16712 | wreck, on the surface |
| 16721 | wreck, below the surface / depth unknown |
| 16722 | wreck, below the surface / depth known |
| 16505 | channel, nominal direction of traffic |
| 16506 | channel, recommended direction of traffic |
| 16507 | channel, enforced direction of traffic |
| 16100 | navigational aid, unclassified |
| 16120 | leading beacon |
| 16121 | cairn |
| 16122 | beacon tower |
| 16124 | light |
| 16125 | leading light |
| 16126 | lighthouse |
| 16127 | radar marker |
| 16128 | lighted radar marker |
| 16141 | navigation mark – left |
| 16142 | navigation mark – right |
| 16143 | navigation mark – north |
| 16144 | navigation mark – south |
| 16145 | navigation mark – east |
| 16146 | navigation mark – west |
| 16147 | navigation mark – rock |
| 16148 | navigation mark – safe |
| 16149 | navigation mark – special |
| 16151 | spar buoy-left |
| 16152 | spar buoy – right |
| 16153 | spar buoy – north |
| 16154 | spar buoy – south |
| 16155 | spar buoy – east |
| 16156 | spar buoy – west |
| 16157 | spar buoy – rock |
| 16158 | spar buoy – safe |
| 16159 | spar buoy – special |
| 16161 | buoy – left |
| 16162 | buoy – right |
| 16163 | buoy – north |
| 16164 | buoy – south |
| 16165 | buoy – east |
| 16166 | buoy – west |
| 16167 | buoy – rock |
| 16168 | buoy – safe |
| 16169 | buoy – special |
| 16171 | lighted buoy- left |
| 16172 | lighted buoy – right |
| 16173 | lighted buoy – north |
| 16174 | lighted buoy – south |
| 16175 | lighted buoy – east |
| 16176 | lighted buoy – west |
| 16177 | lighted buoy – rock |
| 16178 | lighted buoy – safe |
| 16179 | lighted buoy – special |
| 16181 | ice buoy – left |
| 16182 | ice buoy – right |
| 16183 | ice buoy – north |
| 16184 | ice buoy – south |
| 16185 | ice buoy – east |
| 16186 | ice buoy – west |
| 16187 | ice buoy – rock |
| 16188 | ice buoy – safe |
| 16189 | ice buoy – special |
| 16191 | lighted ice buoy- left |
| 16192 | lighted ice buoy – right |
| 16193 | lighted ice buoy – north |
| 16194 | lighted ice buoy – south |
| 16195 | lighted ice buoy – east |
| 16196 | lighted ice buoy – west |
| 16197 | lighted ice buoy – rock |
| 16198 | lighted ice buoy – safe |
| 16199 | lighted ice buoy – special |
| 16201 | edge mark- left |
| 16202 | edge mark – right |
| 16203 | edge mark – north |
| 16204 | edge mark – south |
| 16205 | edge mark – east |
| 16206 | edge mark – west |
| 16211 | lighted edge mark – left |
| 16212 | lighted edge mark – right |
| 16213 | lighted edge mark – north |
| 16214 | lighted edge mark – south |
| 16215 | lighted edge mark – east |
| 16216 | lighted edge mark – west |

### Values for transport texts

| Value | Legend |
| --- | --- |
| 10111 | free height |
| 12101 | road name |
| 12105 | road bridge number |
| 12106 | road ferry number |
| 12181 | local road number |
| 12182 | main road number |
| 12183 | E-, trunk- or principle road number |
| 12191 | road number |
| 12301 | light-traffic route name |
| 12302 | light-traffic route legend |
| 14201 | railway traffic site name |
| 14311 | railway kilometre post reading |
| 16101 | navigational aid name |
| 16102 | navigational aid legend |
| 16503 | channel bed level (2.2mm 1:20 000) |
| 16504 | channel bed level (1.8mm 1:20 000) |
| 16508 | channel underkeel clearance |
| 16703 | depth of wreck |

### Values for transmission lines

| Value | Legend |
| --- | --- |
| 22200 | transformer substation |
| 22310 | power line, unclassified |
| 22311 | power line, unknown |
| 22312 | power line, distribution voltage |
| 24100 | telephone line |
| 26100 | unclassified pipe line |
| 26111 | gas pipe |
| 26112 | solid substance pipe |
| 26113 | heating pipe |
| 26114 | water pipe |
| 26115 | steam pipe |
| 26116 | sewer |
| 26117 | oil pipe |

### Values for transmission points

| Value | Legend |
| --- | --- |
| 22100 | transformer |
| 22392 | high-voltage power line pylon |
| 24200 | emergency telephone |
| 26190 | pipe symbol, unclassified |
| 26191 | gas pipe symbol |
| 26192 | solid substance pipe symbol |
| 26193 | heating pipe symbol |
| 26194 | water pipe symbol |
| 26195 | steam pipe symbol |
| 26196 | drain pipe symbol |
| 26197 | oil pipe symbol |
| 26200 | water intake plant |

### Terrain1-area

| Value | Legend |
| --- | --- |
| 32100 | mineral resources extraction area, unclassified |
| 32111 | mineral resources extraction area, course-grained material |
| 32112 | mineral resources extraction area, fine-grained material |
| 32113 | mineral resources extraction area, organic material |
| 32200 | cemetery |
| 32300 | landfill |
| 32400 | airfield area |
| 32411 | airfield runway, paved |
| 32412 | airfield runway, unpaved |
| 32413 | other airfield area |
| 32414 | other air transport area |
| 32415 | other airfield area, paved |
| 32416 | other airfield area, unpaved |
| 32417 | other air traffic area, paved |
| 32418 | other air traffic area, unpaved |
| 32421 | motor traffic site |
| 32431 | shunting yard |
| 32500 | quarry |
| 32600 | cultivated land, unclassified |
| 32611 | cultivated land, field |
| 32612 | cultivated land, garden |
| 32800 | meadow |
| 32900 | park |
| 33000 | infill |
| 33100 | infill |
| 34100 | sport and recreation area |
| 34300 | rock exposure |
| 34700 | sandy bed |
| 35300 | boulders |
| 35400 | mire, unclassified |
| 35411 | mire, passable, treeless |
| 35412 | mire, passable, forest growing |
| 35421 | mire, difficult terrain, treeless |
| 35422 | mire, difficult terrain, forest growing |
| 36200 | water with no flow |
| 36211 | sea water |
| 36313 | flowing watercourse, areal |

### Terrain1-line

| Value | Legend |
| --- | --- |
| 30300 | dam |
| 30400 | lock gate |
| 34400 | escarpment |
| 34800 | slope |
| 34500 | fissure |
| 35200 | avenue of trees |
| 35500 | mire string |
| 30100 | artificial shoreline |
| 30200 | unclassified boundary line |
| 30211 | other definite boundary line |
| 30212 | indefinite boundary line |
| 30900 | water area boundary line |
| 30999 | technical boundary line |
| 36300 | flowing watercourse, unclassified |
| 36311 | flowing watercourse, width under 2m |
| 36312 | flowing watercourse, width 2.5m |
| 36314 | 1:100 000 flowing watercourse, width 5-20m |

### Terrain1-point

| Value | Legend |
| --- | --- |
| 32191 | organic substance |
| 32192 | fine-grained material |
| 32193 | coarse-grained material |
| 32291 | cemetery symbol |
| 32591 | quarry symbol |
| 32891 | meadow symbol |
| 33091 | infill symbol |
| 32720 | prepared forestland, unclassified |
| 32721 | drained area |
| 32722 | trenched area |
| 32700 | forestland vegetation, unclassified |
| 32710 | coniferous forest |
| 32713 | deciduous forest |
| 32714 | mixed forest |
| 32715 | brush |
| 32719 | bushland |
| 34100 | rock exposure symbol |
| 34200 | sparse boulder field |
| 34600 | stone |
| 34900 | natural site of special interest |
| 35100 | tree |
| 36100 | spring |
| 36391 | arrow showing direction of flow |
| 36400 | water pit |

### Terrain1-text

| Value | Legend |
| --- | --- |
| 32101 | name of mineral resources extraction area |
| 32102 | legend for mineral resources extraction area |
| 32201 | cemetery name |
| 32202 | cemetery legend |
| 32301 | landfill name |
| 32302 | landfill legend |
| 32401 | name of motor traffic site |
| 32402 | legend for motor traffic site |
| 32501 | quarry name |
| 32502 | quarry legend |
| 32901 | park name |
| 32902 | park legend |
| 33001 | infill name |
| 33002 | infill legend |
| 33101 | sport and recreation area name |
| 33102 | sport and recreational area legend |
| 34601 | stone name |
| 34901 | name of natural site of special interest |
| 34902 | legend of natural site of special interest |
| 35010 | meadow or field name |
| 35020 | forest area name |
| 35030 | mire name |
| 35040 | elevation name |
| 35050 | depression name |
| 35060 | peninsula name |
| 35070 | island name |
| 35080 | shoal name |
| 35090 | other terrain name |
| 35101 | tree name |
| 35102 | tree legend |
| 36101 | spring name |
| 36102 | spring legend |
| 36201 | name of water with no flow |
| 36291 | water level number |
| 36301 | name of watercourse |
| 32602 | cultivated land legend |
| 36500 | legend for other typographic feature |

### Terrain2-area

| Value | Legend |
| --- | --- |
| 38300 | silting water area |
| 38400 | flooded area |
| 38600 | boulders in water |
| 38700 | shoal |
| 38900 | storage area |
| 39100 | other open space – unclassified |
| 39110 | other open space – open forest area |
| 39120 | other open space – brush |
| 39130 | other open space – open space |
| 39210 | 1:100 000 building area |
| 39211 | 1:50 000 residential building area, 3+ storeys |
| 39212 | 1:50 000 residential building area, 1-2 storeys |
| 39213 | 1:50 000 industrial building area |

### Terrain2-line

| Value | Legend |
| --- | --- |
| 30100 | artificial shoreline |
| 30200 | boundary line of topographic compartment, unclassified |
| 30211 | definite boundary line |
| 30212 | indefinite boundary line |
| 30999 | technical boundary line |
| 38200 | rapids |
| 39000 | log chute |
| 39500 | forest boundary |

### Terrain2-point

| Value | Legend |
| --- | --- |
| 38100 | rushes symbol |
| 38991 | storage area symbol |
| 38500 | boulders in water, unclassified |
| 38511 | boulders in water, underwater |
| 38512 | boulders in water, on mean water level |
| 38513 | boulders in water, above the water |
| 38800 | log-floating facility |

### Terrain2-text

| Value | Legend |
| --- | --- |
| 38100 | rushes symbol |
| 38991 | storage area symbol |
| 38500 | boulders in water, unclassified |
| 38511 | boulders in water, underwater |
| 38512 | boulders in water, on mean water level |
| 38513 | boulders in water, above the water |
| 38800 | log-floating facility |

### Man-made structures

| Value | Legend |
| --- | --- |
| 42200 | building, unclassified |
| 42210 | residential building, number of storeys unspecified |
| 42211 | residential building, 1-2 storeys |
| 42212 | residential building, 3+ storeys |
| 42220 | office or public building, number of storeys unspecified |
| 42221 | office or public building, 1-2 storeys |
| 42222 | office or public building, 3+ storeys |
| 42230 | holiday building, number of storeys unspecified |
| 42231 | holiday building, 1-2 storeys |
| 42232 | holiday building, 3+ storeys |
| 42240 | industrial building, number of storeys unspecified |
| 42241 | industrial building, 1-2 storeys |
| 42242 | industrial building, 3+ storeys |
| 42250 | religious building, number of storeys unspecified |
| 42251 | religious building, 1-2 storeys |
| 42252 | religious building, 3+ storeys |
| 42260 | other building, number of storeys unspecified |
| 42261 | other building, 1-2 storeys |
| 42262 | other building, 3+ storeys |
| 44300 | basin |

### Man-made structures as lines

| Value | Legend |
| --- | --- |
| 42100 | building, unclassified |
| 42110 | residential building, number of storeys unspecified |
| 42111 | residential building, 1-2 storeys |
| 42112 | residential building, 3+ storeys |
| 42120 | office or public building, number of storeys unspecified |
| 42121 | office or public building, 1-2 storeys |
| 42122 | office or public building, 3+ storeys |
| 42130 | holiday building, number of storeys unspecified |
| 42131 | holiday building, 1-2 storeys |
| 42132 | holiday building, 3+ storeys |
| 42140 | industrial building, number of storeys unspecified |
| 42141 | industrial building, 1-2 storeys |
| 42142 | industrial building, 3+ storeys |
| 42150 | religious building, number of storeys unspecified |
| 42151 | religious building, 1-2 storeys |
| 42152 | religious building, 3+ storeys |
| 42160 | other building, number of storeys unspecified |
| 42161 | other building, 1-2 storeys |
| 42162 | other building, 3+ storeys |
| 42200 | building area boundary line |
| 44100 | breakwater |
| 44300 | basin line |
| 44500 | cableway |
| 45111 | jetty- width under 5 metres |
| 45112 | jetty - width over 5 metres |
| 44200 | unclassified fence |
| 44211 | fence made of artificial material |
| 44213 | planted fence |
| 45700 | construction |

### Man-made structure point

| Value | Legend |
| --- | --- |
| 44591 | cableway support pole |
| 44600 | bell tower |
| 44700 | airport approach light |
| 44800 | mast |
| 44900 | monument |
| 45000 | observation tower |
| 45200 | gate |
| 45300 | chimney |
| 45400 | tar-burning pit |
| 45500 | wind power generator |
| 45710 | camp-fire site |
| 45800 | water tower |

### Man-made structure text

| Value | Legend |
| --- | --- |
| 42101 | building name |
| 42102 | building legend |
| 42201 | building complex name |
| 42202 | building complex legend |
| 44301 | basin name |
| 44302 | basin legend |
| 44803 | mast height |
| 44901 | monument name |
| 44902 | monument legend |
| 45303 | chimney height |
| 48111 | settlement name - town or city |
| 48112 | settlement name - other municipality |
| 48120 | settlement name -village, district, neighbourhood |
| 48130 | settlement name - house |
| 48190 | settlement name - other settlement |
| 44202 | fence legend |
| 44402 | cableway legend |
| 45002 | observation tower legend |
| 45402 | tar-burning pit legend |
| 45502 | wind power generator legend |
| 45702 | construction legend |
| 45802 | water tower legend |

### Elevation text

| Value | Legend |
| --- | --- |
| 52191 | contour elevation value |
| 54191 | depth contour depth value |
| 52210 | spot height |
| 54210 | depth point |

### Conservation area

| Value | Legend |
| --- | --- |
| 72100 | primeval forest |
| 72220 | nature reserve |
| 72400 | protected area |
| 72500 | protected forest |
| 72600 | natural forest |

### Conservation area line

| Value | Legend |
| --- | --- |
| 30999 | technical line |
| 72100 | primeval forest boundary line |
| 72200 | nature reserve boundary line |
| 72400 | protected area boundary line |
| 72500 | protected forest boundary line |
| 72600 | natural forest boundary line |

### Conservation area point

| Value | Legend |
| --- | --- |
| 72300 | unclassified |
| 72310 | protected stone |
| 72320 | protected tree |
| 72330 | ancient relic |
| 72340 | other protected feature |

### Conservation area text

| Value | Legend |
| --- | --- |
| 72201 | nature reserve name |
| 72202 | nature reserve legend |
| 72301 | protected feature name |
| 72302 | protected feature legend |
| 72401 | protected area name |
| 72402 | protected area legend |
| 72102 | primeval forest legend |
| 72501 | protected forest legend |
| 72602 | natural forest legend |

### Administrative division line

| Value | Legend |
| --- | --- |
| 30999 | technical line |
| 82100 | outer limit of marine territorial waters |
| 82200 | extreme limit of border zone |
| 82300 | outer limit of inland territorial waters |
| 82400 | outer and inner archipelago border |
| 84111 | state border |
| 84112 | provincial border |
| 84113 | municipal border |
| 84114 | border not surveyed |
| 84115 | regional border |

### Administrative division text

| Value | Legend |
| --- | --- |
| 82102 | legend for outer limit of marine territorial waters |
| 82202 | legend for extreme limit of border zone |
| 82302 | legend for outer limit of inland territorial waters |
| 82402 | legend for outer and inner archipelago border |
| 82501 | name of state boundary mark |
| 84300 | municipal administrative centre unclassified |
| 84301 | municipal administrative centre –provincial capital |
| 84302 | municipal administrative centre –other town or city |
| 84301 | municipal administrative centre –other municipality |
| 85100 | legend for municipal administrative centre |

### Level precision

| Value | Legend |
| --- | --- |
| 0 | Undefined |
| 500 | 0,5 m |
| 800 | 0,8 m |
| 1000 | 1 m |
| 2000 | 2 m |
| 3000 | 3 m |
| 4000 | 4 m |
| 5000 | 5 m |
| 7500 | 7,5 m |
| 8000 | 8 m |
| 10000 | 10 m |
| 12500 | 12,5 m |
| 15000 | 15 m |
| 20000 | 20 m |
| 25000 | 25 m |
| 30000 | 30 m |
| 40000 | 40 m |
| 80000 | 80 m |
| 100000 | 100 m |

### Vertical positioning accuracy

| Value | Legend |
| --- | --- |
| 1 | Undefined |
| 500 | 0,5 m |
| 800 | 0,8 m |
| 1000 | 1 m |
| 2000 | 2 m |
| 3000 | 3 m |
| 4000 | 4 m |
| 5000 | 5 m |
| 7500 | 7,5 m |
| 8000 | 8 m |
| 10000 | 10 m |
| 12500 | 12,5 m |
| 15000 | 15 m |
| 20000 | 20 m |
| 25000 | 25 m |
| 30000 | 30 m |
| 40000 | 40 m |
| 80000 | 80 m |
| 100000 | 100 m |

### Typesetting codes

The typesetting code is made up of the typeface code, text size and the spacing code.
For example, in typesetting code 6241:

- 6 = typeface code,

- 24 = size (points = 1/72 inches),

- 1 = spacing code (missing = not spaced, 1 = spaced 2 = amply spaced)